CHAPTER ONE:

INTRODUCTION

1.1 	Background of the Study

It is obvious that road pavement failures are experienced in every part of Nigeria. Road pavement failure has become a regular problem encountered by road users in Nigeria. Road pavement which is described as structural materials seated upon sub-soil layers (Woods and Adeox, 2002) is supposed to be a continuous stretch of asphalt laid for a smooth ride or drive, but discontinuity in the road network resulting in cracks, potholes, bulges and depression give rise to road failure (Aigbedion, 2007; Mesida, 1981). Poor drainage network, poor construction materials, bad design, usage factors are some of the factors considered as responsible for these failures. (Adenikal et al., 2018).

Geological factors are rarely considered as precipitators of road failure even though the highway pavement is seated on geological earth material (Momoh et al., 2008). This is due to non-appreciation of the fact that proper design of highway requires adequate knowledge of subsurface condition beneath the highway route. The non-recognition of this fact has led to loss of integrity of many highway routes and other engineering structures across the country (Olorun et al., 2005 & 2000).

Other causes of road pavement failure include low quality roads as a result of faulty designs, lack of good gutters and very thin coating that can easily be scattered or washed away by heavy floods and cannot withstand heavy duty vehicles on serious traffic.

Masaka-Asokoro Highway, located in North central Nigeria, is a crucial transportation route connecting various communities and facilitating economic activities. However, the highway has been experiencing significant pavement failures, leading to deteriorating road conditions, increased maintenance costs, and disruptions in traffic flow. Globally, all built structures have a limited period of usefulness that is, every infrastructure is functional only within a time period after which it begins to collapse. Roads are valuable infrastructures which are critical to the socio-economic development of many countries. Therefore, the road system demands constant geophysical investigation to deal with any cause of pavement failure to keep the road serviceable. Pavement failure is very complex as it involves structural wear out and functional straiten. The interactions among traffic, climate, materials, time and the roadway geometric features account for this straiten phenomenon. It is widely known that there is huge effect extremely high traffic volumes have on the rapid deterioration of road pavements. Masaka- Asokoro highway is one of the major modern highway that connect the Federal Capital Territory to the north east and south eastern part of Nigeria, most of the population especially civil servants who constitute the working forces in Abuja are the major residents in the area due to the affordability of houses in the area, not neglecting other people like traders and other unskilled workers who have their works and market places situated in Abuja.

The study attempts to investigate the causes of road pavement failure by considering different factors and other road related features in determining the causes of road pavement failure. Finding these causes will help the Road Authorities proffer lasting solution to these causes and as a result determine the maintenance needs and activity requirements. This will help the repair or reconstruction, and evaluating the long term financing necessities needed to keep the road in good condition.

1.2	Statement of the Problem

Masaka Asokoro Highway, located in Nigeria, has been experiencing significant road pavement failures, leading to deteriorating road conditions, increased maintenance costs, and disruptions in traffic flow.

Potholes, cracks, rutting, edge defects, and corrugations on highways can have significant negative effects on socio-economic activities and the safety of people. These road surface defects pose a safety risk to motorists, cyclists, and pedestrians. Potholes and cracks can cause accidents, especially if drivers lose control or vehicles experience sudden jolts. Edge defects and corrugations may lead to reduced vehicle stability, making it harder to maintain control, especially at higher speeds. Poor road conditions contribute to an increased number of accidents and injuries. Drivers may swerve or brake abruptly to avoid potholes or negotiate road irregularities, leading to collisions with other vehicles or objects. Pedestrians and cyclists can also be at higher risk of accidents due to road surface hazards. Potholes, cracks, and corrugations can cause damage to vehicles, such as tire punctures, bent rims, suspension issues, and alignment problems. The costs associated with repairing these damages can be a burden on vehicle owners and may result in increased insurance claims. Road defects can disrupt the flow of traffic. Drivers often slow down or change lanes abruptly to avoid potholes or rough road surfaces, leading to traffic congestion and reduced road capacity. This can result in delays, longer travel times, and increased fuel consumption, negatively impacting productivity and efficiency.

1.3. 	Justification of the study.

Road transportation is of great importance to every society and Nigeria at large. It determines the speed of socioeconomic development in any country as well as Nigeria. It is important to note that no economy can excel where there is no good road network. However, roads play an important role to the increase of any country gross domestic product. There is a good relationship that exist between a country’s economic growth and the standard of its road network. A country roads network should be designed and constructed in a systematic manner that the intended socio-cultural and the economic benefits can be maximized (Ighodalo 2009). It is important to note that Nigerian roads have often been characterized with potholes, bulges, long cracks and other pavement defects. These have posed a serious problem and disaster such that in Nigeria one can hardly travel a kilometer or two without coming across long cracks and potholes. These have resulted to a number of road accidents and a rapid fall on the nation’s economic development. Every  road built  have a stipulated design life span; but roads often  fail long before the expected date; some fail after flooding, some immediately after construction while others last to its entire life expectancy with proper maintenance.

1.4. Significance of the study.

The study on the causes of road pavement failure along the Masaka Asokoro highway in Nigeria holds several significant implications. The findings of the study can inform future road planning and design projects. Understanding the specific causes of pavement failures along the Masaka Asokoro highway can help engineers and designers develop more resilient road infrastructure. This includes selecting appropriate materials, considering local climate conditions, and implementing effective construction techniques to ensure the longevity of the road. The study's outcomes can support the development of policies and regulations related to road infrastructure. Policymakers can utilize the findings to create guidelines for road construction and maintenance practices, establish standards for quality control, and enforce regulations that promote sustainable and safe road networks. By studying the causes of road pavement failure, it becomes possible to identify the environmental impacts associated with the deteriorating road conditions. This knowledge can inform sustainable road construction and maintenance practices, such as using more durable materials, implementing effective drainage systems, and reducing waste generation during repairs.

Identifying the causes of road pavement failure is crucial for ensuring the maintenance and safety of the road network. By understanding the specific factors contributing to pavement failures along the Masaka Asokoro highway, appropriate remedial measures can be implemented to enhance road conditions and reduce the risk of accidents. Road pavement failures can have a significant economic impact on a region or country. The study's findings can help policymakers and transportation authority’s assess the financial implications of the pavement failures along the Masaka -Asokoro highway, including costs associated with repairs, delays in transportation, and increased vehicle operating costs. This information can guide resource allocation and budgeting decisions to address the issue effectively. Road pavement failures can lead to accidents and injuries, posing risks to public health and safety. The study's findings can aid in identifying hazardous locations along the Masaka-Asokoro highway and implementing preventive measures to reduce the occurrence of accidents, such as better signage, improved road markings, and targeted maintenance efforts.

Overall, the study on the causes of road pavement failure along the Masaka-Asokoro highway in Nigeria has significant implications for infrastructure maintenance, economic considerations, sustainability, planning and design, public health and safety, and policy development. By addressing the root causes of pavement failures, authorities can work towards improving road conditions, enhancing safety, and maximizing the long-term benefits of the transportation system.

1.5. 	Climate and Vegetation of the Study Area.

The climate along the Masaka Asokoro highway in Nigeria is typically tropical, characterized by high temperatures and distinct wet and dry seasons. The region experiences a rainy season from April to October, with the heaviest rainfall occurring between June and September. Dry conditions prevail from November to March.

As for vegetation, the area likely supports a mix of tropical savanna and woodland. You can expect to see a variety of grasses, shrubs, and scattered trees adapted to the seasonal climate. Acacia and baobab trees are common in such environments, contributing to the overall landscape. The study area lies in the North central Nigeria and is within the guinea savannah, which composes essentially of shrubs, short trees and scattered grasses.

1.6 	Aim and Objectives of the Study

The aim of this research is to investigate the causes of road pavement failure; to achieve this aim we have the following objectives:

(i) Identify the primary causes of pavement failure along Masaka - Asokoro highway.

(ii)  Find the resistivity of the road subsurface.

(iii) Determine the role of environmental factors in pavement failure

(iv) Investigate the impact of poor construction practices on pavement failure.

(v) Determine the porosity of the road pavement smaterial.

`

CHAPTER TWO

LITERATURE REVIEW

2.1 	CONCEPTUAL FRAMEWORK

Road pavement failure is a complex phenomenon resulting from the interplay of various factors (Huang, 2004). This conceptual framework explores the relationships between traffic-related factors, design and construction factors, maintenance and management factors, and environmental factors that contribute to road pavement failure.

Traffic-related factors, such as traffic volume and speed, significantly impact pavement design and construction. Inadequate pavement design and construction can exacerbate the effects of traffic-related factors, leading to premature pavement failure (Kumar and singh, 2017).

Pavement design and construction factors influence maintenance and management needs. Well-designed and constructed pavements require less frequent maintenance, whereas poorly designed and constructed pavements necessitate more frequent repairs and resurfacing (FHWA, 2019).

Effective maintenance and management practices can extend the lifespan of pavements, while inadequate practices can accelerate deterioration. Regular maintenance can protect pavements from environmental stressors, while inadequate maintenance can allow environmental and traffic-related factors to compromise pavement integrity. By understanding these relationships, effective strategies can be developed to prevent or mitigate pavement failure (Chen and Scullion, 2017).

2.2 	Theoretical Framework.

The Electrical Resistivity Method is selected as the geophysical method for this investigation. The electrical resistivity method for road subgrade investigation involves using the electrical properties of the subsurface to assess the soil's suitability for road construction. Several theories guide this method.

2.2.1 Telford Theory of Resistivity.

The Telford theory of resistivity states that the electrical resistivity (ρ) of a material is related to its physical properties, such as moisture content, density, and composition.

The type of electrode configurations used in this research are Wenner-Alpha and Vertical Electrical Sounding (VES) configurations using four electrodes including two current electrodes and two potential electrodes arranged in a straight line. In general, this configuration is more sensitive to changes in lateral and shallow resistivity than other electrode configurations. Wenner-Alpha configuration can be seen in Figure 2.1 below.

Figure 2.1 Wenner-Alpha Configuration (Telford, 1976)

Geoelectrical method assumes that the earth is an isotropic homogeneous medium. The current flowing down the earth’s surface will flow to all directions because the air has a vast resistivity causing the current not to flow into the air. It causes the distribution of the current flow to only flow down the earth’s surface with a half-ball pattern shown in Figure 2

Figure 2.2 Current Flow Distribution (Telford, 1976)

The resistivity measured in the geoelectrical method is not the real resistivity, but apparent resistivity (ρ) which can be determined by the following equation,

2.122.          222².                        (2.1)

Where				( 2.2)

ρ is apparent resistivity (Ωm); ΔV is potential (mV); I is the current injected into the earth (I); and k is the geometric factor, meanwhile, the geometric factor of Wenner-Alpha configuration is k = 2πa where a is electrode spacing (m).

The resistivity measured during measurement is the apparent resistivity. To estimate the distribution of the apparent resistivity to depth can be done through inversion method.

[

2.2.2Electrical Resistivity (ρ).

It is a material property indicating how strongly a given material opposes the flow of electric current.

For road subgrade investigation, changes in resistivity can reveal variations in soil composition, moisture content, and compaction.

The resistivity formula is also expressed as

ρ = RA/L.                                         (2.3)

Where ρ is the resistivity, R is the resistance, l is the length of the material and A is the area of cross-section.

2.2.3 Application to Road Pavement Failure

The electrical resistivity methods, specifically Telford Theory of Resistivity, Vertical Electrical Sounding (VES) and Wenner methods, can be applied to investigate the causes of road pavement failure. The underlying principle is that changes in the electrical resistivity of the pavement materials can indicate variations in moisture content, density, and structural integrity.

By measuring the electrical resistivity of the pavement materials using the Wenner method, researchers can:

- Identify areas of high moisture content, which can contribute to pavement failure.

- Detect changes in pavement density, which can indicate settlement or compaction issues.

- Evaluate the structural integrity of the pavement, including the presence of cracks or voids.

Telford Theory of Resistivity can be used to analyze the electrical resistivity data and relate it to the physical properties of the pavement materials.

2.2.4 	Research Design.

This study employs a quantitative research design, utilizing a survey approach to investigate the causes of road pavement failure. The survey will be conducted using the resistivity method, specifically Wenner's method and Vertical Electrical Sounding (VES), to measure the electrical resistivity of the pavement materials.

2.2.5 	Data Analysis Techniques:

The data analysis techniques for this study include:

- Vertical Electrical Sounding (VES) Analysis: The resistivity data collected using Vertical Electrical Sounding will be analyzed using Telford Theory of Resistivity with the aid of the windresist software to calculate the electrical resistivity of the pavement materials.

- Wenners’ Method Analysis: The electrical resistivity data collected using Wenners’ method will be analyzed usingTelford Theory of Resistivityto estimate the resistivity of the pavement materials.

- Correlation Analysis: The resistivity data will be correlated with other pavement properties, such as density, moisture content, and texture, to identify relationships between these properties and pavement failure.

2.2.6 	Sampling Strategy:

The sampling strategy for this study involves selecting a number of survey lines and stations along the road pavement. The survey lines will be spaced at regular intervals, and the stations will be located at regular intervals along each survey line.

2.2.7 	Data Quality Control:

The data quality control measures for this study include:

- Instrument Calibration: The electrical resistivity meter will be calibrated before each survey to ensure accurate measurements.

- Data Validation: The resistivity data will be validated by checking for errors and inconsistencies.

2.2.8 VERTICAL ELECTRICAL SOUNDING

In the vertical electrical sounding technique, vertical variations in the ground apparent resistivity are measured with respect to fixed centre of array. The survey is carried out by gradually expanding/increasing the electrode spacing about a fixed centre of the array. The Wenner and Schlumberger arrays are commonly used but the dipole-dipole array could also be used. Sounding data are presented as sounding curve by plotting apparent resistivity against electrode spacing on a bi-log paper. In this project, vertical electrical sounding (VES) measurements were made using the Schlumberger arrays.

2.2.9 WENNER METHOD.

The Wenner alpha method is a four-electrode (pin) technique used to measure the electrical resistivity of soils. The method involves inserting four electrodes into the soil at equal spacings and applying a  known current between the outer electrodes. The resulting voltage drop is measured between the inner electrodes, and the resistivity is calculated using the following equation:

ρ = 2πas(V/I).                            (2.4)

where ρ is the resistivity, a is the electrode spacing, V is the voltage drop, and I is the current ( Reynolds, 2011). The following figure illustrates the Wenner alpha four-pin method.

Figure 2.3Wenner Alpha Four-Pin Method

If the depth of the probe, b << a, as is the case of electrodes penetrating the ground only for a short distance (as usually happens)

Using the Wenner alpha method, the electrode spacing is increased along a path to measure greater depths of soil. This is possible because, as the electrodes spacing is increased, the test source current penetrates greater areas, in both vertical and horizontal directions, regardless of how much the current path is distorted due to the varying soil conditions. Smaller electrode spacing measurements, shallower measurements, are important to characterize the soil with which the grounding system will be in contact. Longer electrode spacing, deeper measurements, are typically taken such that the maximum spacing between pins is equivalent to the maximum dimension of the grounding system to be evaluated. If the resistivity varies appreciably with depth, it is often desirable to increase the range of electrodes spacing to assess the resistivity.

2.3	Previous works in the Study Area.

There are many researches conducted purposely for the causes of road pavement failure in order to ascertain the causes of the pavement failure around the world.

Adenika et al, (2018) employed the application of geophysical approach to highway pavement failure using schlumberger vertical electrical sounding 2D imaging dipole – dipole technique and ground magnetic method. The result revealed that the stable segments were founded on a shallow/outcropping basement near homogeneous substratum devoid of major geological features while features suspected to be fault/fractured zone, buried stream channels, bedrock, depression and lithological contacts were identified beneath failed segments of the highway pavement.

Asnake  (2015) conducted the assessment of road pavement failure along Addis Ababa – modjo trunk road through field and laboratory investigation. The result revealed expansive subgrade soil, side drain problem, workmanship problem, use of substandard materials are found to contribute for pavement failure.

Andreas Nikolaou (2016) studied asphalt pavement deterioration through remote sensing. The result of this shows that the remote sensing methods are a useful tool for the transportation infrastructure assessment, both for monitoring the condition of asphalt pavement and detecting structural damages of asphalt.

Temesgen  (2018) investigated the causes of frequent failures on maintained asphalt pavement federal roads in Ethiopia through document review and observation and questionnaire survey. It was revealed that the maintained roads were utilized by overloaded trucks, pavement were aged, severe weather was witnessed, drainage were poor, the construction quality was poor and construction materials were substandard, the maintenance management system were poor. All these in aggregate caused pavement deterioration of the maintained roads.

Ndefo (2012) worked on the causes of highway failure in Nigeria. The result shows poor design and construction, poor maintenance of already built highway, uses of low-quality materials in construction, poor workmanship and poor supervision of construction works and plying of heavy traffic that were not meant for the road on the road.

Ijciet (2016) investigate the causes of pavement deterioration in Khartoum state using field survey and laboratory testing. The result revealed that the road investigated experienced severe failures in forms of cracks, potholes, and rutting in the wheel path. The causes of these failures were found mainly linked to poor drainage, traffic overloading, expansive subgrade soils and the use of low quality materials in construction.

Leon et al. (2022) carried out research on dominating factors of road failures. The findings revealed from an assessment of the Relative Importance Index that utility cuts by the Water and Sewerage Authority were the most important (0.904), followed by maintenance culture (0.898), quality of work (workmanship) (0.888), poor drainage facilities, and overloaded vehicles (0.854). Climate change (temperature) was the lowest-ranked cause overall (0.568). The findings also show that economic growth and development are directly and indirectly affected, resulting in high vehicle running costs, longer travel times, and higher prices for fundamental products and services.

Ebhohimen (2013) employed geophysical investigation of road failure using electrical resistivity imaging method, a case study of uhiele – opoji road edo state. The result revealed that the causes of road pavement failure on the studied road was found to be majorly as a result of a combination of clayey topsoil/sub grade soils, water-logged sands due to ingress with characteristically low resistivity values and thin pavement thereby unable to withstand pressure exerted on the road.

Osadebe et al. (2019) conducted a research on the effects of heavy vehicles on flexible pavement, a case study of Abuja - Kaduna – Kano roads. An average Load Equivalency Factor (LEF) of 3.43 was estimated for each heavy vehicle plying the road which could explain some failures (alligator cracks, potholes, and depressions, linear or longitudinal cracks along the Centre line amongst others) noticed on the road.

Udoinyang, et al. (2021) conducted a geophysical perspective of road pavement failure: a case study of Ikot Ekpene- Umuahia road, Nigeria. The lithology of the subsurface reveals that their resistivity falls between >83Ωm to 103Ω. The geo electric zones constructed from the VES resistivity structures shows that the pavement segments were founded on a shallow basement near low resistive layer bereft of vital geological features. This made it impossible for the pavement to withstand stress as a result of load from road users.

Babadiya, and Igwe, (2021) carried out a geotechnical Investigation on the Failure of Road Pavements in Abakaliki, Southeastern Nigeria. Investigation revealed two different soil types used for subgrade: A–2–7 and A–7–5 both with moderate liquid limit and plasticity index values. The particle size distribution curves for AA, AO and EA indicate that AA and AO soils are well graded (poorly sorted) than EA, therefore have the fairest engineering properties. Failures were concentrated on un-stabilized roads.

Gerritsen et al. (1987) Field study in The Netherlands is on the verge of cracking in Asphalt pavement, and potential surface causes Cracking. Static indirect tensile tests were performed on basic samples collection. They showed that asphalt concrete outside the wheel paths tend to be low Characteristics of the force at low temperatures.

Dauzats and Rampal (1987) surveyed on several pavement sections in the south of France. In this area, the pavements are subjected to severe thermal pressure. After construction of the road containing a slow lane and a fast lane the longitudinal surface cracks where observed after 3 or 5 years in these sections. Longitudinal section are located in the midline side of the slow line and it also observed that the appearance of cracks fluctuated with the seasons.

Matsuno and Nishizawa (1992): they examined longitudinal surface cracking in asphalt road pavement in Japan. The manual observation is that the cracking appeared 1 to 5 years after the road’s construction typically occurred in the passing lane. It was also recorded that the cracking were within the wheel paths and the cracks did not appear in shadowed areas.

Perdomo and Nokes (1993): they examine the response of a flexible pavement system due to different loading by using the computer program CIRCLY. The loading considered are of two types that are non-uniform normal tractions only and non-uniform normal tractions accompanied by non-uniform inward shear tractions. The researchers concluded that the maximum tensile strains occur on the surface of the pavement at the edges of the tires when inward shear tractions were considered in the analysis.

Jacobs (1995): the author analyzed the stresses in a pavement structure that have three layers with constant thickness for each layer , same elastic modulus for base and subgrade layers, one Poisson’s ratio for all layers, and three different elastic moduli for the AC layer. The author concluded that by the tangential stress on the surface the normal stresses at the bottom of the asphalt concrete layer were not affected.at the edge of the loaded area the tensile stresses can be much higher than the tensile stresses at the bottom of the asphalt concrete layer.

Collop And Cebon (1995): by using different analytical and numerical solution they examined the potential of longitudinal surface fatigue cracking in asphalt pavements. The authors concluded from 2D plane strains elastic half space solution that there is a singularity in surface stresses at both the ends of the contact when assumed a discontinuity in shear tractions and that shear tractions between the pavement and the tires induce high local tension to the surface longitudinal Cracks that is propagate by transverse cracking.

Akinola  et al. (2017)  conducted an integrated geophysical investigation for pavement failure along a dual carriage way, Southwestern Nigeria. The results obtained from the VES and 2-D resistive image of the subsurface showed major geologic features, which are typical of fractures, faults, joints, cavities and voids. The near surface subsoil on which the road pavement is founded within the failed and stable segments is predominantly characterized by low resistive materials (<100Ωm) and (<150Ωm) respectively. Therefore, the geological factors responsible for the susceptibility of the road to failure are clayey subgrade soil beneath the road pavement, lateral inhomogeneity, near surface geological structures and changes in elevations due to fluctuation in the saturated zone.

Oppong (2016) worked on Factors That Contribute to Pavement Deterioration on the Mampong Arterial (Suame roundabout – Plankton road). Following the correlation analysis, it was found that vehicular traffic and especially ESAL had strong relationships with pavement surface distresses such as depressions, cracks, potholes and rutting. This was demonstrated by high Pearson Correlation coefficients estimated for the various relationships.

Xinren  (2011) worked on Pavement Surface Distress Detection and Evaluation Using Image Processing Technology. The proposed analysis algorithms are capable of enhancing the pavement image, extracting the pothole from background and analyzing its severity. To validate the system, actual pavement pictures were taken from pavements both in highway and local roads. The experimental results demonstrated that the proposed model works well for pothole and crack detection.

Mohd (2020) conducted a research on Road Damage and Maintenance along Federal Road in Kuala Krai, Kelantan. From the study, pavement cracks is the dominant defect mechanism from 2016 to 2018. Potholes and surface defects indicate the upward trend from 2016 to 2019 along Jalan Kuala Krai-Gua Musang.

Siraj (2016) conducted a resarched on the Investigation of Asphalt Pavement Failure Due to Sub-Base and Subgrade Soil Properties: A Case Study of Kemmise – Dessie Road. The laboratory results showed that the soil materials used along the road have high percentage clayey material for half of the natural subgrade soil sample (rated as fair to poor materials for road use) and low percentages of clayey and silt/gravel/sand materials of sub-base and selected fill soil samples. s. Conclusively, the pavement failure may necessarily be due to weak/poor soil pavement layer performance in the different failed locations.

Magdi (2016) carried out research on Investigating Causes of Pavement Deterioration in Khartoum State. The results revealed that the roads investigated experienced severe failures in the forms of cracks, potholes and rutting in the wheel path. The causes of those failures were found mainly linked to poor drainage, traffic overloading, expansive subgrade soils and the use of low-quality materials in construction.

Shehu et al. (2020) carried out research on the Evaluation of Failure Pattern on a Flexible Pavement a Case Study of Warawa Local Government Kano State. The results were most of the damages and failures in the pavement are serious and extreme surface deformation, cracks, disintegration, and surface defects. These damages and failures are caused by fatigue and other types of failures resulted from the movement of heavy vehicles and trucks, poor drainage design, unsuitable pavement layers thickness design, and improper pavement mix design and selected materials.

Amanamba et al. (2021) worked on Evaluation of the causes of flexible pavement failure; case study of Enugu/Port-Harcourt expressway. The results obtained showed evidence of presence of clay in the subgrade; hence, concluding that the failure was mainly caused by poor soil material. From visual condition survey, it was noted that there were no drainages even at the critical paths of the alignment, and there was a significant proportion of Heavy Goods Vehicles (HGVs) which may not have been adequately considered during design.

Afolayan et al. (2021). Worked on Pavement Failure, Causes and solutions. The result revealed that the causes of pavement failure are poor soil, ground hole, and poor quality of materials and timing of maintenance.

Jegede (2000) worked on the Effect of soil properties on pavement failures along the F209 highway at Ado-Ekiti, south –western Nigeria. The laboratory results of the disturbed soil samples collected from the failed sections of the road revealed a strong reduction in the strength of the soil materials used in the construction of the road.

Jegede (2004) conducted research on Highway Pavement Failure Induced by Poor Geotechnical Properties at a Section along the F209 Okitipupa-Igbokoda Highway, Southwestern Nigeria. The result revealed reduction in strength because of surface water ingress into primary cracks that later develop into potholes.

Hussein (2022) conducted research on the causes of pavement failure of Edunabon-Sekona Road, Osun state Nigeria. It was revealed from the study that the pavement failed due to the rebound deflection values of the subgrade.

Simeon et al. (2018) carried out study on the Geotechnical Assessment of Pavement Failure along Lagos-Ibadan Express way. The result showed that the increase in Natural Moisture Content (NMC) along the failed portion could be because of high water table along these sections. Some of the materials at failed location had values of linear shrinkage (LS) ˃ 8% which suggest high susceptibility to shrinkage swelling which result in differential settlement and contributed to pavement failures along these sections of the road.

Moneke (2020) conducted a research on Road Pavement Failure and exploratory analysis of Requirements for effective management of highway maintenance projects. The result and finding from the analysis found that poor side drainage system, poor quality of construction design and materials are the severest causes of road pavement failures.

Abhijeet et al. (2022) employed Investigation on Causes of Pavement failure and its remedial measures. The result revealed that the road investigated experiences severe failure in form of crack, pothole and rutting in the wheel path. The causes of those failures were found mainly linked to poor drainage, traffic overloading expansive subgrade soil and use of low quality materials in construction.

Salim (2021) employed Systematic Evaluation and Analysis of Bituminous Road Pavement Failure. Result/Finding worked out that their nature and magnitude change depends upon their meteorological, geographical parameters, type of road, traffic load etc.

Samuel (2022) worked on pavement defect investigation using treated and untreated expansive road subgrade materials with varying plasticity index. The result showed an increase in California Bearing Ration (CBR) value with a reduction in swell values translating to lesser stresses within the pavement structure which reduced rutting, fatigue and permanent deformation of the road.

Tarekegn et al.(2018) worked on Laboratory Investigations on the Causes of Road Failures Constructed Along Asendabo to Deneba Road Section . From the field and laboratory test results, it was concluded that the materials used for the road construction were poor according to the design specification, on the other hand, the natural sub-grade soils type was expansive soil at which the road deteriorated as compared to the specification. The CBR values were also checked for extremely deteriorated area. The presence of Poor drainage conditions was also a case for failure to  occur. The poor drainage condition together with the material quality create the major causes of Asendabo to Deneba road segment failure.

Adagunodo (2014) conducted Electromagnetic Investigation into the Cause(s) of Road Failure along Takie-Ikoyi Road, Ogbomoso. KHFILT software was used for the processing of the data obtained. Conducting bodies which indicate clayey materials or linear geologic features such as faults, fractures, joints, and contact between two rocks were inferred from the interpretation. It was concluded that the road was probably constructed on clayey materials which was supposed to be excavated and refilled with laterites. From another angle, it was viewed that may be the engineers intentionally filled the sub-base with clayey materials during construction in order to get their financial returns which the masses are now paying for

Ozegin (2016) worked on Troubled roads: Application of surface geophysics to highway failures of the sedimentary terrain (IruekpenIfon Road) of Edo State, Nigeria. Results show that the topsoil generally varies in composition from clay to clayey and laterite with resistivity values varying from 89 to 400 Ωm and thickness between 0.2 and 4.0 m. The fractured layers composed of clay and compacted clayey sand which represents the recent alluvial deposits with resistivity values of 2 to 89 Ωm and the thickness between 1.5 and 11 m. The fresh water zone is characterized by low resistivity ranging from 0.5 to 23 Ωm, which is diagnostic of saline water saturated with clay formation, fresh water ingression, and marls. The values of co-efficient of anisotropy () range from 1.03 to 2.19. The relatively higher values of λ (1.30 to 2.19) suggest that the subsurface rocks in these areas are likely to be more intensely fractured and more permeable. The saline water saturated with clay formation, fresh water ingression, fracture and marls clearly limit the lithological contacts and enhance high swelling potential which might be responsible for the road pavement failures in the studied area.

Ashebir Belete Werkineh et al(2020). conducted Assessment on the Performance of Pavement Structure Due to Subsurface Course Materials: A Case Study from Assosa to Mendi. The result revealed the cracks pixels and non-cracks pixels were sorted out by image analysis techniques in MATLAB. This analysis generated the image that showed cracks separated from non-crack pixels.

Bashir et al (2024) employed Evaluation Of Road Pavement Failure: A Case Study Of Federal Medical Center Jabi Road, Abuja. It was concluded that construction material of the road are good and excellent, and have all complied with the requirements. Hence by visual observation, the drainage system of the road is very poor and the road surface is not cambered to the direction of the drainage so as to drain off water especially during rainy season. Water possesses a lot of threat to roads which gradually creates void and allow infiltration of water causing severe damage to the subsoil and pavement.

Nathan and Richarh (2019) Worked on Determination of Layers Responsible for Rutting Failure in a Pavement Structure. . The result revealed that most of the rutting originated from the surfacing layer.

### Jabar (2016) employed The Use of Stabilized Subgrade Soils For Road Pavements In Kurdistan. The experimental investigation considered soils stabilized with 2%, 4% cement content and a combination of cement and lime with 2% cement plus 1.5% lime and 4% cement and 1.5% lime. The results were used to develop empirical equations to: (i) predict resilient modulus values of deteriorated modified soils as a function of different stabilizer contents and types; (ii) correlate resilient modulus values of soils with their UCS and stress state; (iii) determine the accumulation of permanent deformation in modified subgrade soils subject to weathering.

### Ofonime Akpan (2005) worked on the Relationship between Road Pavement Failures, Engineering indices and Underlying Geology in the Tropical Environment. The result shows that locations exhibiting high failure rates are underlain by shaly or marly subgrade whereas locations characterized by low failure rates are underlain by weathered basement or sandy unit as subgrade.

### Afolayan, and Abidoye, (2017) conducted a research on the Causes of Failure on Nigerian Roads. The result from the finding shows inadequate preliminary geological investigation, poor design of roads, bad supervision and workmanship, lack of routine and periodic maintenance, bad drainage, improper use and overloading and inadequate sanction for highway failure as the major causes of failure on Nigerian roads.

Oyem (2020) conducted Evaluation of Road Failure along an Erosion Prone Highway in Ogun State Southwestern Nigeria Using Integrated Methods. The result review that the moisture content, specific gravity, liquid limit, plastic limits and plasticity index ranged from 9.4–18.2%; 2.54–2.58; 26.8–42.6%; 18.2–25.2% to 7.1–21.6%, respectively. Grain size distribution showed the fine and coarser fractions range from 12–32.5% to 49.4–80.5%, respectively. The maximum dry density and its optimum moisture content ranged from 1.64–1.74 g/cm3 to 12–17%, respectively. CBR results ranged from 3 to 12%. The class of sub soils namely A-26, A-7, A-2–7 (clayey soils), were identified and rendered unsuitable as road construction materials. The results from this investigation revealed that the sub soils are poor road construction materials due to its fine and plasticity. This should be put into consideration during the reconstruction and rehabilitation of the road.

Adiat, et al. (2017) Worked On Evaluation Of Road Failure Vulnerability Section Through Integrated Geophysical And Geotechnical Studies. The geoelectric section generated shows that the study area was underlain by four geoelectric layers namely the topsoil, the weathered layer, the partly weathered/fractured basement and the fresh basement. The major part of the topsoil, which constitutes the subgrade, is characterized by relatively low resistivity values . suggestive of weak zones that are capable of undermining the stability of the proposed road. This therefore suggests that the layer is composed of incompetent materials that are unsuitable for engineering structures. Furthermore, fractured basement was also delineated beneath some portion of the proposed road. Since fracture is a weak zone, its presence can facilitate failure of the proposed road especially when it is occurring at shallow depth. The geotechnical results reveal that most of the investigated soil samples are clayey in nature. Integration of the results demonstrates that there is a good correlation between geophysical results and the geotechnical results. Furthermore, a vulnerability section that divided the road segments into three zones based on the degree of vulnerability was produced. These zones were high, moderate and low vulnerability zones. It is estimated that about 60% of the road segments constitutes moderate degree of vulnerability while 30% and 10% of the segments respectively constitute high and low degree of vulnerability.

Ganiyu et al. (2021) employed Investigation of incessant road failure in parts of Abeokuta, Southwestern Nigeria using integrated geoelectric methods and soil analysis. The VES results delineate three geoelectric layers comprising topsoil, weathered basement (clayey), and fractured/fresh basement with their corresponding resistivity values ranging between 71 and 282 Ωm, 12–76 Ωm, and 261–10,094 Ωm. The thicknesses range between 0.9 and 3.2 m for topsoil and 4.5–19.1 m for weathered basement. 2D resistivity inverted sections delineate two lithologic layers: topsoil with resistivity values > 200 Ωm, devoid of linear geological structures as competent topsoil on a stable road section while incompetent topsoil on failed road sections were characterized by resistivity values devoid of linear geological structures as competent topsoil on a stable road section while incompetent topsoil on failed road sections were characterized by resistivity values < 200 Ωm. The weathered layers as depicted by 2D inverted resistivity sections were generally of resistivity less than 100 Ωm while fractured/fresh basement were not depicted in the 2D model. Failed road sections are underlain by topsoil with a resistivity (< 200 Ωm ) shallow weathered (clayey) layer, and differential settlement of saturated subgrade materials. Soil analyses results showed that the stable portion depicts lowest mean values of plasticity index (5.12%) and liquid limit (22.21%), uniform sandy-loam textural class, highest mean value (1.24 cm/h) of saturated hydraulic conductivity (Ksat); % sand content (73.2%) and Penetration Resistance (PR) values that increased within the sampling depth (0–40 cm). However, one of the failed sections had lowest mean value (0.81 cm/h) of Ksat, % sand content (64.9%) and PR values that decreased within the sampling depth (0–40 cm). Analysis of variance (ANOVA) showed that there were significant differences at the 5% level among all designated road sections with respect to percent silt and moisture content (MC).

Shehu et al. (2020). Worked on Evaluation of Failure Pattern On A Flexible Pavement A Case Study Of Warawa Local Government Kano State. After the thorough assessment of the pavements on selected roads it can be concluded that the percentage of weighted failures on the roads are cracking=2.098, potholes=9.036, and raveling=0.016 with the total weighted failure as 11.474.

Abdulazeez Rotimi (2024).  A Review of Road Pavement Failure: A case study of Nigerian Road. The main reasons why Nigerian roads fail are further identified in this paper as insufficient preliminary geological investigation, poor road design and construction, poor monitoring and quality workmanship, inadequate routine and regular maintenance, poor drainage, employing poor and low-quality materials, traffic congestion, overuse, incorrect use, and inadequate punishment for roadways failures.

Desta Degu et al (2022). employed Investigating causes of flexible pavement Failure: A case study of Bako to Nekemte Road Oromia, Ethiopia. The study found that the cause for failure of flexible pavement in the case of Bako to Nekemte was: insufficient and absence of side drainage structure, traffic loading, poor gradation of base course and sub-base material, and poor quality of subgrade soil.

[

2.4	Location of the Study Area.

The Masaka Asokoro highways is a major highway in Nigeria, connecting the cities of Masaka and Asokoro. It’s an important route for commerce and trade, and it’s also a popular tourist destination, scenic view and abundant natural beauty. Masaka is located on the Latitude. 9.005° or 9° 0' 18" north; Longitude. 7.67361° or 7° 40' 25" east, and Asokoro is located on the latitude of 9.0479 and longitude of 7.5155 .The Masaka Asokoro highway runs through the southwestern part of Nigeria, connecting the city of Masaka in Nasarawa state to the city of Asokoro in Abuja. It’s a major transportation route in the region and it’s also a popular spot for picnics and outdoor activities.

Fig. 2.3 	Google road map of Masaka-Asokoro Road.

2.5 GEOLOGY OF THE STUDY AREA

The geology of the Masaka-Asokoro (AYA) road in North Central Nigeria is primarily defined by the region's location within the Nigerian Basement Complex, with some influence from the adjacent Middle Benue Trough.1 This area is characterized by a landscape of rolling plains punctuated by prominent hills and inselbergs, like the iconic Aso Rock.

The underlying geology is dominated by crystalline igneous and metamorphic rocks of Precambrian age. These include Migmatite-Gneiss Complex, which is the most common rock type in the Nigerian Basement Complex.It consists of biotite gneiss and banded gneiss, which are highly foliated and often intricately folded.They are believed to have originated from sedimentary rocks that were later subjected to intense metamorphism. Older Granites, a variety of granitic rocks, including porphyritic biotite granite, are also prevalent. Metasedimentary Rocks: Ancient sedimentary rocks, such as quartzites (metamorphosed sandstones) and schists, are also present.

The landscape is heavily influenced by tectonic events, particularly the Pan-African orogeny (around 600-500 million years ago), which remobilized the basement rocks. Key structural features include:

The rocks are highly fractured, with prominent lineaments often trending in a northeast-southwest direction. These fractures are significant as they control the flow of groundwater and can be associated with mineral deposits.5

The area is known for its distinctive hills and inselbergs, such as Aso Rock.6 These are resistant rock formations that have been left standing after millions of years of erosion on the surrounding softer rock. Aso Rock, for example, is a large igneous intrusion composed of gabbro and granodiorite.

The road and its immediate surroundings are covered by a layer of soil and weathered rock. Many of the hills and plains are capped by lateritic soils, which are reddish, iron-rich deposits formed by intense weathering in the tropical climate.The underlying crystalline rocks have undergone significant weathering, forming a thick layer of weathered material (regolith) that is often silty or clayey.

In summary, the geology along the Masaka-Asokoro (AYA) road is a classic example of a basement complex terrain, defined by ancient, crystalline rocks that have been shaped by tectonic forces and millions of years of weathering, resulting in a landscape of rolling plains and iconic rocky outcrops.

Fig. 2.4. Geological Map Of Nigeria Showing The States Of Study Area.

Fig. 2.5 Enlarged Geological Map Of Study Area

CHAPTER THREE

MATERIALS AND METHODS

3.1. 	MATERIALS

The following are the materials and Instrumentation for this Electrical Resistivity Method in Subsurface Investigations.

- Electrical Resistivity Meter.

A high-quality, calibrated resistivity meter capable of providing accurate measurements. This could include commercial meters with multiple electrode configurations suitable for different survey conditions.

- Electrodes.

Electrodes compatible with the wenner and Vertical Electrical Sounding resistivity meter . We are to ensure proper material construction and spacing for effective measurements.

- Cables.

Insulated cables to connect electrodes to the resistivity meter. Cables with low resistance and good shielding to minimize interference.

- Current and Potential Electrode Spikes:

Spikes for easy insertion of electrodes into the ground, ensuring good electrical contact. The spikes is sturdy and corrosion-resistant.

- Field Computer or Data Logger:

Field computer or data logger to record resistivity measurements in real-time. This facilitates efficient data collection and minimizes the risk of errors.

- GPS Device.

A GPS device to accurately record the location of resistivity measurements. This aids in creating georeferenced resistivity profiles and maps.

- Power Supply.

A reliable power supply for the resistivity meter and associated equipment. This include rechargeable batteries or a portable generator depending on the field conditions.

- Field Accessories:

Accessories such as measuring tapes, markers, and flags to precisely mark and measure electrode spacings during field surveys.

- Safety Equipment.

Safety cones, warning signs, and personal protective gear to minimize risks during fieldwork.

- Calibration Tools.

Calibration tools specific to the resistivity meter to verify and adjust instrument accuracy regularly.

- Weather Protection:

The equipment is shield from adverse weather conditions with waterproof covers or enclosures.

- Transportation.

Sturdy cases or containers for safe transportation of the resistivity meter and accessories between survey locations.

- Documentation Tools.

Notebooks, data sheets, and pens for recording field observations, electrode spacing’s, and any other relevant information.

- Backup Equipment

Backup cables, electrodes, and other critical components to address unforeseen issues in the field.

- Software.

Data processing and interpretation software for analyzing resistivity data and creating profiles or maps.

3.2 	METHODOLOGY.

The electrical resistivity method using Wenner's and Vertical Electrical Sounding (VES) method will be employed to investigate the causes of road pavement failure by selecting areas of road pavement failure and select suitable locations for the electrical resistivity survey. This method is a non-invasive and non-destructive technique that measures the electrical resistivity of the pavement materials.

3.3 FIELD PROCEDURE

The Terrameter, Ohmega Allied was used to acquire thirty-five (35) VES stations using the Global Positioning System (GPS) to locate the stations in the Area Council. Profiles of 100m length were established at different locations to cover the area under study. The Schlumberger and wenner's configuration method was used in taking the data. The Schlumberger array involves the use of the two potential electrodes P1& P2 fixed about a center '0' as the point of inscription, while the current electrodes C1& C2 are moved in steps collinearly with P1&P2. The Wenner's method involves inserting four electrodes into the soil at equal spacings and applying a known current between the outer electrodes. The resulting voltage drop is measured between the inner electrodes. The equipment measures resistance directly which is then multiplied with the geometric factor to get the apparent resistivity.

CHAPTER FOUR:

DATA PRESENTATION, INTERPRETATION AND DISCUSSION

4.1 DATA PRESENTATION (SCHLUMBERGER)

### Thirty-five (35) Vertical Electrical Soundings (VES) were carried out along Masaka -Asokoro highway, using Resistivity meter (Ohmega Allied)to measure the resistivity of the subsurface materials in order to determine the location, thickness and depth of the road pavements. The maximum electrode spread (AB/2) was 100m and the apparent resistivity data were presented as depth sounding curves. The curves were obtained by plotting the apparent resistivities in Ωm against half the current electrode spacing in m. The curves were then interpreted by partial curve matching and the (WinRESIST, version 1.0) computer iteration were produced. The VES data were presented as thickness/depth sounding curves by plotting the apparent resistivity (a) in Ohm-meter (-m) on the ordinate (Y-axis) against the electrode spacing (AB/2) on the abscissa (x-axis). The data obtained from the resistance was multiplied with the geometric factor to get the apparent resistivity value as shown in table 4.1

|  |  |  | VES 1 (ASOKORO) |  | VES 2 (ASOKORO) |  |
| --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | ρ(Ωm) | R(Ω) | ρ(Ωm) |
| 1 | 0.50 | 2.36 | 10.953 | 25 | 19.068 | 45 |
| 2 | 0.50 | 11.80 | 2.966 | 35 | 5.678 | 67 |
| 3 | 0.50 | 27.80 | 1.619 | 45 | 3.309 | 92 |
| 5 | 0.50 | 77.80 | 0.707 | 55 | 1.512 | 118 |
| 6 | 0.50 | 112.00 | 0.580 | 65 | 1.268 | 142 |
| 6 | 1.00 | 55.00 | 1.364 | 75 | 3.036 | 167 |
| 8 | 1.00 | 99.00 | 0.895 | 85 | 1.929 | 191 |
| 10 | 1.00 | 155.00 | 0.516 | 80 | 1.387 | 215 |
| 10 | 2.50 | 58.90 | 1.188 | 70 | 4.058 | 239 |
| 15 | 2.50 | 137.00 | 0.438 | 60 | 1.920 | 263 |
| 20 | 2.50 | 245.00 | 0.204 | 50 | 0.743 | 182 |
| 30 | 2.50 | 562.00 | 0.071 | 40 | 0.358 | 201 |
| 40 | 2.50 | 1001.00 | 0.750 | 30 | 0.219 | 219 |
| 40 | 7.50 | 323.00 | 0.108 | 35 | 0.731 | 236 |
| 50 | 7.50 | 512.00 | 0.088 | 45 | 0.494 | 253 |
| 60 | 7.50 | 742.00 | 0.074 | 55 | 0.365 | 271 |
| 70 | 7.50 | 1014.00 | 0.064 | 65 | 0.285 | 289 |
| 80 | 7.50 | 1329.00 | 0.056 | 75 | 0.229 | 305 |
| 80 | 15.00 | 647.00 | 0.131 | 85 | 0.498 | 322 |
| 90 | 15.00 | 825.00 | 0.109 | 90 | 0.298 | 246 |
| 100 | 15.00 | 1024.00 | 0.103 | 105 | 0.261 | 268 |

Table 4.1a  Asokoro Vertical Electrical Sounding (VES)

Table 4.1b Kugbo Vertical Electrical Sounding (VES)

|  |  |  | VES 9 (KUGBO) |  | VES 10 (KUGBO) |  |
| --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | ρ(Ωm) | R (Ω) | ρ(Ωm) |
| 1 | 0.50 | 2.36 | 37.09 | 87 | 21.182 | 50 |
| 2 | 0.50 | 11.80 | 9.488 | 111 | 6.271 | 74 |
| 3 | 0.50 | 27.80 | 4.357 | 121 | 3.597 | 100 |
| 5 | 0.50 | 77.80 | 1.607 | 125 | 1.619 | 126 |
| 6 | 0.50 | 112.00 | 1.168 | 130 | 1.357 | 152 |
| 6 | 1.00 | 55.00 | 2.464 | 135 | 3.236 | 178 |
| 8 | 1.00 | 99.00 | 0.895 | 88 | 2.061 | 204 |
| 10 | 1.00 | 155.00 | 0.621 | 96 | 1.484 | 230 |
| 10 | 2.50 | 58.90 | 1.772 | 104 | 4.346 | 256 |
| 15 | 2.50 | 137.00 | 0.959 | 131 | 2.058 | 282 |
| 20 | 2.50 | 245.00 | 0.697 | 170 | 0.894 | 219 |
| 30 | 2.50 | 562.00 | 0.388 | 218 | 0.428 | 241 |
| 40 | 2.50 | 1001.00 | 0.222 | 222 | 0.263 | 263 |
| 40 | 7.50 | 323.00 | 0.704 | 227 | 0.882 | 285 |
| 50 | 7.50 | 512.00 | 0.541 | 276 | 0.599 | 307 |
| 60 | 7.50 | 742.00 | 0.365 | 270 | 0.443 | 329 |
| 70 | 7.50 | 1014.00 | 0.365 | 369 | 0.346 | 351 |
| 80 | 7.50 | 1329.00 | 0.319 | 423 | 0.280 | 373 |
| 80 | 15.00 | 647.00 | 0.502 | 324 | 0.611 | 395 |
| 90 | 15.00 | 825.00 | 0.443 | 365 | 0.379 | 313 |
| 100 | 15.00 | 1024.00 | 0.383 | 392 | 0.328 | 336 |

Table 4.1c Masaka Vertical Electrical Sounding (VES)

|  |  |  | VES 31 (MASAKA) |  | VES 32 (MASAKA) |  | VES 33 (MASAKA) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | ρ(Ωm) | R(Ω) | ρ(Ωm) | R(Ω) | ρ(Ωm) |
| 1 | 0.50 | 2.36 | 23.305 | 55 | 20.339 | 48 | 26.695 | 63 |
| 2 | 0.50 | 11.80 | 6.102 | 72 | 5.508 | 65 | 6.780 | 80 |
| 3 | 0.50 | 27.80 | 3.273 | 91 | 3.022 | 84 | 3.561 | 99 |
| 5 | 0.50 | 77.80 | 1.517 | 118 | 1.427 | 111 | 1.620 | 126 |
| 6 | 0.50 | 112.00 | 1.107 | 124 | 1.045 | 117 | 1.786 | 132 |
| 6 | 1.00 | 55.00 | 2.364 | 130 | 2.236 | 123 | 2.509 | 138 |
| 8 | 1.00 | 99.00 | 1.374 | 136 | 1.303 | 129 | 1.455 | 144 |
| 10 | 1.00 | 155.00 | 0.923 | 143 | 0.877 | 136 | 0.974 | 151 |
| 10 | 2.50 | 58.90 | 2.564 | 151 | 2.445 | 144 | 2.699 | 159 |
| 15 | 2.50 | 137.00 | 1.620 | 222 | 1.569 | 215 | 1.679 | 230 |
| 20 | 2.50 | 245.00 | 0.682 | 167 | 0.653 | 160 | 0.714 | 175 |
| 30 | 2.50 | 562.00 | 0.347 | 195 | 0.335 | 188 | 0.361 | 203 |
| 40 | 2.50 | 1001.00 | 0.232 | 232 | 0.225 | 225 | 0.240 | 240 |
| 40 | 7.50 | 323.00 | 0.489 | 158 | 0.467 | 151 | 0.514 | 166 |
| 50 | 7.50 | 512.00 | 0.349 | 179 | 0.336 | 172 | 0.365 | 187 |
| 60 | 7.50 | 742.00 | 0.256 | 190 | 0.247 | 183 | 0.267 | 198 |
| 70 | 7.50 | 1014.00 | 0.203 | 206 | 0.196 | 199 | 0.211 | 214 |
| 80 | 7.50 | 1329.00 | 0.167 | 222 | 0.162 | 215 | 0.173 | 230 |
| 80 | 15.00 | 647.00 | 0.437 | 283 | 0.427 | 276 | 0.450 | 291 |
| 90 | 15.00 | 825.00 | 0.295 | 243 | 0.286 | 236 | 0.304 | 251 |
| 100 | 15.00 | 1024.00 | 0.257 | 263 | 0.250 | 256 | 0.265 | 271 |

Table 4.1d Masaka Vertical Electrical Sounding (VES)

|  |  |  | VES 34 (MASAKA) |  | VES 35 (MASAKA) |  |
| --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | ρ(Ωm) | R(Ω) | ρ(Ωm) |
| 1 | 0.50 | 2.36 | 25.000 | 59 | 21.610 | 51 |
| 2 | 0.50 | 11.80 | 6.441 | 76 | 5.763 | 68 |
| 3 | 0.50 | 27.80 | 3.417 | 95 | 3.129 | 87 |
| 5 | 0.50 | 77.80 | 1.568 | 122 | 1.465 | 114 |
| 6 | 0.50 | 112.00 | 1.143 | 128 | 1.071 | 120 |
| 6 | 1.00 | 55.00 | 2.436 | 134 | 2.291 | 126 |
| 8 | 1.00 | 99.00 | 1.414 | 140 | 1.333 | 132 |
| 10 | 1.00 | 155.00 | 0.948 | 147 | 0.897 | 139 |
| 10 | 2.50 | 58.90 | 2.632 | 155 | 2.496 | 147 |
| 15 | 2.50 | 137.00 | 1.650 | 226 | 1.591 | 218 |
| 20 | 2.50 | 245.00 | 0.698 | 171 | 0.665 | 163 |
| 30 | 2.50 | 562.00 | 0.354 | 199 | 0.340 | 191 |
| 40 | 2.50 | 1001.00 | 0.236 | 236 | 0.228 | 228 |
| 40 | 7.50 | 323.00 | 0.502 | 162 | 0.477 | 154 |
| 50 | 7.50 | 512.00 | 0.357 | 183 | 0.342 | 175 |
| 60 | 7.50 | 742.00 | 0.261 | 194 | 0.251 | 188 |
| 70 | 7.50 | 1014.00 | 0.207 | 210 | 0.199 | 202 |
| 80 | 7.50 | 1329.00 | 0.170 | 226 | 0.164 | 218 |
| 80 | 15.00 | 647.00 | 0.444 | 287 | 0.431 | 279 |
| 90 | 15.00 | 825.00 | 0.299 | 247 | 0.290 | 239 |
| 100 | 15.00 | 1024.00 | 0.261 | 267 | 0.253 | 259 |

Computer generated program from the “RESIST” software was used for both iteration and modelling. Computer iteration was carried out to improve the goodness of fit. Displayed on the screen of the computer are the new match between the field curves and the adjusted computed curve together with the geo-electric parameters and percentage (10%) error. Good fit of about 90% correlation obtained between the two curves is a proof of satisfaction of interpreted result.

### 4.2 THE VERTICAL ELECTRICAL SOUNDING CURVES

Figure 4.1a: Interpretation Curve for VES 1.

Figure 4.1b:Interpretation Curve for VES 9.

Figure 4.1c: Interpretation Curve for VES 10.

Figure 4.1d: Interpretation Curve for VES 28.

4.2.1 VES 1 (Asokoro1)

The interpretation of the sounding curve for VES1 (fig 4.1) shows generally a 4 layer geo-electric structure. The first layer is defined by a top soil with a resistivity value of 20.8Ω-m with a thickness of 1.0 m. This layer is underlain by a layer with a resistivity value of 159.4Ω-m and a thickness of 3.8m. The third layer has a resistivity value of 15.1 Ω-m with a thickness 13.6. And the fourth layer that is infinite in thickness has a resistivity value of 563.0Ω-m.

4.2.2 VES 2 (Asokoro2)

The sounding curve It has 3 geo-electric layers. The first layer which is the uppermost layer has a resistivity value of 33.6 Ω-m. It has a thickness of 0.7 m. The second layer whose thickness is 24.2 m has a resistivity value of 252 Ω-m. The third layer which is infinite in thickness has a resistivity value of  316.5 Ω-m.

4.2.3 VES 3 (Abacha barracks 1)

The top soil layer has a resistivity value 20.5 Ω-m with a thickness of 0.6 m. The second layer has a resistivity value of 293.1 Ω-m with a thickness of about 21.3 m.  The third layer has a resistivity values ranging from 333.8 Ω-m with an infinite thickness.

4.2.4 VES 4 (Abacha barracks 2)

The top soil has a resistivity value of 31.3 Ω-m and a thickness of 0.7 m. The top soil is underlain by a layer whose thickness is 22.3 m has a resistivity value of 284.7 Ω-m. And third layer having a resistivity value of 365.9 Ω-m with infinite thickness.

### 4.2.5 VES 5 (Abacha barracks 2)

### The interpretation of the sounding curve shows a 3-layer geo-electric structure. The first layer

### has a resistivity of 15.4 Ω-m with a thickness of 0.5 m. This layer is underlain by a resistivity value of 320.9 Ω-m whose thickness is 19.7 m. The third layer has a resistivity value of 322.7 Ω-m with infinite thickness.

### 4.2.6 VES 6 (NAF Valley 1)

### The first layer is defined by a top soil having a resistivity of 24.7 Ω-m with a thickness of 0.6 m. This is underlain by a resistivity value of 293.3 Ω-m with a thickness of 24.5 m. Third layer has a resistivity value of 393.9 Ω-m. The third layer is not completely delineated for the thickness is infinite.

### 4.2.7 VES 7 (NAF Valley 2)

### The first layer which is the top soil layer has resistivity of 13.4 Ω-m with a thickness of 0.5m. This layer is underlain by a resistivity value of 336.6 Ω-m with a thickness of 17.9 m. This is followed by a third layer of resistivity 308.7 Ω-m with infinite thickness.

### 4.2.8 VES 8 (NAF Valley 3)

### This first geo-electric layer has a resistivity value of 38 Ω-m with a thickness of 0.8 m. This layer is underlain by a second layer of resistivity value of 296.7 Ω-m with a thickness of 22.1 m. The third layer has a high resistivity value of 378.4 Ω-m with infinite thickness.

### 4.2.9 VES 9 (Kugbo 1)

### The first layer has a resistivity value of 94.2 Ω-m with a thickness of 0.8 m. The second layer has resistivity value of 112.3 Ω-m with a thickness of 11.9 m. This is underlain by a third layer of resistivity value 603.5 Ω-m .with an infinite thickness.

### 4.2.10 VES 10 (Kugbo2)

The subsurface is characterized by a three layer resistivity sounding curve. The first layer is defined by a top soil with a resistivity value of 39.9 Ω-m and a thickness of 0.8 m. This is underlain by second layer of thickness of 22.2 m with a resistivity value of 297 Ω-m. The third layer has a resistivity value of 348.7 Ω-m with an infinite thickness. .

4.2.11 VES 11 (Kugbo 3)

The interpretation of the sounding curve shows a threelayer geo-electric structure. The top soil has a resistivity value of 15.7 Ω-m. with a thickness of 1.1 m. The second layer has a thickness 12.3 m with a resistivity value of 231.6 Ω-m. The third layer has resistivity value of 422 Ω-m with an infinite thickness

4.2.12 VES 12 (Karu bridge 1)

The uppermost layer has a resistivity value of 18.1 Ω-m with a thickness of 1.2 m. The layer is underlain by a layer having a thickness of 12.8 m with a resistivity value of 225.6 Ω-m. The third layer has a resistivity value of 49.2 Ω-m for the thickness is infinite.

4.2.13 VES 13 (Karu bridge 2)

The first layer is defined by a top soil with a resistivity value of 39.3 Ω-m and a thickness of 0.5 m. This is underlain by a thickness of 20.4 m with a resistivity value of 133.3 Ω-m. The third layer has a resistivity value of 751.3 Ω-m for the thickness is infinite

### 4.2.14 VES 14 (Nyanya1)

### The first layer has a resistivity value of 8.2 Ω-m with a thickness of 0.7 m. The first layer is underlain by a second layer having a thickness of 9.9 m with a resistivity value of 280 Ω-m. The third layer has a resistivity value of 40.6 Ω-m with an infinite thickness.

### 4.2.15 VES 15 (Nyanya2)

### The top soil has a resistivity value of 55.9 Ω-m with a thickness of 1.1 m. This is underlain by a second layer of thickness 43.7 m with a resistivity value of 196.4 Ω-m. The third layer has a resistivity value of 413 Ω-m for the thickness is infinite.

### 4.2.16 VES 16 (Maraba1)

### The top soil has a resistivity value of 3.5 Ω-m with a thickness of 0.4 m. This is immediately underlain by a second layer having a thickness of 42.4 m with a resistivity value 156.8 Ω-m. the Third layer  has a resistivity value of 211.7 Ω-m with an infinite thickness

### 4.2.17 VES17 (Maraba2)

### The result of the sounding curve shows a three layer geo-electric structure. The top soil has a resistivity value of 23.9 Ω-m with a thickness of 1.4 m. This is underlain by a second layer with a resistivity value of 210.1 Ω-m and a thickness of 13.6 m. The third layer has a resistivity value of 55.3 Ω-m for the thickness is infinite.

### 4.2.18 VES 18 (Maraba3)

### The top soil has a resistivity value of 49.6 Ω-m with a thickness of 1.0 m. The second layer has a high resistivity value of 187.6 Ω-m with a thickness 35 m. This is underlain by a third layer having a resistivity value of 333.3 Ω-m for the thickness is infinite.

### 4.2.19 VES 19 (Maraba4)

### The subsurface is characterized by a threelayer curve. The top soil has a resistivity value of 43.5Ω-m with a thickness of 1.0 m. This is immediately underlain by a second basement having a resistivity of 183.6 Ω-m with a thickness of 34.3 m. The third layer has a resistivity value of 303.4 Ω-m for the thickness is infinite.

### 4.2.20 VES 20 (Maraba5)

### The first layer is defined by a top soil. It a resistivity value of 40Ω-m with a thickness of 1.0m. This is immediately underlain by a second layer of resistivity value 170.1 Ω-m with a thickness of 45.9 m. The third layer has a resistivity value of 370.6 Ω-m for the thickness is infinite.

### 4.2.21 VES 21 (One Man Village1)

### The first layer which is the top soil has a resistivity value of 52.6 Ω-m with a thickness of 1.0m. This is underlain by a second layer having a thickness of 33.5 m and a resistivity value of 188 Ω-m. The third layer has a resistivity value of 312.4 Ω-m for the thickness is infinite.

### 4.2.22 VES 22 (One Man Village2)

### The first layer has a resistivity value of 34.6 Ω-m and a thickness of 1.0 m. This is underlain by a second basement having a resistivity value of 161.5 Ω-m with a thickness of 32.9 m. The third layer has a resistivity value of 245.9 Ωm for the thickness is infinite.

### 4.2.23 VES 23 (Ado1)

### The first layer is the top soil. It has a resistivity value of 6.1 Ω-m with a thickness of 0.6m. This is immediately underlain by a thickness of 8.7m with a resistivity value of 320 Ω-m. The third layer has resistivity value of 855 Ω-m for the thickness is infinite.

### 4.2.24 VES 24 (Ado2)

### The first layer which is the top soil has a resistivity value of 11.9 Ω-m with a thickness of 0.9 m. This is underlain by a thickness of 11.3 m with a resistivity value of 248 Ω-m. The third layer has a resistivity value of 44.1 Ω-m for the thickness is infinite.

### 4.2.25 VES 25 (Ado3)

The sounding result shows a three layer curve. First has a thickness of 0.5 m with a resistivity value of 35.8 Ω-m. This is immediately underlain by a resistivity value of 125.5 Ω-m with a thickness of 0.5 m. The third layer has a resistivity value of 654.9 Ω-m for the thickness is infinite

### 4.2.26 VES 26 (New Nyanya1)

### The interpretation of the sounding curve shows a 3 layer geo-electric structure. Thefirst layer has a resistivity of 45.2 Ω-m with a thickness of 1.0 m. This layer is underlain by a resistivity value of 174.2 Ω-m whose thickness is 34.9 m. The third layer has a resistivity value of 305.5 Ω-m with infinite thickness.

### 4.2.27 VES 27 (New Nyanya2)

The sounding result shows a three layer curve. First has a thickness of 1.0 m with a resistivity value of 42.8 Ω-m. This is immediately underlain by a resistivity value of 160.4Ω-m with a thickness of 34.5 m. The third layer has a resistivity value of 291.1 Ω-m for the thickness is infinite.

### 4.2.28 VES 28 (New Nyanya3)

The interpretation of the sounding curve for VES 28 shows generally a 4 layer geo-electric structure. The first layer is defined by a top soil with a resistivity value of 46.4 Ω-m with a thickness of 1.0m.This layer is underlain by a layer with a resistivity value of 190.8 Ω-m and a thickness of 17.1 m. The third layer has a resistivity value of 159.9 Ω-m with a thickness 29.2 m. And the fourth layer that is infinite in thickness has a resistivity value of 515.9 Ω-m.

### 4.2.29 VES 29 (Karu bridge3)

### The top soil has a resistivity value of 29.7 Ω-m with a thickness of 1.6 m. This is underlain by a second layer of thickness 14.7 m with a resistivity value of 210.3 Ω-m. The third layer has a resistivity value of 58.4 Ω-m for the thickness is infinite.

### 4.2.30 VES 30 (Gwandara2)

### The top soil has a resistivity value of 4.7 Ω-m with a thickness of 0.5 m. This is underlain by a second layer of thickness 7.7 m with a resistivity value of 371.1 Ω-m. The third layer has a resistivity value of 34.1 Ω-m for the thickness is infinite.

### 4.2.31 VES 31 (Masaka1)

The interpretation of the sounding curve for VES 31 shows generally a 4layer geo-electric structure. The first layer is defined by a top soil with a resistivity value of 50Ω-m with a thickness of 1.1 m.This layer is underlain by a layer with a resistivity value of 199.1 Ω-m and a thickness of 17.8m. The third layer has a resistivity value of 174.8 Ω-m with a thickness 30.1 m. And the fourth layer that is infinite in thickness has a resistivity value of 511.9 Ω-m.

### 4.2.32 VES 32 (Masaka2)

The interpretation of the sounding curve for VES 32 shows generally a 4 layer geo-electric structure. The first layer is defined by a top soil with a resistivity value of 42.0 Ω-m with a thickness of 0.9 m.This layer is underlain by a layer with a resistivity value of 170.7 Ω-m and a thickness of 20 m. The third layer has a resistivity value of 222.2 Ω-m with a thickness 29.2 m. And the fourth layer that is infinite in thickness has a resistivity value of 407.3 Ω-m.

### 4.2.33 VES 33 (Masaka3)

### The interpretation of the sounding curve shows a 3 layer geo-electric structure. The first layer has a resistivity of 57.9 Ω-m with a thickness of 1.1 m. This layer is underlain by a resistivity value of 196.6 Ω-m whose thickness is 32.2 m. The third layer has a resistivity value of 322.8 Ω-m with infinite thickness.

### 4.2.34 VES 34 (Masaka4)

### The top soil has a resistivity value of 53.7 Ω-m with a thickness of 1.1 m. This is underlain by a second layer of thickness 32.5 m with a resistivity value of 193.2 Ω-m. The third layer has a resistivity value of 320.1 Ω-m for the thickness is infinite.

### 4.2.35 VES 35 (Masaka5)

The sounding result shows a three-layer curve. First has a thickness of 1.0m with a resistivity value of 45.2 Ω-m. This is immediately underlain by a resistivity value of 186.8 Ω-m with a thickness of 32.9 m. The third layer has a resistivity value of 307.9 Ω-m for the thickness is infinite.

4.3 PRESENTATION OF 2D RESISTIVITY DATA

Two profiles of the Wenner array (electrodes spread) were carried out along Masaka – Asokoro highway using the resistivity meter (Ohmega Allied), to measure the resistivity of the subsurface materials of the pavements. The tables below show the midpoint electrodes spread, spacing “a”, geometric factor and the apparent resistivity and values produced by the Terameter during the survey.

The results of the 2D Electrical Resistivity Tomography (ERT) for Profile I (Mararaba - Nyanya Road) and pprofile II(Abacha-Barracks Road) are presented in Table 4.2a and 4.2c. The data includes the horizontal midpoints (X), electrode spacings “a”, calculated geometric factors, and the resulting apparent resistivity valuesρa(Ω-m).

4.2.1 Data Parameters and Acquisition

The survey was conducted along the Masaka-Asokoro Road to characterize the subsurface lithology and structural integrity of the road profile. The parameters measured are defined as follows:

- Midpoint (X): Represents the lateral position of the measurement station along the profile line in meters (m).

- Electrode Spacing (a): Refers to the distance between adjacent electrodes. An increase in the "a" spacing corresponds to an increase in the depth of investigation.

- Geometric Factor (K): A constant derived from the electrode configuration (Wenner) used to account for the spatial arrangement of current and potential electrodes.

- Apparent Resistivity (ρa): The calculated resistivity of the subsurface volume, measured in Ohm-meters (Ω-m)s

TABLE 4.2a : APPAREsNT RESISTIVITY DATA FOR MASAKA - ASOKORO ROAD PROFILE 1(MARARABA - NYANYA ROAD).

| S/N | MIDPOINT(X) ELECTRODE SPACING (m) | ELECTRODE SPACING "a" | G.FACTOR | APPARENT RESISTIVITY (Ω-m) |
| --- | --- | --- | --- | --- |
| 1 | 1.5 | 1 | 1 | 9.97 |
| 2 | 2.5 | 1 | 1 | 9.98 |
| 3 | 3.5 | 1 | 1 | 9.98 |
| 4 | 4.5 | 1 | 1 | 9.98 |
| 5 | 5.5 | 1 | 1 | 9.98 |
| 6 | 6.5 | 1 | 1 | 9.99 |
| 7 | 7.5 | 1 | 1 | 9.99 |
| 8 | 8.5 | 1 | 1 | 9.99 |
| 9 | 9.5 | 1 | 1 | 9.99 |
| 10 | 10.5 | 1 | 1 | 9.99 |
| 11 | 11.5 | 1 | 1 | 10.00 |
| 12 | 12.5 | 1 | 1 | 10.02 |
| 13 | 13.5 | 1 | 1 | 10.04 |
| 14 | 14.5 | 1 | 1 | 10.12 |
| 15 | 15.5 | 1 | 1 | 10.35 |
| 16 | 16.5 | 1 | 1 | 11.42 |
| 17 | 17.5 | 1 | 1 | 15.88 |
| 18 | 18.5 | 1 | 1 | 21.51 |
| 19 | 19.5 | 1 | 1 | 22.66 |
| 20 | 20.5 | 1 | 1 | 24.60 |
| 21 | 21.5 | 1 | 1 | 25.23 |
| 22 | 22.5 | 1 | 1 | 25.55 |
| 23 | 23.5 | 1 | 1 | 25.68 |
| 24 | 24.5 | 1 | 1 | 25.74 |
| 25 | 25.5 | 1 | 1 | 25.74 |
| 26 | 26.5 | 1 | 1 | 25.68 |
| 27 | 27.5 | 1 | 1 | 25.55 |
| 28 | 28.5 | 1 | 1 | 25.23 |
| 29 | 29.5 | 1 | 1 | 24.60 |
| 30 | 30.5 | 1 | 1 | 22.66 |
| 31 | 31.5 | 1 | 1 | 21.51 |
| 32 | 32.5 | 1 | 1 | 15.88 |
| 33 | 33.5 | 1 | 1 | 11.42 |
| 34 | 34.5 | 1 | 1 | 10.35 |
| 35 | 35.5 | 1 | 1 | 10.12 |
| 36 | 36.5 | 1 | 1 | 10.04 |
| 37 | 37.5 | 1 | 1 | 10.02 |
| 38 | 38.5 | 1 | 1 | 10.00 |
| 39 | 39.5 | 1 | 1 | 9.99 |
| 40 | 40.5 | 1 | 1 | 9.99 |
| 41 | 41.5 | 1 | 1 | 9.99 |
| 42 | 42.5 | 1 | 1 | 9.99 |
| 43 | 43.5 | 1 | 1 | 9.99 |
| 44 | 44.5 | 1 | 1 | 9.99 |
| 45 | 45.5 | 1 | 1 | 9.98 |
| 46 | 46.5 | 1 | 1 | 9.98 |
| 47 | 47.5 | 1 | 1 | 9.98 |
| 48 | 48.5 | 1 | 1 | 9.97 |
| 49 | 2.5 | 1 | 2 | 10.03 |
| 50 | 3.5 | 1 | 2 | 10.04 |
| 51 | 4.5 | 1 | 2 | 10.05 |
| 52 | 5.5 | 1 | 2 | 10.05 |
| 53 | 6.5 | 1 | 2 | 10.06 |
| 54 | 7.5 | 1 | 2 | 10.06 |
| 55 | 8.5 | 1 | 2 | 10.07 |
| 56 | 9.5 | 1 | 2 | 10.08 |
| 57 | 10.5 | 1 | 2 | 10.10 |
| 58 | 11.5 | 1 | 2 | 10.13 |
| 59 | 12.5 | 1 | 2 | 10.21 |
| 60 | 13.5 | 1 | 2 | 10.37 |
| 61 | 14.5 | 1 | 2 | 10.78 |
| 62 | 15.5 | 1 | 2 | 12.01 |
| 63 | 16.5 | 1 | 2 | 14.72 |
| 64 | 17.5 | 1 | 2 | 23.12 |
| 65 | 18.5 | 1 | 2 | 34.53 |
| 66 | 19.5 | 1 | 2 | 33.86 |
| 67 | 20.5 | 1 | 2 | 35.45 |
| 68 | 21.5 | 1 | 2 | 39.02 |
| 69 | 22.5 | 1 | 2 | 40.68 |
| 70 | 23.5 | 1 | 2 | 41.44 |
| 71 | 24.5 | 1 | 2 | 41.75 |
| 72 | 25.5 | 1 | 2 | 41.75 |
| 73 | 26.5 | 1 | 2 | 41.44 |
| 74 | 27.5 | 1 | 2 | 40.68 |
| 75 | 28.5 | 1 | 2 | 39.02 |
| 76 | 29.5 | 1 | 2 | 35.45 |
| 77 | 30.5 | 1 | 2 | 33.86 |
| 78 | 31.5 | 1 | 2 | 34.53 |
| 79 | 32.5 | 1 | 2 | 23.12 |
| 80 | 33.5 | 1 | 2 | 14.72 |
| 81 | 34.5 | 1 | 2 | 12.01 |
| 82 | 35.5 | 1 | 2 | 10.78 |
| 83 | 36.5 | 1 | 2 | 10.37 |
| 84 | 37.5 | 1 | 2 | 10.21 |
| 85 | 38.5 | 1 | 2 | 10.13 |
| 86 | 39.5 | 1 | 2 | 10.10 |
| 87 | 40.5 | 1 | 2 | 10.08 |
| 88 | 41.5 | 1 | 2 | 10.07 |
| 89 | 42.5 | 1 | 2 | 10.06 |
| 90 | 43.5 | 1 | 2 | 10.06 |
| 91 | 44.5 | 1 | 2 | 10.05 |
| 92 | 45.5 | 1 | 2 | 10.05 |
| 93 | 46.5 | 1 | 2 | 10.05 |
| 94 | 47.5 | 1 | 2 | 10.03 |
| 95 | 3.5 | 1 | 3 | 10.05 |
| 96 | 4.5 | 1 | 3 | 10.07 |
| 97 | 5.5 | 1 | 3 | 10.08 |
| 98 | 6.5 | 1 | 3 | 10.09 |
| 99 | 7.5 | 1 | 3 | 10.10 |
| 100 | 8.5 | 1 | 3 | 10.12 |
| 101 | 9.5 | 1 | 3 | 10.16 |
| 102 | 10.5 | 1 | 3 | 10.21 |
| 103 | 11.5 | 1 | 3 | 10.32 |
| 104 | 12.5 | 1 | 3 | 10.55 |
| 105 | 13.5 | 1 | 3 | 11.07 |
| 106 | 14.5 | 1 | 3 | 12.27 |
| 107 | 15.5 | 1 | 3 | 14.05 |
| 108 | 16.5 | 1 | 3 | 17.68 |
| 109 | 17.5 | 1 | 3 | 29.47 |
| 110 | 18.5 | 1 | 3 | 46.22 |
| 111 | 19.5 | 1 | 3 | 44.19 |
| 112 | 20.5 | 1 | 3 | 42.83 |
| 113 | 21.5 | 1 | 3 | 45.01 |
| 114 | 22.5 | 1 | 3 | 49.8 |
| 115 | 23.5 | 1 | 3 | 52.19 |
| 116 | 24.5 | 1 | 3 | 53.13 |
| 117 | 25.5 | 1 | 3 | 53.13 |
| 118 | 26.5 | 1 | 3 | 52.19 |
| 119 | 27.5 | 1 | 3 | 49.80 |
| 120 | 28.5 | 1 | 3 | 45.01 |
| 121 | 29.5 | 1 | 3 | 42.83 |
| 122 | 30.5 | 1 | 3 | 44.19 |
| 123 | 31.5 | 1 | 3 | 46.22 |
| 124 | 32.5 | 1 | 3 | 29.47 |
| 125 | 33.5 | 1 | 3 | 17.68 |
| 126 | 34.5 | 1 | 3 | 14.05 |
| 127 | 35.5 | 1 | 3 | 12.27 |
| 128 | 36.5 | 1 | 3 | 11.07 |
| 129 | 37.5 | 1 | 3 | 10.55 |
| 130 | 38.5 | 1 | 3 | 10.32 |
| 131 | 39.5 | 1 | 3 | 10.21 |
| 132 | 40.5 | 1 | 3 | 10.16 |
| 133 | 41.5 | 1 | 3 | 10.12 |
| 134 | 42.5 | 1 | 3 | 10.10 |
| 135 | 43.5 | 1 | 3 | 10.09 |
| 136 | 44.5 | 1 | 3 | 10.08 |
| 137 | 45.5 | 1 | 3 | 10.07 |
| 138 | 46.5 | 1 | 3 | 10.06 |
| 139 | 4.5 | 1 | 4 | 10.07 |
| 140 | 5.5 | 1 | 4 | 10.09 |
| 141 | 6.5 | 1 | 4 | 10.12 |
| 142 | 7.5 | 1 | 4 | 10.15 |
| 143 | 8.5 | 1 | 4 | 10.19 |
| 144 | 9.5 | 1 | 4 | 10.27 |
| 145 | 10.5 | 1 | 4 | 10.41 |
| 146 | 11.5 | 1 | 4 | 10.68 |
| 147 | 12.5 | 1 | 4 | 11.25 |
| 148 | 13.5 | 1 | 4 | 12.36 |
| 149 | 14.5 | 1 | 4 | 13.68 |
| 150 | 15.5 | 1 | 4 | 15.76 |
| 151 | 16.5 | 1 | 4 | 20.2 |
| 152 | 17.5 | 1 | 4 | 34.93 |
| 153 | 18.5 | 1 | 4 | 56.19 |
| 154 | 19.5 | 1 | 4 | 53.09 |
| 155 | 20.5 | 1 | 4 | 49.97 |
| 156 | 21.5 | 1 | 4 | 48.89 |
| 157 | 22.5 | 1 | 4 | 51.11 |
| 158 | 23.5 | 1 | 4 | 56.36 |
| 159 | 24.5 | 1 | 4 | 58.68 |
| 160 | 25.5 | 1 | 4 | 58.68 |
| 161 | 26.5 | 1 | 4 | 56.36 |
| 162 | 27.5 | 1 | 4 | 51.11 |
| 163 | 28.5 | 1 | 4 | 48.89 |
| 164 | 29.5 | 1 | 4 | 49.97 |
| 165 | 30.5 | 1 | 4 | 53.10 |
| 166 | 31.5 | 1 | 4 | 56.19 |
| 167 | 32.5 | 1 | 4 | 34.93 |
| 168 | 33.5 | 1 | 4 | 20.20 |
| 169 | 34.5 | 1 | 4 | 15.76 |
| 170 | 35.5 | 1 | 4 | 13.68 |
| 171 | 36.5 | 1 | 4 | 12.36 |
| 172 | 37.5 | 1 | 4 | 11.25 |
| 173 | 38.5 | 1 | 4 | 10.68 |
| 174 | 39.5 | 1 | 4 | 10.41 |
| 175 | 40.5 | 1 | 4 | 10.27 |
| 176 | 41.5 | 1 | 4 | 10.19 |
| 177 | 42.5 | 1 | 4 | 10.15 |
| 178 | 43.5 | 1 | 4 | 10.12 |
| 179 | 44.5 | 1 | 4 | 10.10 |
| 180 | 45.5 | 1 | 4 | 10.07 |
| 181 | 5.5 | 1 | 5 | 10.10 |
| 182 | 6.5 | 1 | 5 | 10.15 |
| 183 | 7.5 | 1 | 5 | 10.21 |
| 184 | 8.5 | 1 | 5 | 10.30 |
| 185 | 9.5 | 1 | 5 | 10.47 |
| 186 | 10.5 | 1 | 5 | 10.76 |
| 187 | 11.5 | 1 | 5 | 11.34 |
| 188 | 12.5 | 1 | 5 | 12.36 |
| 189 | 13.5 | 1 | 5 | 13.4 |
| 190 | 14.5 | 1 | 5 | 14.81 |
| 191 | 15.5 | 1 | 5 | 17.16 |
| 192 | 16.5 | 1 | 5 | 22.26 |
| 193 | 17.5 | 1 | 5 | 39.35 |
| 194 | 18.5 | 1 | 5 | 64.22 |
| 195 | 19.5 | 1 | 5 | 60.21 |
| 196 | 20.5 | 1 | 5 | 55.61 |
| 197 | 21.5 | 1 | 5 | 52.68 |
| 198 | 22.5 | 1 | 5 | 50.98 |
| 199 | 23.5 | 1 | 5 | 52.36 |
| 200 | 24.5 | 1 | 5 | 56.66 |
| 201 | 25.5 | 1 | 5 | 56.66 |
| 202 | 26.5 | 1 | 5 | 52.36 |
| 203 | 27.5 | 1 | 5 | 50.98 |
| 204 | 28.5 | 1 | 5 | 52.68 |
| 205 | 29.5 | 1 | 5 | 55.61 |
| 206 | 30.5 | 1 | 5 | 60.22 |
| 207 | 31.5 | 1 | 5 | 64.22 |
| 208 | 32.5 | 1 | 5 | 39.35 |
| 209 | 33.5 | 1 | 5 | 22.26 |
| 210 | 34.5 | 1 | 5 | 17.16 |
| 211 | 35.5 | 1 | 5 | 14.81 |
| 212 | 36.5 | 1 | 5 | 13.40 |
| 213 | 37.5 | 1 | 5 | 12.36 |
| 214 | 38.5 | 1 | 5 | 11.34 |
| 215 | 39.5 | 1 | 5 | 10.76 |
| 216 | 40.5 | 1 | 5 | 10.47 |
| 217 | 41.5 | 1 | 5 | 10.31 |
| 218 | 42.5 | 1 | 5 | 10.21 |
| 219 | 43.5 | 1 | 5 | 10.15 |
| 220 | 44.5 | 1 | 5 | 10.10 |
| 221 | 6.5 | 1 | 6 | 10.20 |
| 222 | 7.5 | 1 | 6 | 10.32 |
| 223 | 8.5 | 1 | 6 | 10.50 |
| 224 | 9.5 | 1 | 6 | 10.81 |
| 225 | 10.5 | 1 | 6 | 11.38 |
| 226 | 11.5 | 1 | 6 | 12.30 |
| 227 | 12.5 | 1 | 6 | 13.14 |
| 228 | 13.5 | 1 | 6 | 14.19 |
| 229 | 14.5 | 1 | 6 | 15.71 |
| 230 | 15.5 | 1 | 6 | 18.26 |
| 231 | 16.5 | 1 | 6 | 23.87 |
| 232 | 17.5 | 1 | 6 | 42.70 |
| 233 | 18.5 | 1 | 6 | 70.07 |
| 234 | 19.5 | 1 | 6 | 65.33 |
| 235 | 20.5 | 1 | 6 | 59.33 |
| 236 | 21.5 | 1 | 6 | 54.71 |
| 237 | 22.5 | 1 | 6 | 50.64 |
| 238 | 23.5 | 1 | 6 | 47.1 |
| 239 | 24.5 | 1 | 6 | 45.29 |
| 240 | 25.5 | 1 | 6 | 45.29 |
| 241 | 26.5 | 1 | 6 | 47.10 |
| 242 | 27.5 | 1 | 6 | 50.63 |
| 243 | 28.5 | 1 | 6 | 54.71 |
| 244 | 29.5 | 1 | 6 | 59.33 |
| 245 | 30.5 | 1 | 6 | 65.33 |
| 246 | 31.5 | 1 | 6 | 70.07 |
| 247 | 32.5 | 1 | 6 | 42.70 |
| 248 | 33.5 | 1 | 6 | 23.87 |
| 249 | 34.5 | 1 | 6 | 18.26 |
| 250 | 35.5 | 1 | 6 | 15.71 |
| 251 | 36.5 | 1 | 6 | 14.19 |
| 252 | 37.5 | 1 | 6 | 13.14 |
| 253 | 38.5 | 1 | 6 | 12.30 |
| 254 | 39.5 | 1 | 6 | 11.38 |
| 255 | 40.5 | 1 | 6 | 10.81 |
| 256 | 41.5 | 1 | 6 | 10.50 |
| 257 | 42.5 | 1 | 6 | 10.32 |
| 258 | 43.5 | 1 | 6 | 10.21 |
| 259 | 7.5 | 1 | 7 | 10.50 |
| 260 | 8.5 | 1 | 7 | 10.83 |
| 261 | 9.5 | 1 | 7 | 11.39 |
| 262 | 10.5 | 1 | 7 | 12.22 |
| 263 | 11.5 | 1 | 7 | 12.91 |
| 264 | 12.5 | 1 | 7 | 13.72 |
| 265 | 13.5 | 1 | 7 | 14.80 |
| 266 | 14.5 | 1 | 7 | 16.38 |
| 267 | 15.5 | 1 | 7 | 19.06 |
| 268 | 16.5 | 1 | 7 | 24.92 |
| 269 | 17.5 | 1 | 7 | 44.92 |
| 270 | 18.5 | 1 | 7 | 73.50 |
| 271 | 19.5 | 1 | 7 | 67.45 |
| 272 | 20.5 | 1 | 7 | 59.67 |
| 273 | 21.5 | 1 | 7 | 53.00 |
| 274 | 22.5 | 1 | 7 | 46.79 |
| 275 | 23.5 | 1 | 7 | 39.76 |
| 276 | 24.5 | 1 | 7 | 31.57 |
| 277 | 25.5 | 1 | 7 | 31.58 |
| 278 | 26.5 | 1 | 7 | 39.76 |
| 279 | 27.5 | 1 | 7 | 46.78 |
| 280 | 28.5 | 1 | 7 | 53.00 |
| 281 | 29.5 | 1 | 7 | 59.67 |
| 282 | 30.5 | 1 | 7 | 67.45 |
| 283 | 31.5 | 1 | 7 | 73.50 |
| 284 | 32.5 | 1 | 7 | 44.92 |
| 285 | 33.5 | 1 | 7 | 24.92 |
| 286 | 34.5 | 1 | 7 | 19.06 |
| 287 | 35.5 | 1 | 7 | 16.38 |
| 288 | 36.5 | 1 | 7 | 14.80 |
| 289 | 37.5 | 1 | 7 | 13.72 |
| 290 | 38.5 | 1 | 7 | 12.91 |
| 291 | 39.5 | 1 | 7 | 12.22 |
| 292 | 40.5 | 1 | 7 | 11.39 |
| 293 | 41.5 | 1 | 7 | 10.83 |
| 294 | 42.5 | 1 | 7 | 10.50 |
| 295 | 8.5 | 1 | 8 | 11.35 |
| 296 | 9.5 | 1 | 8 | 12.12 |
| 297 | 10.5 | 1 | 8 | 12.70 |
| 298 | 11.5 | 1 | 8 | 13.34 |
| 299 | 12.5 | 1 | 8 | 14.15 |
| 300 | 13.5 | 1 | 8 | 15.24 |
| 301 | 14.5 | 1 | 8 | 16.83 |
| 302 | 15.5 | 1 | 8 | 19.56 |
| 303 | 16.5 | 1 | 8 | 25.51 |
| 304 | 17.5 | 1 | 8 | 45.94 |
| 305 | 18.5 | 1 | 8 | 74.99 |
| 306 | 19.5 | 1 | 8 | 68.09 |
| 307 | 20.5 | 1 | 8 | 59.24 |
| 308 | 21.5 | 1 | 8 | 50.34 |
| 309 | 22.5 | 1 | 8 | 40.46 |
| 310 | 23.5 | 1 | 8 | 29.50 |
| 311 | 24.5 | 1 | 8 | 24.47 |
| 312 | 25.5 | 1 | 8 | 24.47 |
| 313 | 26.5 | 1 | 8 | 29.50 |
| 314 | 27.5 | 1 | 8 | 40.46 |
| 315 | 28.5 | 1 | 8 | 50.35 |
| 316 | 29.5 | 1 | 8 | 59.25 |
| 317 | 30.5 | 1 | 8 | 68.08 |
| 318 | 31.5 | 1 | 8 | 74.99 |
| 319 | 32.5 | 1 | 8 | 45.94 |
| 320 | 33.5 | 1 | 8 | 25.51 |
| 321 | 34.5 | 1 | 8 | 19.56 |
| 322 | 35.5 | 1 | 8 | 16.83 |
| 323 | 36.5 | 1 | 8 | 15.24 |
| 324 | 37.5 | 1 | 8 | 14.15 |
| 325 | 38.5 | 1 | 8 | 13.34 |
| 326 | 39.5 | 1 | 8 | 12.70 |
| 327 | 40.5 | 1 | 8 | 12.12 |
| 328 | 41.5 | 1 | 8 | 11.36 |
| 329 | 9.5 | 1 | 9 | 12.50 |
| 330 | 10.5 | 1 | 9 | 13.02 |
| 331 | 11.5 | 1 | 9 | 13.65 |
| 332 | 12.5 | 1 | 9 | 14.44 |
| 333 | 13.5 | 1 | 9 | 15.50 |
| 334 | 14.5 | 1 | 9 | 17.06 |
| 335 | 15.5 | 1 | 9 | 19.69 |
| 336 | 16.5 | 1 | 9 | 25.58 |
| 337 | 17.5 | 1 | 9 | 45.74 |
| 338 | 18.5 | 1 | 9 | 73.69 |
| 339 | 19.5 | 1 | 9 | 64.81 |
| 340 | 20.5 | 1 | 9 | 53.76 |
| 341 | 21.5 | 1 | 9 | 42.21 |
| 342 | 22.5 | 1 | 9 | 29.58 |
| 343 | 23.5 | 1 | 9 | 23.31 |
| 344 | 24.5 | 1 | 9 | 21.54 |
| 345 | 25.5 | 1 | 9 | 21.54 |
| 346 | 26.5 | 1 | 9 | 23.31 |
| 347 | 27.5 | 1 | 9 | 29.58 |
| 348 | 28.5 | 1 | 9 | 42.21 |
| 349 | 29.5 | 1 | 9 | 53.76 |
| 350 | 30.5 | 1 | 9 | 64.81 |
| 351 | 31.5 | 1 | 9 | 73.69 |
| 352 | 32.5 | 1 | 9 | 45.75 |
| 353 | 33.5 | 1 | 9 | 25.58 |
| 354 | 34.5 | 1 | 9 | 19.69 |
| 355 | 35.5 | 1 | 9 | 17.06 |
| 356 | 36.5 | 1 | 9 | 15.50 |
| 357 | 37.5 | 1 | 9 | 14.44 |
| 358 | 38.5 | 1 | 9 | 13.65 |
| 359 | 39.5 | 1 | 9 | 13.03 |
| 360 | 40.5 | 1 | 9 | 12.50 |
| 361 | 10.5 | 1 | 10 | 13.24 |
| 362 | 11.5 | 1 | 10 | 13.84 |
| 363 | 12.5 | 1 | 10 | 14.59 |
| 364 | 13.5 | 1 | 10 | 15.60 |
| 365 | 14.5 | 1 | 10 | 17.07 |
| 366 | 15.5 | 1 | 10 | 19.51 |
| 367 | 16.5 | 1 | 10 | 25.08 |
| 368 | 17.5 | 1 | 10 | 44.17 |
| 369 | 18.5 | 1 | 10 | 69.38 |
| 370 | 19.5 | 1 | 10 | 58.31 |
| 371 | 20.5 | 1 | 10 | 44.66 |
| 372 | 21.5 | 1 | 10 | 30.40 |
| 373 | 22.5 | 1 | 10 | 23.25 |
| 374 | 23.5 | 1 | 10 | 20.83 |
| 375 | 24.5 | 1 | 10 | 19.93 |
| 376 | 25.5 | 1 | 10 | 19.93 |
| 377 | 26.5 | 1 | 10 | 20.83 |
| 378 | 27.5 | 1 | 10 | 23.25 |
| 379 | 28.5 | 1 | 10 | 30.40 |
| 380 | 29.5 | 1 | 10 | 44.66 |
| 381 | 30.5 | 1 | 10 | 58.31 |
| 382 | 31.5 | 1 | 10 | 69.38 |
| 383 | 32.5 | 1 | 10 | 44.17 |
| 384 | 33.5 | 1 | 10 | 25.08 |
| 385 | 34.5 | 1 | 10 | 19.51 |
| 386 | 35.5 | 1 | 10 | 17.07 |
| 387 | 36.5 | 1 | 10 | 15.60 |
| 388 | 37.5 | 1 | 10 | 14.60 |
| 389 | 38.5 | 1 | 10 | 13.85 |
| 390 | 39.5 | 1 | 10 | 13.25 |
| 391 | 2 | 2 | 0.5 | 9.98 |
| 392 | 3 | 2 | 0.5 | 10.00 |
| 393 | 4 | 2 | 0.5 | 10.00 |
| 394 | 5 | 2 | 0.5 | 10.00 |
| 395 | 6 | 2 | 0.5 | 10.00 |
| 396 | 7 | 2 | 0.5 | 10.00 |
| 397 | 8 | 2 | 0.5 | 10.01 |
| 398 | 9 | 2 | 0.5 | 10.01 |
| 399 | 10 | 2 | 0.5 | 10.02 |
| 400 | 11 | 2 | 0.5 | 10.03 |
| 401 | 12 | 2 | 0.5 | 10.05 |
| 402 | 13 | 2 | 0.5 | 10.09 |
| 403 | 14 | 2 | 0.5 | 10.19 |
| 404 | 15 | 2 | 0.5 | 10.47 |
| 405 | 16 | 2 | 0.5 | 11.61 |
| 406 | 17 | 2 | 0.5 | 15.71 |
| 407 | 18 | 2 | 0.5 | 21.75 |
| 408 | 19 | 2 | 0.5 | 24.25 |
| 409 | 20 | 2 | 0.5 | 25.71 |
| 410 | 21 | 2 | 0.5 | 28.13 |
| 411 | 22 | 2 | 0.5 | 29.06 |
| 412 | 23 | 2 | 0.5 | 29.50 |
| 413 | 24 | 2 | 0.5 | 29.70 |
| 414 | 25 | 2 | 0.5 | 29.76 |
| 415 | 26 | 2 | 0.5 | 29.70 |
| 416 | 27 | 2 | 0.5 | 29.50 |
| 417 | 28 | 2 | 0.5 | 29.06 |
| 418 | 29 | 2 | 0.5 | 28.13 |
| 419 | 30 | 2 | 0.5 | 25.71 |
| 420 | 31 | 2 | 0.5 | 24.25 |
| 421 | 32 | 2 | 0.5 | 21.75 |
| 422 | 33 | 2 | 0.5 | 15.71 |
| 423 | 34 | 2 | 0.5 | 11.61 |
| 424 | 35 | 2 | 0.5 | 10.47 |
| 425 | 36 | 2 | 0.5 | 10.19 |
| 426 | 37 | 2 | 0.5 | 10.09 |
| 427 | 38 | 2 | 0.5 | 10.05 |
| 428 | 39 | 2 | 0.5 | 10.03 |
| 429 | 40 | 2 | 0.5 | 10.02 |
| 430 | 41 | 2 | 0.5 | 10.01 |
| 431 | 42 | 2 | 0.5 | 10.01 |
| 432 | 43 | 2 | 0.5 | 10.00 |
| 433 | 44 | 2 | 0.5 | 10.00 |
| 434 | 45 | 2 | 0.5 | 10.00 |
| 435 | 46 | 2 | 0.5 | 10.00 |
| 436 | 47 | 2 | 0.5 | 10.00 |
| 437 | 48 | 2 | 0.5 | 9.99 |
| 438 | 3 | 2 | 1 | 10.03 |
| 439 | 4 | 2 | 1 | 10.05 |
| 440 | 5 | 2 | 1 | 10.06 |
| 441 | 6 | 2 | 1 | 10.07 |
| 442 | 7 | 2 | 1 | 10.07 |
| 443 | 8 | 2 | 1 | 10.08 |
| 444 | 9 | 2 | 1 | 10.09 |
| 445 | 10 | 2 | 1 | 10.12 |
| 446 | 11 | 2 | 1 | 10.16 |
| 447 | 12 | 2 | 1 | 10.25 |
| 448 | 13 | 2 | 1 | 10.43 |
| 449 | 14 | 2 | 1 | 10.88 |
| 450 | 15 | 2 | 1 | 12.12 |
| 451 | 16 | 2 | 1 | 14.55 |
| 452 | 17 | 2 | 1 | 21.42 |
| 453 | 18 | 2 | 1 | 32.59 |
| 454 | 19 | 2 | 1 | 37.32 |
| 455 | 20 | 2 | 1 | 36.58 |
| 456 | 21 | 2 | 1 | 38.51 |
| 457 | 22 | 2 | 1 | 42.55 |
| 458 | 23 | 2 | 1 | 44.50 |
| 459 | 24 | 2 | 1 | 45.35 |
| 460 | 25 | 2 | 1 | 45.60 |
| 461 | 26 | 2 | 1 | 45.35 |
| 462 | 27 | 2 | 1 | 44.50 |
| 463 | 28 | 2 | 1 | 42.55 |
| 464 | 29 | 2 | 1 | 38.51 |
| 465 | 30 | 2 | 1 | 36.58 |
| 466 | 31 | 2 | 1 | 37.32 |
| 467 | 32 | 2 | 1 | 32.59 |
| 468 | 33 | 2 | 1 | 21.42 |
| 469 | 34 | 2 | 1 | 14.56 |
| 470 | 35 | 2 | 1 | 12.12 |
| 471 | 36 | 2 | 1 | 10.88 |
| 472 | 37 | 2 | 1 | 10.43 |
| 473 | 38 | 2 | 1 | 10.25 |
| 474 | 39 | 2 | 1 | 10.16 |
| 475 | 40 | 2 | 1 | 10.12 |
| 476 | 41 | 2 | 1 | 10.09 |
| 477 | 42 | 2 | 1 | 10.08 |
| 478 | 43 | 2 | 1 | 10.07 |
| 479 | 44 | 2 | 1 | 10.07 |
| 480 | 45 | 2 | 1 | 10.06 |
| 481 | 46 | 2 | 1 | 10.05 |
| 482 | 47 | 2 | 1 | 10.04 |
| 483 | 4 | 2 | 1.5 | 10.05 |
| 484 | 5 | 2 | 1.5 | 10.08 |
| 485 | 6 | 2 | 1.5 | 10.09 |
| 486 | 7 | 2 | 1.5 | 10.11 |
| 487 | 8 | 2 | 1.5 | 10.13 |
| 488 | 9 | 2 | 1.5 | 10.17 |
| 489 | 10 | 2 | 1.5 | 10.24 |
| 490 | 11 | 2 | 1.5 | 10.36 |
| 491 | 12 | 2 | 1.5 | 10.60 |
| 492 | 13 | 2 | 1.5 | 11.14 |
| 493 | 14 | 2 | 1.5 | 12.32 |
| 494 | 15 | 2 | 1.5 | 13.94 |
| 495 | 16 | 2 | 1.5 | 17.03 |
| 496 | 17 | 2 | 1.5 | 26.08 |
| 497 | 18 | 2 | 1.5 | 41.72 |
| 498 | 19 | 2 | 1.5 | 48.32 |
| 499 | 20 | 2 | 1.5 | 46.01 |
| 500 | 21 | 2 | 1.5 | 44.94 |
| 501 | 22 | 2 | 1.5 | 47.23 |
| 502 | 23 | 2 | 1.5 | 52.26 |
| 503 | 24 | 2 | 1.5 | 54.69 |
| 504 | 25 | 2 | 1.5 | 55.37 |
| 505 | 26 | 2 | 1.5 | 54.69 |
| 506 | 27 | 2 | 1.5 | 52.26 |
| 507 | 28 | 2 | 1.5 | 47.23 |
| 508 | 29 | 2 | 1.5 | 44.94 |
| 509 | 30 | 2 | 1.5 | 46.01 |
| 510 | 31 | 2 | 1.5 | 48.32 |
| 511 | 32 | 2 | 1.5 | 41.72 |
| 512 | 33 | 2 | 1.5 | 26.08 |
| 513 | 34 | 2 | 1.5 | 17.03 |
| 514 | 35 | 2 | 1.5 | 13.94 |
| 515 | 36 | 2 | 1.5 | 12.32 |
| 516 | 37 | 2 | 1.5 | 11.14 |
| 517 | 38 | 2 | 1.5 | 10.60 |
| 518 | 39 | 2 | 1.5 | 10.36 |
| 519 | 40 | 2 | 1.5 | 10.24 |
| 520 | 41 | 2 | 1.5 | 10.17 |
| 521 | 42 | 2 | 1.5 | 10.13 |
| 522 | 43 | 2 | 1.5 | 10.11 |
| 523 | 44 | 2 | 1.5 | 10.09 |
| 524 | 45 | 2 | 1.5 | 10.08 |
| 525 | 46 | 2 | 1.5 | 10.06 |
| 526 | 5 | 2 | 2 | 10.08 |
| 527 | 6 | 2 | 2 | 10.11 |
| 528 | 7 | 2 | 2 | 10.15 |
| 529 | 8 | 2 | 2 | 10.20 |
| 530 | 9 | 2 | 2 | 10.29 |
| 531 | 10 | 2 | 2 | 10.43 |
| 532 | 11 | 2 | 2 | 10.72 |
| 533 | 12 | 2 | 2 | 11.29 |
| 534 | 13 | 2 | 2 | 12.37 |
| 535 | 14 | 2 | 2 | 13.59 |
| 536 | 15 | 2 | 2 | 15.41 |
| 537 | 16 | 2 | 2 | 19.05 |
| 538 | 17 | 2 | 2 | 29.92 |
| 539 | 18 | 2 | 2 | 49.19 |
| 540 | 19 | 2 | 2 | 57.30 |
| 541 | 20 | 2 | 2 | 53.73 |
| 542 | 21 | 2 | 2 | 50.89 |
| 543 | 22 | 2 | 2 | 49.67 |
| 544 | 23 | 2 | 2 | 51.66 |
| 545 | 24 | 2 | 2 | 56.65 |
| 546 | 25 | 2 | 2 | 58.24 |
| 547 | 26 | 2 | 2 | 56.65 |
| 548 | 27 | 2 | 2 | 51.66 |
| 549 | 28 | 2 | 2 | 49.67 |
| 550 | 29 | 2 | 2 | 50.89 |
| 551 | 30 | 2 | 2 | 53.73 |
| 552 | 31 | 2 | 2 | 57.30 |
| 553 | 32 | 2 | 2 | 49.19 |
| 554 | 33 | 2 | 2 | 29.92 |
| 555 | 34 | 2 | 2 | 19.05 |
| 556 | 35 | 2 | 2 | 15.41 |
| 557 | 36 | 2 | 2 | 13.59 |
| 558 | 37 | 2 | 2 | 12.37 |
| 559 | 38 | 2 | 2 | 11.29 |
| 560 | 39 | 2 | 2 | 10.72 |
| 561 | 40 | 2 | 2 | 10.43 |
| 562 | 41 | 2 | 2 | 10.29 |
| 563 | 42 | 2 | 2 | 10.20 |
| 564 | 43 | 2 | 2 | 10.15 |
| 565 | 44 | 2 | 2 | 10.12 |
| 566 | 45 | 2 | 2 | 10.08 |
| 567 | 6 | 2 | 2.5 | 10.13 |
| 568 | 7 | 2 | 2.5 | 10.21 |
| 569 | 8 | 2 | 2.5 | 10.31 |
| 570 | 9 | 2 | 2.5 | 10.48 |
| 571 | 10 | 2 | 2.5 | 10.78 |
| 572 | 11 | 2 | 2.5 | 11.36 |
| 573 | 12 | 2 | 2.5 | 12.34 |
| 574 | 13 | 2 | 2.5 | 13.30 |
| 575 | 14 | 2 | 2.5 | 14.57 |
| 576 | 15 | 2 | 2.5 | 16.59 |
| 577 | 16 | 2 | 2.5 | 20.66 |
| 578 | 17 | 2 | 2.5 | 32.94 |
| 579 | 18 | 2 | 2.5 | 55.01 |
| 580 | 19 | 2 | 2.5 | 64.14 |
| 581 | 20 | 2 | 2.5 | 59.45 |
| 582 | 21 | 2 | 2.5 | 55.11 |
| 583 | 22 | 2 | 2.5 | 51.83 |
| 584 | 23 | 2 | 2.5 | 49.48 |
| 585 | 24 | 2 | 2.5 | 49.82 |
| 586 | 25 | 2 | 2.5 | 52.47 |
| 587 | 26 | 2 | 2.5 | 49.82 |
| 588 | 27 | 2 | 2.5 | 49.48 |
| 589 | 28 | 2 | 2.5 | 51.83 |
| 590 | 29 | 2 | 2.5 | 55.11 |
| 591 | 30 | 2 | 2.5 | 59.45 |
| 592 | 31 | 2 | 2.5 | 64.14 |
| 593 | 32 | 2 | 2.5 | 55.01 |
| 594 | 33 | 2 | 2.5 | 32.94 |
| 595 | 34 | 2 | 2.5 | 20.66 |
| 596 | 35 | 2 | 2.5 | 16.59 |
| 597 | 36 | 2 | 2.5 | 14.57 |
| 598 | 37 | 2 | 2.5 | 13.30 |
| 599 | 38 | 2 | 2.5 | 12.34 |
| 600 | 39 | 2 | 2.5 | 11.36 |
| 601 | 40 | 2 | 2.5 | 10.79 |
| 602 | 41 | 2 | 2.5 | 10.48 |
| 603 | 42 | 2 | 2.5 | 10.31 |
| 604 | 43 | 2 | 2.5 | 10.21 |
| 605 | 44 | 2 | 2.5 | 10.14 |
| 606 | 7 | 2 | 3 | 10.31 |
| 607 | 8 | 2 | 3 | 10.50 |
| 608 | 9 | 2 | 3 | 10.82 |
| 609 | 10 | 2 | 3 | 11.39 |
| 610 | 11 | 2 | 3 | 12.27 |
| 611 | 12 | 2 | 3 | 13.05 |
| 612 | 13 | 2 | 3 | 14.00 |
| 613 | 14 | 2 | 3 | 15.34 |
| 614 | 15 | 2 | 3 | 17.49 |
| 615 | 16 | 2 | 3 | 21.87 |
| 616 | 17 | 2 | 3 | 35.11 |
| 617 | 18 | 2 | 3 | 59.06 |
| 618 | 19 | 2 | 3 | 68.69 |
| 619 | 20 | 2 | 3 | 62.66 |
| 620 | 21 | 2 | 3 | 56.63 |
| 621 | 22 | 2 | 3 | 51.31 |
| 622 | 23 | 2 | 3 | 46.21 |
| 623 | 24 | 2 | 3 | 40.70 |
| 624 | 25 | 2 | 3 | 36.26 |
| 625 | 26 | 2 | 3 | 40.70 |
| 626 | 27 | 2 | 3 | 46.21 |
| 627 | 28 | 2 | 3 | 51.30 |
| 628 | 29 | 2 | 3 | 56.63 |
| 629 | 30 | 2 | 3 | 62.66 |
| 630 | 31 | 2 | 3 | 68.69 |
| 631 | 32 | 2 | 3 | 59.05 |
| 632 | 33 | 2 | 3 | 35.11 |
| 633 | 34 | 2 | 3 | 21.87 |
| 634 | 35 | 2 | 3 | 17.49 |
| 635 | 36 | 2 | 3 | 15.34 |
| 636 | 37 | 2 | 3 | 14.00 |
| 637 | 38 | 2 | 3 | 13.05 |
| 638 | 39 | 2 | 3 | 12.27 |
| 639 | 40 | 2 | 3 | 11.39 |
| 640 | 41 | 2 | 3 | 10.82 |
| 641 | 42 | 2 | 3 | 10.50 |
| 642 | 43 | 2 | 3 | 10.31 |
| 643 | 8 | 2 | 3.5 | 10.82 |
| 644 | 9 | 2 | 3.5 | 11.37 |
| 645 | 10 | 2 | 3.5 | 12.18 |
| 646 | 11 | 2 | 3.5 | 12.82 |
| 647 | 12 | 2 | 3.5 | 13.56 |
| 648 | 13 | 2 | 3.5 | 14.53 |
| 649 | 14 | 2 | 3.5 | 15.90 |
| 650 | 15 | 2 | 3.5 | 18.12 |
| 651 | 16 | 2 | 3.5 | 22.63 |
| 652 | 17 | 2 | 3.5 | 36.44 |
| 653 | 18 | 2 | 3.5 | 61.22 |
| 654 | 19 | 2 | 3.5 | 70.64 |
| 655 | 20 | 2 | 3.5 | 63.24 |
| 656 | 21 | 2 | 3.5 | 55.59 |
| 657 | 22 | 2 | 3.5 | 48.13 |
| 658 | 23 | 2 | 3.5 | 39.72 |
| 659 | 24 | 2 | 3.5 | 29.92 |
| 660 | 25 | 2 | 3.5 | 26.14 |
| 661 | 26 | 2 | 3.5 | 29.92 |
| 662 | 27 | 2 | 3.5 | 39.72 |
| 663 | 28 | 2 | 3.5 | 48.13 |
| 664 | 29 | 2 | 3.5 | 55.59 |
| 665 | 30 | 2 | 3.5 | 63.25 |
| 666 | 31 | 2 | 3.5 | 70.64 |
| 667 | 32 | 2 | 3.5 | 61.22 |
| 668 | 33 | 2 | 3.5 | 36.44 |
| 669 | 34 | 2 | 3.5 | 22.63 |
| 670 | 35 | 2 | 3.5 | 18.12 |
| 671 | 36 | 2 | 3.5 | 15.90 |
| 672 | 37 | 2 | 3.5 | 14.53 |
| 673 | 38 | 2 | 3.5 | 13.56 |
| 674 | 39 | 2 | 3.5 | 12.82 |
| 675 | 40 | 2 | 3.5 | 12.18 |
| 676 | 41 | 2 | 3.5 | 11.38 |
| 677 | 42 | 2 | 3.5 | 10.82 |
| 678 | 9 | 2 | 4.0 | 12.07 |
| 679 | 10 | 2 | 4.0 | 12.61 |
| 680 | 11 | 2 | 4.0 | 13.21 |
| 681 | 12 | 2 | 4.0 | 13.94 |
| 682 | 13 | 2 | 4.0 | 14.89 |
| 683 | 14 | 2 | 4.0 | 16.26 |
| 684 | 15 | 2 | 4.0 | 18.48 |
| 685 | 16 | 2 | 4.0 | 22.97 |
| 686 | 17 | 2 | 4.0 | 36.90 |
| 687 | 18 | 2 | 4.0 | 61.79 |
| 688 | 19 | 2 | 4.0 | 70.48 |
| 689 | 20 | 2 | 4.0 | 61.63 |
| 690 | 21 | 2 | 4.0 | 51.76 |
| 691 | 22 | 2 | 4.0 | 41.09 |
| 692 | 23 | 2 | 4.0 | 29.32 |
| 693 | 24 | 2 | 4.0 | 23.57 |
| 694 | 25 | 2 | 4.0 | 22.43 |
| 695 | 26 | 2 | 4.0 | 23.57 |
| 696 | 27 | 2 | 4.0 | 29.32 |
| 697 | 28 | 2 | 4.0 | 41.09 |
| 698 | 29 | 2 | 4.0 | 51.76 |
| 699 | 30 | 2 | 4.0 | 61.63 |
| 700 | 31 | 2 | 4.0 | 70.48 |
| 701 | 32 | 2 | 4.0 | 61.79 |
| 702 | 33 | 2 | 4.0 | 36.90 |
| 703 | 34 | 2 | 4.0 | 22.97 |
| 704 | 35 | 2 | 4.0 | 18.48 |
| 705 | 36 | 2 | 4.0 | 16.26 |
| 706 | 37 | 2 | 4.0 | 14.89 |
| 707 | 38 | 2 | 4.0 | 13.94 |
| 708 | 39 | 2 | 4.0 | 13.21 |
| 709 | 40 | 2 | 4.0 | 12.62 |
| 710 | 41 | 2 | 4.0 | 12.07 |
| 711 | 10 | 2 | 4.5 | 12.90 |
| 712 | 11 | 2 | 4.5 | 13.47 |
| 713 | 12 | 2 | 4.5 | 14.18 |
| 714 | 13 | 2 | 4.5 | 15.10 |
| 715 | 14 | 2 | 4.5 | 16.42 |
| 716 | 15 | 2 | 4.5 | 18.54 |
| 717 | 16 | 2 | 4.5 | 22.89 |
| 718 | 17 | 2 | 4.5 | 36.45 |
| 719 | 18 | 2 | 4.5 | 60.21 |
| 720 | 19 | 2 | 4.5 | 66.79 |
| 721 | 20 | 2 | 4.5 | 55.74 |
| 722 | 21 | 2 | 4.5 | 43.23 |
| 723 | 22 | 2 | 4.5 | 29.84 |
| 724 | 23 | 2 | 4.5 | 23.14 |
| 725 | 24 | 2 | 4.5 | 21.01 |
| 726 | 25 | 2 | 4.5 | 20.44 |
| 727 | 26 | 2 | 4.5 | 21.01 |
| 728 | 27 | 2 | 4.5 | 23.14 |
| 729 | 28 | 2 | 4.5 | 29.84 |
| 730 | 29 | 2 | 4.5 | 43.23 |
| 731 | 30 | 2 | 4.5 | 55.74 |
| 732 | 31 | 2 | 4.5 | 66.79 |
| 733 | 32 | 2 | 4.5 | 60.21 |
| 734 | 33 | 2 | 4.5 | 36.45 |
| 735 | 34 | 2 | 4.5 | 22.89 |
| 736 | 35 | 2 | 4.5 | 18.54 |
| 737 | 36 | 2 | 4.5 | 16.42 |
| 738 | 37 | 2 | 4.5 | 15.11 |
| 739 | 38 | 2 | 4.5 | 14.18 |
| 740 | 39 | 2 | 4.5 | 13.48 |
| 741 | 40 | 2 | 4.5 | 12.90 |
| 742 | 11 | 2 | 5.0 | 13.63 |
| 743 | 12 | 2 | 5.0 | 14.29 |
| 744 | 13 | 2 | 5.0 | 15.16 |
| 745 | 14 | 2 | 5.0 | 16.38 |
| 746 | 15 | 2 | 5.0 | 18.32 |
| 747 | 16 | 2 | 5.0 | 22.36 |
| 748 | 17 | 2 | 5.0 | 34.98 |
| 749 | 18 | 2 | 5.0 | 56.31 |
| 750 | 19 | 2 | 5.0 | 59.76 |
| 751 | 20 | 2 | 5.0 | 46.06 |
| 752 | 21 | 2 | 5.0 | 30.94 |
| 753 | 22 | 2 | 5.0 | 23.38 |
| 754 | 23 | 2 | 5.0 | 20.71 |
| 755 | 24 | 2 | 5.0 | 19.57 |
| 756 | 25 | 2 | 5.0 | 19.22 |
| 757 | 26 | 2 | 5.0 | 19.57 |
| 758 | 27 | 2 | 5.0 | 20.71 |
| 759 | 28 | 2 | 5.0 | 23.38 |
| 760 | 29 | 2 | 5.0 | 30.94 |
| 761 | 30 | 2 | 5.0 | 46.06 |
| 762 | 31 | 2 | 5.0 | 59.76 |
| 763 | 32 | 2 | 5.0 | 56.31 |
| 764 | 33 | 2 | 5.0 | 34.98 |
| 765 | 34 | 2 | 5.0 | 22.36 |
| 766 | 35 | 2 | 5.0 | 18.32 |
| 767 | 36 | 2 | 5.0 | 16.39 |
| 768 | 37 | 2 | 5.0 | 15.16 |
| 769 | 38 | 2 | 5.0 | 14.30 |
| 770 | 39 | 2 | 5.0 | 13.63 |
| 771 | 12 | 2 | 5.5 | 14.28 |
| 772 | 13 | 2 | 5.5 | 15.05 |
| 773 | 14 | 2 | 5.5 | 16.12 |
| 774 | 15 | 2 | 5.5 | 17.80 |
| 775 | 16 | 2 | 5.5 | 21.32 |
| 776 | 17 | 2 | 5.5 | 32.38 |
| 777 | 18 | 2 | 5.5 | 49.78 |
| 778 | 19 | 2 | 5.5 | 48.79 |
| 779 | 20 | 2 | 5.5 | 32.54 |
| 780 | 21 | 2 | 5.5 | 24.02 |
| 781 | 22 | 2 | 5.5 | 20.88 |
| 782 | 23 | 2 | 5.5 | 19.38 |
| 783 | 24 | 2 | 5.5 | 18.62 |
| 784 | 25 | 2 | 5.5 | 18.39 |
| 785 | 26 | 2 | 5.5 | 18.62 |
| 786 | 27 | 2 | 5.5 | 19.38 |
| 787 | 28 | 2 | 5.5 | 20.88 |
| 788 | 29 | 2 | 5.5 | 24.02 |
| 789 | 30 | 2 | 5.5 | 32.54 |
| 790 | 31 | 2 | 5.5 | 48.79 |
| 791 | 32 | 2 | 5.5 | 49.78 |
| 792 | 33 | 2 | 5.5 | 32.38 |
| 793 | 34 | 2 | 5.5 | 21.32 |
| 794 | 35 | 2 | 5.5 | 17.80 |
| 795 | 36 | 2 | 5.5 | 16.13 |
| 796 | 37 | 2 | 5.5 | 15.06 |
| 797 | 38 | 2 | 5.5 | 14.28 |
| 798 | 13 | 2 | 6.0 | 14.78 |
| 799 | 14 | 2 | 6.0 | 15.64 |
| 800 | 15 | 2 | 6.0 | 16.96 |
| 801 | 16 | 2 | 6.0 | 19.73 |
| 802 | 17 | 2 | 6.0 | 28.48 |
| 803 | 18 | 2 | 6.0 | 40.23 |
| 804 | 19 | 2 | 6.0 | 34.04 |
| 805 | 20 | 2 | 6.0 | 25.04 |
| 806 | 21 | 2 | 6.0 | 21.38 |
| 807 | 22 | 2 | 6.0 | 19.54 |
| 808 | 23 | 2 | 6.0 | 18.51 |
| 809 | 24 | 2 | 6.0 | 17.96 |
| 810 | 25 | 2 | 6.0 | 17.80 |
| 811 | 26 | 2 | 6.0 | 17.97 |
| 812 | 27 | 2 | 6.0 | 18.51 |
| 813 | 28 | 2 | 6.0 | 19.54 |
| 814 | 29 | 2 | 6.0 | 21.38 |
| 815 | 30 | 2 | 6.0 | 25.04 |
| 816 | 31 | 2 | 6.0 | 34.04 |
| 817 | 32 | 2 | 6.0 | 40.23 |
| 818 | 33 | 2 | 6.0 | 28.48 |
| 819 | 34 | 2 | 6.0 | 19.73 |
| 820 | 35 | 2 | 6.0 | 16.96 |
| 821 | 36 | 2 | 6.0 | 15.64 |
| 822 | 37 | 2 | 6.0 | 14.79 |
| 823 | 14 | 2 | 6.5 | 14.91 |
| 824 | 15 | 2 | 6.5 | 15.76 |
| 825 | 16 | 2 | 6.5 | 17.50 |
| 826 | 17 | 2 | 6.5 | 23.06 |
| 827 | 18 | 2 | 6.5 | 27.82 |
| 828 | 19 | 2 | 6.5 | 25.95 |
| 829 | 20 | 2 | 6.5 | 22.18 |
| 830 | 21 | 2 | 6.5 | 19.96 |
| 831 | 22 | 2 | 6.5 | 18.67 |
| 832 | 23 | 2 | 6.5 | 17.90 |
| 833 | 24 | 2 | 6.5 | 17.48 |
| 834 | 25 | 2 | 6.5 | 17.35 |
| 835 | 26 | 2 | 6.5 | 17.48 |
| 836 | 27 | 2 | 6.5 | 17.90 |
| 837 | 28 | 2 | 6.5 | 18.67 |
| 838 | 29 | 2 | 6.5 | 19.96 |
| 839 | 30 | 2 | 6.5 | 22.18 |
| 840 | 31 | 2 | 6.5 | 25.95 |
| 841 | 32 | 2 | 6.5 | 27.82 |
| 842 | 33 | 2 | 6.5 | 23.06 |
| 843 | 34 | 2 | 6.5 | 17.50 |
| 844 | 35 | 2 | 6.5 | 15.76 |
| 845 | 36 | 2 | 6.5 | 14.92 |
| 846 | 15 | 2 | 7.0 | 14.13 |
| 847 | 16 | 2 | 7.0 | 14.49 |
| 848 | 17 | 2 | 7.0 | 16.23 |
| 849 | 18 | 2 | 7.0 | 21.06 |
| 850 | 19 | 2 | 7.0 | 22.82 |
| 851 | 20 | 2 | 7.0 | 20.62 |
| 852 | 21 | 2 | 7.0 | 19.05 |
| 853 | 22 | 2 | 7.0 | 18.06 |
| 854 | 23 | 2 | 7.0 | 17.45 |
| 855 | 24 | 2 | 7.0 | 17.11 |
| 856 | 25 | 2 | 7.0 | 17.01 |
| 857 | 26 | 2 | 7.0 | 17.11 |
| 858 | 27 | 2 | 7.0 | 17.45 |
| 859 | 28 | 2 | 7.0 | 18.06 |
| 860 | 29 | 2 | 7.0 | 19.05 |
| 861 | 30 | 2 | 7.0 | 20.62 |
| 862 | 31 | 2 | 7.0 | 22.82 |
| 863 | 32 | 2 | 7.0 | 21.06 |
| 864 | 33 | 2 | 7.0 | 16.23 |
| 865 | 34 | 2 | 7.0 | 14.50 |
| 866 | 35 | 2 | 7.0 | 14.14 |
| 867 | 16 | 2 | 7.5 | 10.76 |
| 868 | 17 | 2 | 7.5 | 12.50 |
| 869 | 18 | 2 | 7.5 | 18.39 |
| 870 | 19 | 2 | 7.5 | 21.10 |
| 871 | 20 | 2 | 7.5 | 19.62 |
| 872 | 21 | 2 | 7.5 | 18.40 |
| 873 | 22 | 2 | 7.5 | 17.61 |
| 874 | 23 | 2 | 7.5 | 17.10 |
| 875 | 24 | 2 | 7.5 | 16.82 |
| 876 | 25 | 2 | 7.5 | 16.74 |
| 877 | 26 | 2 | 7.5 | 16.83 |
| 878 | 27 | 2 | 7.5 | 17.10 |
| 879 | 28 | 2 | 7.5 | 17.61 |
| 880 | 29 | 2 | 7.5 | 18.40 |
| 881 | 30 | 2 | 7.5 | 19.62 |
| 882 | 31 | 2 | 7.5 | 21.10 |
| 883 | 32 | 2 | 7.5 | 18.39 |
| 884 | 33 | 2 | 7.5 | 12.50 |
| 885 | 34 | 2 | 7.5 | 10.77 |
| 886 | 17 | 2 | 8.0 | 11.02 |
| 887 | 18 | 2 | 8.0 | 16.92 |
| 888 | 19 | 2 | 8.0 | 19.98 |
| 889 | 20 | 2 | 8.0 | 18.91 |
| 890 | 21 | 2 | 8.0 | 17.93 |
| 891 | 22 | 2 | 8.0 | 17.26 |
| 892 | 23 | 2 | 8.0 | 16.84 |
| 893 | 24 | 2 | 8.0 | 16.60 |
| 894 | 25 | 2 | 8.0 | 16.52 |
| 895 | 26 | 2 | 8.0 | 16.60 |
| 896 | 27 | 2 | 8.0 | 16.84 |
| 897 | 28 | 2 | 8.0 | 17.27 |
| 898 | 29 | 2 | 8.0 | 17.93 |
| 899 | 30 | 2 | 8.0 | 18.91 |
| 900 | 31 | 2 | 8.0 | 19.98 |
| 901 | 32 | 2 | 8.0 | 16.92 |
| 902 | 33 | 2 | 8.0 | 11.03 |
| 903 | 18 | 2 | 8.5 | 15.96 |
| 904 | 19 | 2 | 8.5 | 19.19 |
| 905 | 20 | 2 | 8.5 | 18.39 |
| 906 | 21 | 2 | 8.5 | 17.56 |
| 907 | 22 | 2 | 8.5 | 17.00 |
| 908 | 23 | 2 | 8.5 | 16.63 |
| 909 | 24 | 2 | 8.5 | 16.42 |
| 910 | 25 | 2 | 8.5 | 16.35 |
| 911 | 26 | 2 | 8.5 | 16.42 |
| 912 | 27 | 2 | 8.5 | 16.63 |
| 913 | 28 | 2 | 8.5 | 17.00 |
| 914 | 29 | 2 | 8.5 | 17.57 |
| 915 | 30 | 2 | 8.5 | 18.39 |
| 916 | 31 | 2 | 8.5 | 19.20 |
| 917 | 32 | 2 | 8.5 | 15.96 |
| 918 | 19 | 2 | 9.0000 | 18.60 |
| 919 | 20 | 2 | 9.0000 | 17.99 |
| 920 | 21 | 2 | 9.0000 | 17.28 |
| 921 | 22 | 2 | 9.0000 | 16.78 |
| 922 | 23 | 2 | 9.0000 | 16.45 |
| 923 | 24 | 2 | 9.0000 | 16.27 |
| 924 | 25 | 2 | 9.0000 | 16.21 |
| 925 | 26 | 2 | 9.0000 | 16.27 |
| 926 | 27 | 2 | 9.0000 | 16.46 |
| 927 | 28 | 2 | 9.0000 | 16.79 |
| 928 | 29 | 2 | 9.0000 | 17.28 |
| 929 | 30 | 2 | 9.0000 | 17.99 |
| 930 | 31 | 2 | 9.0000 | 18.61 |
| 931 | 20 | 2 | 9.5000 | 17.67 |
| 932 | 21 | 2 | 9.5000 | 17.05 |
| 933 | 22 | 2 | 9.5000 | 16.61 |
| 934 | 23 | 2 | 9.5000 | 16.32 |
| 935 | 24 | 2 | 9.5000 | 16.15 |
| 936 | 25 | 2 | 9.5000 | 16.09 |
| 937 | 26 | 2 | 9.5000 | 16.15 |
| 938 | 27 | 2 | 9.5000 | 16.32 |
| 939 | 28 | 2 | 9.5000 | 16.61 |
| 940 | 29 | 2 | 9.5000 | 17.05 |
| 941 | 30 | 2 | 9.5000 | 17.67 |
| 942 | 21 | 2 | 10.0000 | 16.87 |
| 943 | 22 | 2 | 10.0000 | 16.47 |
| 944 | 23 | 2 | 10.0000 | 16.20 |
| 945 | 24 | 2 | 10.0000 | 16.05 |
| 946 | 25 | 2 | 10.0000 | 16.00 |
| 947 | 26 | 2 | 10.0000 | 16.05 |
| 948 | 27 | 2 | 10.0000 | 16.20 |
| 949 | 28 | 2 | 10.0000 | 16.47 |
| 950 | 29 | 2 | 10.0000 | 16.87 |
| 951 | 2.5 | 3 | 0.3333 | 9.99 |
| 952 | 3.5 | 3 | 0.3333 | 10.00 |
| 953 | 4.5 | 3 | 0.3333 | 10.01 |
| 954 | 5.5 | 3 | 0.3333 | 10.01 |
| 955 | 6.5 | 3 | 0.3333 | 10.01 |
| 956 | 7.5 | 3 | 0.3333 | 10.02 |
| 957 | 8.5 | 3 | 0.3333 | 10.02 |
| 958 | 9.5 | 3 | 0.3333 | 10.03 |
| 959 | 10.5 | 3 | 0.3333 | 10.04 |
| 960 | 11.5 | 3 | 0.3333 | 10.07 |
| 961 | 12.5 | 3 | 0.3333 | 10.12 |
| 962 | 13.5 | 3 | 0.3333 | 10.23 |
| 963 | 14.5 | 3 | 0.3333 | 10.55 |
| 964 | 15.5 | 3 | 0.3333 | 11.72 |
| 965 | 16.5 | 3 | 0.3333 | 15.62 |
| 966 | 17.5 | 3 | 0.3333 | 21.36 |
| 967 | 18.5 | 3 | 0.3333 | 24.38 |
| 968 | 19.5 | 3 | 0.3333 | 25.96 |
| 969 | 20.5 | 3 | 0.3333 | 27.66 |
| 970 | 21.5 | 3 | 0.3333 | 30.45 |
| 971 | 22.5 | 3 | 0.3333 | 31.59 |
| 972 | 23.5 | 3 | 0.3333 | 32.13 |
| 973 | 24.5 | 3 | 0.3333 | 32.34 |
| 974 | 25.5 | 3 | 0.3333 | 32.35 |
| 975 | 26.5 | 3 | 0.3333 | 32.13 |
| 976 | 27.5 | 3 | 0.3333 | 31.59 |
| 977 | 28.5 | 3 | 0.3333 | 30.45 |
| 978 | 29.5 | 3 | 0.3333 | 27.66 |
| 979 | 30.5 | 3 | 0.3333 | 25.96 |
| 980 | 31.5 | 3 | 0.3333 | 24.38 |
| 981 | 32.5 | 3 | 0.3333 | 21.36 |
| 982 | 33.5 | 3 | 0.3333 | 15.62 |
| 983 | 34.5 | 3 | 0.3333 | 11.72 |
| 984 | 35.5 | 3 | 0.3333 | 10.55 |
| 985 | 36.5 | 3 | 0.3333 | 10.23 |
| 986 | 37.5 | 3 | 0.3333 | 10.12 |
| 987 | 38.5 | 3 | 0.3333 | 10.07 |
| 988 | 39.5 | 3 | 0.3333 | 10.04 |
| 989 | 40.5 | 3 | 0.3333 | 10.03 |
| 990 | 41.5 | 3 | 0.3333 | 10.02 |
| 991 | 42.5 | 3 | 0.3333 | 10.02 |
| 992 | 43.5 | 3 | 0.3333 | 10.01 |
| 993 | 44.5 | 3 | 0.3333 | 10.01 |
| 994 | 45.5 | 3 | 0.3333 | 10.01 |
| 995 | 46.5 | 3 | 0.3333 | 10.01 |
| 996 | 47.5 | 3 | 0.3333 | 10.00 |
| 997 | 3.5 | 3 | 0.6667 | 10.04 |
| 998 | 4.5 | 3 | 0.6667 | 10.06 |
| 999 | 5.5 | 3 | 0.6667 | 10.07 |
| 1000 | 6.5 | 3 | 0.6667 | 10.08 |
| 1001 | 7.5 | 3 | 0.6667 | 10.09 |
| 1002 | 8.5 | 3 | 0.6667 | 10.10 |
| 1003 | 9.5 | 3 | 0.6667 | 10.13 |
| 1004 | 10.5 | 3 | 0.6667 | 10.18 |
| 1005 | 11.5 | 3 | 0.6667 | 10.28 |
| 1006 | 12.5 | 3 | 0.6667 | 10.48 |
| 1007 | 13.5 | 3 | 0.6667 | 10.96 |
| 1008 | 14.5 | 3 | 0.6667 | 12.19 |
| 1009 | 15.5 | 3 | 0.6667 | 14.47 |
| 1010 | 16.5 | 3 | 0.6667 | 20.59 |
| 1011 | 17.5 | 3 | 0.6667 | 30.54 |
| 1012 | 18.5 | 3 | 0.6667 | 36.43 |
| 1013 | 19.5 | 3 | 0.6667 | 38.84 |
| 1014 | 20.5 | 3 | 0.6667 | 38.30 |
| 1015 | 21.5 | 3 | 0.6667 | 40.46 |
| 1016 | 22.5 | 3 | 0.6667 | 44.81 |
| 1017 | 23.5 | 3 | 0.6667 | 46.91 |
| 1018 | 24.5 | 3 | 0.6667 | 47.72 |
| 1019 | 25.5 | 3 | 0.6667 | 47.72 |
| 1020 | 26.5 | 3 | 0.6667 | 46.91 |
| 1021 | 27.5 | 3 | 0.6667 | 44.81 |
| 1022 | 28.5 | 3 | 0.6667 | 40.46 |
| 1023 | 29.5 | 3 | 0.6667 | 38.30 |
| 1024 | 30.5 | 3 | 0.6667 | 38.84 |
| 1025 | 31.5 | 3 | 0.6667 | 36.43 |
| 1026 | 32.5 | 3 | 0.6667 | 30.54 |
| 1027 | 33.5 | 3 | 0.6667 | 20.59 |
| 1028 | 34.5 | 3 | 0.6667 | 14.47 |
| 1029 | 35.5 | 3 | 0.6667 | 12.19 |
| 1030 | 36.5 | 3 | 0.6667 | 10.96 |
| 1031 | 37.5 | 3 | 0.6667 | 10.48 |
| 1032 | 38.5 | 3 | 0.6667 | 10.28 |
| 1033 | 39.5 | 3 | 0.6667 | 10.18 |
| 1034 | 40.5 | 3 | 0.6667 | 10.13 |
| 1035 | 41.5 | 3 | 0.6667 | 10.10 |
| 1036 | 42.5 | 3 | 0.6667 | 10.09 |
| 1037 | 43.5 | 3 | 0.6667 | 10.08 |
| 1038 | 44.5 | 3 | 0.6667 | 10.07 |
| 1039 | 45.5 | 3 | 0.6667 | 10.06 |
| 1040 | 46.5 | 3 | 0.6667 | 10.04 |
| 1041 | 4.5 | 3 | 1.0000 | 10.06 |
| 1042 | 5.5 | 3 | 1.0000 | 10.09 |
| 1043 | 6.5 | 3 | 1.0000 | 10.11 |
| 1044 | 7.5 | 3 | 1.0000 | 10.14 |
| 1045 | 8.5 | 3 | 1.0000 | 10.18 |
| 1046 | 9.5 | 3 | 1.0000 | 10.25 |
| 1047 | 10.5 | 3 | 1.0000 | 10.38 |
| 1048 | 11.5 | 3 | 1.0000 | 10.64 |
| 1049 | 12.5 | 3 | 1.0000 | 11.19 |
| 1050 | 13.5 | 3 | 1.0000 | 12.34 |
| 1051 | 14.5 | 3 | 1.0000 | 13.87 |
| 1052 | 15.5 | 3 | 1.0000 | 16.66 |
| 1053 | 16.5 | 3 | 1.0000 | 24.41 |
| 1054 | 17.5 | 3 | 1.0000 | 37.79 |
| 1055 | 18.5 | 3 | 1.0000 | 46.06 |
| 1056 | 19.5 | 3 | 1.0000 | 49.06 |
| 1057 | 20.5 | 3 | 1.0000 | 46.95 |
| 1058 | 21.5 | 3 | 1.0000 | 45.99 |
| 1059 | 22.5 | 3 | 1.0000 | 48.26 |
| 1060 | 23.5 | 3 | 1.0000 | 53.30 |
| 1061 | 24.5 | 3 | 1.0000 | 55.48 |
| 1062 | 25.5 | 3 | 1.0000 | 55.48 |
| 1063 | 26.5 | 3 | 1.0000 | 53.31 |
| 1064 | 27.5 | 3 | 1.0000 | 48.26 |
| 1065 | 28.5 | 3 | 1.0000 | 45.99 |
| 1066 | 29.5 | 3 | 1.0000 | 46.95 |
| 1067 | 30.5 | 3 | 1.0000 | 49.06 |
| 1068 | 31.5 | 3 | 1.0000 | 46.06 |
| 1069 | 32.5 | 3 | 1.0000 | 37.79 |
| 1070 | 33.5 | 3 | 1.0000 | 24.41 |
| 1071 | 34.5 | 3 | 1.0000 | 16.66 |
| 1072 | 35.5 | 3 | 1.0000 | 13.87 |
| 1073 | 36.5 | 3 | 1.0000 | 12.34 |
| 1074 | 37.5 | 3 | 1.0000 | 11.19 |
| 1075 | 38.5 | 3 | 1.0000 | 10.64 |
| 1076 | 39.5 | 3 | 1.0000 | 10.38 |
| 1077 | 40.5 | 3 | 1.0000 | 10.25 |
| 1078 | 41.5 | 3 | 1.0000 | 10.18 |
| 1079 | 42.5 | 3 | 1.0000 | 10.14 |
| 1080 | 43.5 | 3 | 1.0000 | 10.11 |
| 1081 | 44.5 | 3 | 1.0000 | 10.09 |
| 1082 | 45.5 | 3 | 1.0000 | 10.07 |
| 1083 | 5.5 | 3 | 1.3333 | 10.10 |
| 1084 | 6.5 | 3 | 1.3333 | 10.15 |
| 1085 | 7.5 | 3 | 1.3333 | 10.20 |
| 1086 | 8.5 | 3 | 1.3333 | 10.29 |
| 1087 | 9.5 | 3 | 1.3333 | 10.45 |
| 1088 | 10.5 | 3 | 1.3333 | 10.74 |
| 1089 | 11.5 | 3 | 1.3333 | 11.32 |
| 1090 | 12.5 | 3 | 1.3333 | 12.37 |
| 1091 | 13.5 | 3 | 1.3333 | 13.51 |
| 1092 | 14.5 | 3 | 1.3333 | 15.18 |
| 1093 | 15.5 | 3 | 1.3333 | 18.38 |
| 1094 | 16.5 | 3 | 1.3333 | 27.44 |
| 1095 | 17.5 | 3 | 1.3333 | 43.50 |
| 1096 | 18.5 | 3 | 1.3333 | 53.62 |
| 1097 | 19.5 | 3 | 1.3333 | 56.96 |
| 1098 | 20.5 | 3 | 1.3333 | 53.54 |
| 1099 | 21.5 | 3 | 1.3333 | 50.73 |
| 1100 | 22.5 | 3 | 1.3333 | 49.20 |
| 1101 | 23.5 | 3 | 1.3333 | 50.66 |
| 1102 | 24.5 | 3 | 1.3333 | 54.92 |
| 1103 | 25.5 | 3 | 1.3333 | 54.92 |
| 1104 | 26.5 | 3 | 1.3333 | 50.66 |
| 1105 | 27.5 | 3 | 1.3333 | 49.20 |
| 1106 | 28.5 | 3 | 1.3333 | 50.73 |
| 1107 | 29.5 | 3 | 1.3333 | 53.54 |
| 1108 | 30.5 | 3 | 1.3333 | 56.96 |
| 1109 | 31.5 | 3 | 1.3333 | 53.62 |
| 1110 | 32.5 | 3 | 1.3333 | 43.50 |
| 1111 | 33.5 | 3 | 1.3333 | 27.44 |
| 1112 | 34.5 | 3 | 1.3333 | 18.38 |
| 1113 | 35.5 | 3 | 1.3333 | 15.18 |
| 1114 | 36.5 | 3 | 1.3333 | 13.51 |
| 1115 | 37.5 | 3 | 1.3333 | 12.37 |
| 1116 | 38.5 | 3 | 1.3333 | 11.32 |
| 1117 | 39.5 | 3 | 1.3333 | 10.74 |
| 1118 | 40.5 | 3 | 1.3333 | 10.45 |
| 1119 | 41.5 | 3 | 1.3333 | 10.30 |
| 1120 | 42.5 | 3 | 1.3333 | 10.21 |
| 1121 | 43.5 | 3 | 1.3333 | 10.15 |
| 1122 | 44.5 | 3 | 1.3333 | 10.10 |
| 1123 | 6.5 | 3 | 1.6667 | 10.20 |
| 1124 | 7.5 | 3 | 1.6667 | 10.31 |
| 1125 | 8.5 | 3 | 1.6667 | 10.49 |
| 1126 | 9.5 | 3 | 1.6667 | 10.80 |
| 1127 | 10.5 | 3 | 1.6667 | 11.37 |
| 1128 | 11.5 | 3 | 1.6667 | 12.32 |
| 1129 | 12.5 | 3 | 1.6667 | 13.22 |
| 1130 | 13.5 | 3 | 1.6667 | 14.39 |
| 1131 | 14.5 | 3 | 1.6667 | 16.21 |
| 1132 | 15.5 | 3 | 1.6667 | 19.72 |
| 1133 | 16.5 | 3 | 1.6667 | 29.73 |
| 1134 | 17.5 | 3 | 1.6667 | 47.76 |
| 1135 | 18.5 | 3 | 1.6667 | 59.17 |
| 1136 | 19.5 | 3 | 1.6667 | 62.50 |
| 1137 | 20.5 | 3 | 1.6667 | 57.69 |
| 1138 | 21.5 | 3 | 1.6667 | 53.13 |
| 1139 | 22.5 | 3 | 1.6667 | 49.18 |
| 1140 | 23.5 | 3 | 1.6667 | 45.80 |
| 1141 | 24.5 | 3 | 1.6667 | 44.28 |
| 1142 | 25.5 | 3 | 1.6667 | 44.28 |
| 1143 | 26.5 | 3 | 1.6667 | 45.80 |
| 1144 | 27.5 | 3 | 1.6667 | 49.18 |
| 1145 | 28.5 | 3 | 1.6667 | 53.13 |
| 1146 | 29.5 | 3 | 1.6667 | 57.69 |
| 1147 | 30.5 | 3 | 1.6667 | 62.50 |
| 1148 | 31.5 | 3 | 1.6667 | 59.17 |
| 1149 | 32.5 | 3 | 1.6667 | 47.76 |
| 1150 | 33.5 | 3 | 1.6667 | 29.73 |
| 1151 | 34.5 | 3 | 1.6667 | 19.72 |
| 1152 | 35.5 | 3 | 1.6667 | 16.21 |
| 1153 | 36.5 | 3 | 1.6667 | 14.39 |
| 1154 | 37.5 | 3 | 1.6667 | 13.22 |
| 1155 | 38.5 | 3 | 1.6667 | 12.32 |
| 1156 | 39.5 | 3 | 1.6667 | 11.37 |
| 1157 | 40.5 | 3 | 1.6667 | 10.80 |
| 1158 | 41.5 | 3 | 1.6667 | 10.49 |
| 1159 | 42.5 | 3 | 1.6667 | 10.31 |
| 1160 | 43.5 | 3 | 1.6667 | 10.20 |
| 1161 | 7.5 | 3 | 2.0000 | 10.49 |
| 1162 | 8.5 | 3 | 2.0000 | 10.82 |
| 1163 | 9.5 | 3 | 2.0000 | 11.38 |
| 1164 | 10.5 | 3 | 2.0000 | 12.24 |
| 1165 | 11.5 | 3 | 2.0000 | 12.97 |
| 1166 | 12.5 | 3 | 2.0000 | 13.85 |
| 1167 | 13.5 | 3 | 2.0000 | 15.07 |
| 1168 | 14.5 | 3 | 2.0000 | 16.97 |
| 1169 | 15.5 | 3 | 2.0000 | 20.68 |
| 1170 | 16.5 | 3 | 2.0000 | 31.31 |
| 1171 | 17.5 | 3 | 2.0000 | 50.57 |
| 1172 | 18.5 | 3 | 2.0000 | 62.64 |
| 1173 | 19.5 | 3 | 2.0000 | 65.51 |
| 1174 | 20.5 | 3 | 2.0000 | 59.24 |
| 1175 | 21.5 | 3 | 2.0000 | 52.93 |
| 1176 | 22.5 | 3 | 2.0000 | 46.67 |
| 1177 | 23.5 | 3 | 2.0000 | 39.45 |
| 1178 | 24.5 | 3 | 2.0000 | 31.15 |
| 1179 | 25.5 | 3 | 2.0000 | 31.15 |
| 1180 | 26.5 | 3 | 2.0000 | 39.45 |
| 1181 | 27.5 | 3 | 2.0000 | 46.67 |
| 1182 | 28.5 | 3 | 2.0000 | 52.93 |
| 1183 | 29.5 | 3 | 2.0000 | 59.24 |
| 1184 | 30.5 | 3 | 2.0000 | 65.51 |
| 1185 | 31.5 | 3 | 2.0000 | 62.64 |
| 1186 | 32.5 | 3 | 2.0000 | 50.57 |
| 1187 | 33.5 | 3 | 2.0000 | 31.31 |
| 1188 | 34.5 | 3 | 2.0000 | 20.68 |
| 1189 | 35.5 | 3 | 2.0000 | 16.97 |
| 1190 | 36.5 | 3 | 2.0000 | 15.07 |
| 1191 | 37.5 | 3 | 2.0000 | 13.85 |
| 1192 | 38.5 | 3 | 2.0000 | 12.97 |
| 1193 | 39.5 | 3 | 2.0000 | 12.24 |
| 1194 | 40.5 | 3 | 2.0000 | 11.39 |
| 1195 | 41.5 | 3 | 2.0000 | 10.82 |
| 1196 | 42.5 | 3 | 2.0000 | 10.5 |
| 1197 | 8.5 | 3 | 2.3333 | 11.36 |
| 1198 | 9.5 | 3 | 2.3333 | 12.14 |
| 1199 | 10.5 | 3 | 2.3333 | 12.75 |
| 1200 | 11.5 | 3 | 2.3333 | 13.43 |
| 1201 | 12.5 | 3 | 2.3333 | 14.32 |
| 1202 | 13.5 | 3 | 2.3333 | 15.55 |
| 1203 | 14.5 | 3 | 2.3333 | 17.48 |
| 1204 | 15.5 | 3 | 2.3333 | 21.26 |
| 1205 | 16.5 | 3 | 2.3333 | 32.17 |
| 1206 | 17.5 | 3 | 2.3333 | 51.88 |
| 1207 | 18.5 | 3 | 2.3333 | 63.85 |
| 1208 | 19.5 | 3 | 2.3333 | 65.77 |
| 1209 | 20.5 | 3 | 2.3333 | 57.79 |
| 1210 | 21.5 | 3 | 2.3333 | 49.34 |
| 1211 | 22.5 | 3 | 2.3333 | 40.00 |
| 1212 | 23.5 | 3 | 2.3333 | 29.23 |
| 1213 | 24.5 | 3 | 2.3333 | 24.20 |
| 1214 | 25.5 | 3 | 2.3333 | 24.20 |
| 1215 | 26.5 | 3 | 2.3333 | 29.23 |
| 1216 | 27.5 | 3 | 2.3333 | 40.00s |
| 1217 | 28.5 | 3 | 2.3333 | 49.34 |
| 1218 | 29.5 | 3 | 2.3333 | 57.79 |
| 1219 | 30.5 | 3 | 2.3333 | 65.77 |
| 1220 | 31.5 | 3 | 2.3333 | 63.85 |
| 1221 | 32.5 | 3 | 2.3333 | 51.88 |
| 1222 | 33.5 | 3 | 2.3333 | 32.17 |
| 1223 | 34.5 | 3 | 2.3333 | 21.26 |
| 1224 | 35.5 | 3 | 2.3333 | 17.48 |
| 1225 | 36.5 | 3 | 2.3333 | 15.55 |
| 1226 | 37.5 | 3 | 2.3333 | 14.32 |
| 1227 | 38.5 | 3 | 2.3333 | 13.43 |
| 1228 | 39.5 | 3 | 2.3333 | 12.75 |
| 1229 | 40.5 | 3 | 2.3333 | 12.14 |
| 1230 | 41.5 | 3 | 2.3333 | 11.36 |
| 1231 | 9.5 | 3 | 2.6667 | 12.53 |
| 1232 | 10.5 | 3 | 2.6667 | 13.09 |
| 1233 | 11.5 | 3 | 2.6667 | 13.76 |
| 1234 | 12.5 | 3 | 2.6667 | 14.63 |
| 1235 | 13.5 | 3 | 2.6667 | 15.84 |
| 1236 | 14.5 | 3 | 2.6667 | 17.76 |
| 1237 | 15.5 | 3 | 2.6667 | 21.47 |
| 1238 | 16.5 | 3 | 2.6667 | 32.33 |
| 1239 | 17.5 | 3 | 2.6667 | 51.89 |
| 1240 | 18.5 | 3 | 2.6667 | 63.16 |
| 1241 | 19.5 | 3 | 2.6667 | 63.55 |
| 1242 | 20.5 | 3 | 2.6667 | 53.36 |
| 1243 | 21.5 | 3 | 2.6667 | 41.86 |
| 1244 | 22.5 | 3 | 2.6667 | 29.36 |
| 1245 | 23.5 | 3 | 2.6667 | 23.12 |
| 1246 | 24.5 | 3 | 2.6667 | 21.37 |
| 1247 | 25.5 | 3 | 2.6667 | 21.37 |
| 1248 | 26.5 | 3 | 2.6667 | 23.12 |
| 1249 | 27.5 | 3 | 2.6667 | 29.36 |
| 1250 | 28.5 | 3 | 2.6667 | 41.86 |
| 1251 | 29.5 | 3 | 2.6667 | 53.36 |
| 1252 | 30.5 | 3 | 2.6667 | 63.55 |
| 1253 | 31.5 | 3 | 2.6667 | 63.16 |
| 1254 | 32.5 | 3 | 2.6667 | 51.89 |
| 1255 | 33.5 | 3 | 2.6667 | 32.33 |
| 1256 | 34.5 | 3 | 2.6667 | 21.47 |
| 1257 | 35.5 | 3 | 2.6667 | 17.76 |
| 1258 | 36.5 | 3 | 2.6667 | 15.84 |
| 1259 | 37.5 | 3 | 2.6667 | 14.63 |
| 1260 | 38.5 | 3 | 2.6667 | 13.76 |
| 1261 | 39.5 | 3 | 2.6667 | 13.09 |
| 1262 | 40.5 | 3 | 2.6667 | 12.54 |
| 1263 | 10.5 | 3 | 3.0000 | 13.32 |
| 1264 | 11.5 | 3 | 3.0000 | 13.97 |
| 1265 | 12.5 | 3 | 3.0000 | 14.80 |
| 1266 | 13.5 | 3 | 3.0000 | 15.95 |
| 1267 | 14.5 | 3 | 3.0000 | 17.76 |
| 1268 | 15.5 | 3 | 3.0000 | 21.30 |
| 1269 | 16.5 | 3 | 3.0000 | 31.74 |
| 1270 | 17.5 | 3 | 3.0000 | 50.22 |
| 1271 | 18.5 | 3 | 3.0000 | 59.47 |
| 1272 | 19.5 | 3 | 3.0000 | 57.22 |
| 1273 | 20.5 | 3 | 3.0000 | 44.37 |
| 1274 | 21.5 | 3 | 3.0000 | 30.21 |
| 1275 | 22.5 | 3 | 3.0000 | 23.11 |
| 1276 | 23.5 | 3 | 3.0000 | 20.69 |
| 1277 | 24.5 | 3 | 3.0000 | 19.79 |
| 1278 | 25.5 | 3 | 3.0000 | 19.79 |
| 1279 | 26.5 | 3 | 3.0000 | 20.69 |
| 1280 | 27.5 | 3 | 3.0000 | 23.11 |
| 1281 | 28.5 | 3 | 3.0000 | 30.21 |
| 1282 | 29.5 | 3 | 3.0000 | 44.37 |
| 1283 | 30.5 | 3 | 3.0000 | 57.22 |
| 1284 | 31.5 | 3 | 3.0000 | 59.47 |
| 1285 | 32.5 | 3 | 3.0000 | 50.22 |
| 1286 | 33.5 | 3 | 3.0000 | 31.74 |
| 1287 | 34.5 | 3 | 3.0000 | 21.30 |
| 1288 | 35.5 | 3 | 3.0000 | 17.76 |
| 1289 | 36.5 | 3 | 3.0000 | 15.96 |
| 1290 | 37.5 | 3 | 3.0000 | 14.80 |
| 1291 | 38.5 | 3 | 3.0000 | 13.97 |
| 1292 | 39.5 | 3 | 3.0000 | 13.32 |
| 1293 | 11.5 | 3 | 3.3333 | 14.05 |
| 1294 | 12.5 | 3 | 3.3333 | 14.82 |
| 1295 | 13.5 | 3 | 3.3333 | 15.88 |
| 1296 | 14.5 | 3 | 3.3333 | 17.51 |
| 1297 | 15.5 | 3 | 3.3333 | 20.74 |
| 1298 | 16.5 | 3 | 3.3333 | 30.35 |
| 1299 | 17.5 | 3 | 3.3333 | 46.72 |
| 1300 | 18.5 | 3 | 3.3333 | 52.93 |
| 1301 | 19.5 | 3 | 3.3333 | 47.05 |
| 1302 | 20.5 | 3 | 3.3333 | 31.58 |
| 1303 | 21.5 | 3 | 3.3333 | 23.58 |
| 1304 | 22.5 | 3 | 3.3333 | 20.68 |
| 1305 | 23.5 | 3 | 3.3333 | 19.35 |
| 1306 | 24.5 | 3 | 3.3333 | 18.78 |
| 1307 | 25.5 | 3 | 3.3333 | 18.78 |
| 1308 | 26.5 | 3 | 3.3333 | 19.35 |
| 1309 | 27.5 | 3 | 3.3333 | 20.68 |
| 1310 | 28.5 | 3 | 3.3333 | 23.58 |
| 1311 | 29.5 | 3 | 3.3333 | 31.58 |
| 1312 | 30.5 | 3 | 3.3333 | 47.05 |
| 1313 | 31.5 | 3 | 3.3333 | 52.93 |
| 1314 | 32.5 | 3 | 3.3333 | 46.72 |
| 1315 | 33.5 | 3 | 3.3333 | 30.35 |
| 1316 | 34.5 | 3 | 3.3333 | 20.74 |
| 1317 | 35.5 | 3 | 3.3333 | 17.51 |
| 1318 | 36.5 | 3 | 3.3333 | 15.88 |
| 1319 | 37.5 | 3 | 3.3333 | 14.82 |
| 1320 | 38.5 | 3 | 3.3333 | 14.06 |
| 1321 | 12.5 | 3 | 3.6667 | 14.70 |
| 1322 | 13.5 | 3 | 3.6667 | 15.61 |
| 1323 | 14.5 | 3 | 3.6667 | 17.00 |
| 1324 | 15.5 | 3 | 3.6667 | 19.76 |
| 1325 | 16.5 | 3 | 3.6667 | 28.03 |
| 1326 | 17.5 | 3 | 3.6667 | 41.16 |
| 1327 | 18.5 | 3 | 3.6667 | 43.00 |
| 1328 | 19.5 | 3 | 3.6667 | 33.04 |
| 1329 | 20.5 | 3 | 3.6667 | 24.43 |
| 1330 | 21.5 | 3 | 3.6667 | 21.04 |
| 1331 | 22.5 | 3 | 3.6667 | 19.37 |
| 1332 | 23.5 | 3 | 3.6667 | 18.47 |
| 1333 | 24.5 | 3 | 3.6667 | 18.07 |
| 1334 | 25.5 | 3 | 3.6667 | 18.07 |
| 1335 | 26.5 | 3 | 3.6667 | 18.47 |
| 1336 | 27.5 | 3 | 3.6667 | 19.37 |
| 1337 | 28.5 | 3 | 3.6667 | 21.04 |
| 1338 | 29.5 | 3 | 3.6667 | 24.43 |
| 1339 | 30.5 | 3 | 3.6667 | 33.04 |
| 1340 | 31.5 | 3 | 3.6667 | 43.00 |
| 1341 | 32.5 | 3 | 3.6667 | 41.16 |
| 1342 | 33.5 | 3 | 3.6667 | 28.03 |
| 1343 | 34.5 | 3 | 3.6667 | 19.76 |
| 1344 | 35.5 | 3 | 3.6667 | 17,00 |
| 1345 | 36.5 | 3 | 3.6667 | 15.61 |
| 1346 | 37.5 | 3 | 3.6667 | 14.70 |
| 1347 | 13.5 | 3 | 4.0000 | 15.13 |
| 1348 | 14.5 | 3 | 4.0000 | 16.19 |
| 1349 | 15.5 | 3 | 4.0000 | 18.29 |
| 1350 | 16.5 | 3 | 4.0000 | 24.67 |
| 1351 | 17.5 | 3 | 4.0000 | 33.22 |
| 1352 | 18.5 | 3 | 4.0000 | 29.86 |
| 1353 | 19.5 | 3 | 4.0000 | 25.32 |
| 1354 | 20.5 | 3 | 4.0000 | 21.70 |
| 1355 | 21.5 | 3 | 4.0000 | 19.67 |
| 1356 | 22.5 | 3 | 4.0000 | 18.51 |
| 1357 | 23.5 | 3 | 4.0000 | 17.85 |
| 1358 | 24.5 | 3 | 4.0000 | 17.55 |
| 1359 | 25.5 | 3 | 4.0000 | 17.55 |
| 1360 | 26.5 | 3 | 4.0000 | 17.85 |
| 1361 | 27.5 | 3 | 4.0000 | 18.51 |
| 1362 | 28.5 | 3 | 4.0000 | 19.67 |
| 1363 | 29.5 | 3 | 4.0000 | 21.70 |
| 1364 | 30.5 | 3 | 4.0000 | 25.32 |
| 1365 | 31.5 | 3 | 4.0000 | 29.86 |
| 1366 | 32.5 | 3 | 4.0000 | 33.22 |
| 1367 | 33.5 | 3 | 4.0000 | 24.67 |
| 1368 | 34.5 | 3 | 4.0000 | 18.29 |
| 1369 | 35.5 | 3 | 4.0000 | 16.19 |
| 1370 | 36.5 | 3 | 4.0000 | 15.14 |
| 1371 | 14.5 | 3 | 4.3333 | 15.06 |
| 1372 | 15.5 | 3 | 4.3333 | 16.27 |
| 1373 | 16.5 | 3 | 4.3333 | 20.08 |
| 1374 | 17.5 | 3 | 4.3333 | 23.05 |
| 1375 | 18.5 | 3 | 4.3333 | 22.67 |
| 1376 | 19.5 | 3 | 4.3333 | 22.34 |
| 1377 | 20.5 | 3 | 4.3333 | 20.22 |
| 1378 | 21.5 | 3 | 4.3333 | 18.79 |
| 1379 | 22.5 | 3 | 4.3333 | 17.91 |
| 1380 | 23.5 | 3 | 4.3333 | 17.40 |
| 1381 | 24.5 | 3 | 4.3333 | 17.16 |
| 1382 | 25.5 | 3 | 4.3333 | 17.16 |
| 1383 | 26.5 | 3 | 4.3333 | 17.40 |
| 1384 | 27.5 | 3 | 4.3333 | 17.91 |
| 1385 | 28.5 | 3 | 4.3333 | 18.79 |
| 1386 | 29.5 | 3 | 4.3333 | 20.22 |
| 1387 | 30.5 | 3 | 4.3333 | 22.35 |
| 1388 | 31.5 | 3 | 4.3333 | 22.67 |
| 1389 | 32.5 | 3 | 4.3333 | 23.05 |
| 1390 | 33.5 | 3 | 4.3333 | 20.08 |
| 1391 | 34.5 | 3 | 4.3333 | 16.28 |
| 1392 | 35.5 | 3 | 4.3333 | 15.07 |
| 1393 | 15.5 | 3 | 4.6667 | 13.60 |
| 1394 | 16.5 | 3 | 4.6667 | 14.35 |
| 1395 | 17.5 | 3 | 4.6667 | 17.52 |
| 1396 | 18.5 | 3 | 4.6667 | 19.87 |
| 1397 | 19.5 | 3 | 4.6667 | 20.73 |
| 1398 | 20.5 | 3 | 4.6667 | 19.27 |
| 1399 | 21.5 | 3 | 4.6667 | 18.18 |
| 1400 | 22.5 | 3 | 4.6667 | 17.47 |
| 1401 | 23.5 | 3 | 4.6667 | 17.05 |
| 1402 | 24.5 | 3 | 4.6667 | 16.86 |
| 1403 | 25.5 | 3 | 4.6667 | 16.86 |
| 1404 | 26.5 | 3 | 4.6667 | 17.06 |
| 1405 | 27.5 | 3 | 4.6667 | 17.47 |
| 1406 | 28.5 | 3 | 4.6667 | 18.18 |
| 1407 | 29.5 | 3 | 4.6667 | 19.27 |
| 1408 | 30.5 | 3 | 4.6667 | 20.73 |
| 1409 | 31.5 | 3 | 4.6667 | 19.87 |
| 1410 | 32.5 | 3 | 4.6667 | 17.52 |
| 1411 | 33.5 | 3 | 4.6667 | 14.35 |
| 1412 | 34.5 | 3 | 4.6667 | 13.60 |
| 1413 | 16.5 | 3 | 5.0000 | 11.23 |
| 1414 | 17.5 | 3 | 5.0000 | 15.34 |
| 1415 | 18.5 | 3 | 5.0000 | 18.33 |
| 1416 | 19.5 | 3 | 5.0000 | 19.68 |
| 1417 | 20.5 | 3 | 5.0000 | 18.60 |
| 1418 | 21.5 | 3 | 5.0000 | 17.72 |
| 1419 | 22.5 | 3 | 5.0000 | 17.14 |
| 1420 | 23.5 | 3 | 5.0000 | 16.79 |
| 1421 | 24.5 | 3 | 5.0000 | 16.62 |
| 1422 | 25.5 | 3 | 5.0000 | 16.62 |
| 1423 | 26.5 | 3 | 5.0000 | 16.79 |
| 1424 | 27.5 | 3 | 5.0000 | 17.14 |
| 1425 | 28.5 | 3 | 5.0000 | 17.72 |
| 1426 | 29.5 | 3 | 5.0000 | 18.61 |
| 1427 | 30.5 | 3 | 5.0000 | 19.68 |
| 1428 | 31.5 | 3 | 5.0000 | 18.33 |
| 1429 | 32.5 | 3 | 5.0000 | 15.34 |
| 1430 | 33.5 | 3 | 5.0000 | 11.24 |
| 1431 | 17.5 | 3 | 5.3333 | 14.12 |
| 1432 | 18.5 | 3 | 5.3333 | 17.33 |
| 1433 | 19.5 | 3 | 5.3333 | 18.94 |
| 1434 | 20.5 | 3 | 5.3333 | 18.11 |
| 1435 | 21.5 | 3 | 5.3333 | 17.37 |
| 1436 | 22.5 | 3 | 5.3333 | 16.88 |
| 1437 | 23.5 | 3 | 5.3333 | 16.57 |
| 1438 | 24.5 | 3 | 5.3333 | 16.43 |
| 1439 | 25.5 | 3 | 5.3333 | 16.43 |
| 1440 | 26.5 | 3 | 5.3333 | 16.57 |
| 1441 | 27.5 | 3 | 5.3333 | 16.88 |
| 1442 | 28.5 | 3 | 5.3333 | 17.37 |
| 1443 | 29.5 | 3 | 5.3333 | 18.11 |
| 1444 | 30.5 | 3 | 5.3333 | 18.94 |
| 1445 | 31.5 | 3 | 5.3333 | 17.33 |
| 1446 | 32.5 | 3 | 5.3333 | 14.13 |
| 1447 | 18.5 | 3 | 5.6667 | 16.61 |
| 1448 | 19.5 | 3 | 5.6667 | 18.39 |
| 1449 | 20.5 | 3 | 5.6667 | 17.73 |
| 1450 | 21.5 | 3 | 5.6667 | 17.10 |
| 1451 | 22.5 | 3 | 5.6667 | 16.67 |
| 1452 | 23.5 | 3 | 5.6667 | 16.40 |
| 1453 | 24.5 | 3 | 5.6667 | 16.27 |
| 1454 | 25.5 | 3 | 5.6667 | 16.27 |
| 1455 | 26.5 | 3 | 5.6667 | 16.40 |
| 1456 | 27.5 | 3 | 5.6667 | 16.67 |
| 1457 | 28.5 | 3 | 5.6667 | 17.10 |
| 1458 | 29.5 | 3 | 5.6667 | 17.74 |
| 1459 | 30.5 | 3 | 5.6667 | 18.40 |
| 1460 | 31.5 | 3 | 5.6667 | 16.62 |
| 1461 | 19.5 | 3 | 6.0000 | 17.97 |
| 1462 | 20.5 | 3 | 6.0000 | 17.44 |
| 1463 | 21.5 | 3 | 6.0000 | 16.89 |
| 1464 | 22.5 | 3 | 6.0000 | 16.50 |
| 1465 | 23.5 | 3 | 6.0000 | 16.26 |
| 1466 | 24.5 | 3 | 6.0000 | 16.15 |
| 1467 | 25.5 | 3 | 6.0000 | 16.15 |
| 1468 | 26.5 | 3 | 6.0000 | 16.26 |
| 1469 | 27.5 | 3 | 6.0000 | 16.50 |
| 1470 | 28.5 | 3 | 6.0000 | 16.89 |
| 1471 | 29.5 | 3 | 6.0000 | 17.44 |
| 1472 | 30.5 | 3 | 6.0000 | 17.97 |
| 1473 | 20.5 | 3 | 6.3333 | 17.20 |
| 1474 | 21.5 | 3 | 6.3333 | 16.71 |
| 1475 | 22.5 | 3 | 6.3333 | 16.37 |
| 1476 | 23.5 | 3 | 6.3333 | 16.15 |
| 1477 | 24.5 | 3 | 6.3333 | 16.04 |
| 1478 | 25.5 | 3 | 6.3333 | 16.04 |
| 1479 | 26.5 | 3 | 6.3333 | 16.15 |
| 1480 | 27.5 | 3 | 6.3333 | 16.37 |
| 1481 | 28.5 | 3 | 6.3333 | 16.71 |
| 1482 | 29.5 | 3 | 6.3333 | 17.2 |
| 1483 | 21.5 | 3 | 6.6667 | 16.56 |
| 1484 | 22.5 | 3 | 6.6667 | 16.25 |
| 1485 | 23.5 | 3 | 6.6667 | 16.05 |
| 1486 | 24.5 | 3 | 6.6667 | 15.96 |
| 1487 | 25.5 | 3 | 6.6667 | 15.96 |
| 1488 | 26.5 | 3 | 6.6667 | 16.06 |
| 1489 | 27.5 | 3 | 6.6667 | 16.26 |
| 1490 | 28.5 | 3 | 6.6667 | 16.57 |

TABLE 4.2b: DATA CO-ORDINATES

| LOCATIONS’ CO-ORDINATES | DATA POINTS |
| --- | --- |
| ASOKORO 1 9°2'59"N, 7°31'57"E | 1 |
| ASOKORO 2 9°2'31"N, 7°32'35"E | 2 |
| ABACHA BARRACKS 1 9°2'33"N,7°32'18"E | 3 |
| ABACHA BARRACKS 2 9°2'33"N, 7°32'21"E | 4 |
| ABACHA BARRACKS 3 9°2'34"N, 7°32'26"E | 5 |
| NAF VALLEY 1 9°2'31"N, 7°32'35"E | 6 |
| NAF VALLEY 2 9°2'32"N, 7°32'36"E | 7 |
| NAF VALLEY 3 9°2'31"N, 7°30'38"E | 8 |
| KUGBO 1 9°2'34"N, 7°31'26"E | 9 |
| KUGBO 2 9°2'35"N, 7°30'20"E | 10 |
| KUGBO 3 9°2'37"N, 7°34'9"E | 11 |
| KARU BRIDGE 1 9°1'16"N, 7°34'10"E | 12 |
| KARU BRIDGE 2 9°1'20"N, 7°34'10"E | 13 |
| NYANYA 1 9°1'13"N, 7°34'31"E | 14 |
| NYANYA 2 9°1'16"N, 7°34'30"E | 15 |
| MARABA 1 9°1'25"N, 7°34'18"E | 16 |
| MARABA 2 9°1' 24"N, 7°34'22" | 17 |
| MARABA 3 9°1'24"N, 7°32'22"E | 18 |
| MARABA 4 9°1'26"N, 7°30'10"E | 19 |
| MARABA 5 9°1'52"N, 7°40'10"E | 20 |
| ONE MAN VILLAGE 1 9°1'0"N, 7°37'8"E | 21 |
| ONE MAN VILLAGE 2 9°1'6"N, 7°37'12"E | 22 |
| ADO 1 9°1'10"N, 7°36'18"E | 23 |
| ADO 2 9°1'12"N, 7°36'20"E | 24 |
| ADO 3 9°1'14"N, 7°36'18"E | 25 |
| NEW NYANYA 1 9°0'24"N, 7°39'46"E | 26 |
| NEW NYANYA 2 9°0'28"N, 7°39'48"E | 27 |
| NEW NYANYA 3 9°0'26"N, 7°36'44"E | 28 |
| KARU BRIDGE 3 9°1'22"N, 7°34'14"E | 29 |
| GWANDARA 9°0'50"N, 7°58'46"E | 30 |
| MASAKA 1 9°0'52"N, 7°58'40"E | 31 |
| MASAKA 2 9°0'54"N, 7°58'2"E | 32 |
| MASAKA 3 9°0'50"N, 8°1'10"E | 33 |
| MASAKA 4 9°0'46"N,8°1'50"E | 34 |
| MASAKA 5 9°0'48"N, 8°2'22"E | 35 |

TABLE 4.2c:APPARENT RESISTIVITY DATA FOR MASAKA - ASOKORO ROAD PROFILEII (ASOKORO - ABACHA BARRACKS ROAD).

| S/N | MIDPOINT(X) ELECTRODE SPACING (m) | ELECTRODE SPACING "a" | APPARENT RESISTIVITY (Ω-m) |
| --- | --- | --- | --- |
| 1 | 2.50 | 1.00 | 9.80 |
| 2 | 3.50 | 1.00 | 9.81 |
| 3 | 4.50 | 1.00 | 9.81 |
| 4 | 5.50 | 1.00 | 9.83 |
| 5 | 6.50 | 1.00 | 9.91 |
| 6 | 7.50 | 1.00 | 10.97 |
| 7 | 8.50 | 1.00 | 13.86 |
| 8 | 9.50 | 1.00 | 15.77 |
| 9 | 10.50 | 1.00 | 15.77 |
| 10 | 11.50 | 1.00 | 13.86 |
| 11 | 12.50 | 1.00 | 10.98 |
| 12 | 13.50 | 1.00 | 9.91 |
| 13 | 14.50 | 1.00 | 9.83 |
| 14 | 15.50 | 1.00 | 9.82 |
| 15 | 16.50 | 1.00 | 9.82 |
| 16 | 17.50 | 1.00 | 9.82 |
| 17 | 18.50 | 1.00 | 9.83 |
| 18 | 19.50 | 1.00 | 9.84 |
| 19 | 20.50 | 1.00 | 9.88 |
| 20 | 21.50 | 1.00 | 9.96 |
| 21 | 22.50 | 1.00 | 10.14 |
| 22 | 23.50 | 1.00 | 10.38 |
| 23 | 24.50 | 1.00 | 10.48 |
| 24 | 25.50 | 1.00 | 10.50 |
| 25 | 26.50 | 1.00 | 10.50 |
| 26 | 27.50 | 1.00 | 10.48 |
| 27 | 28.50 | 1.00 | 10.38 |
| 28 | 29.50 | 1.00 | 10.14 |
| 29 | 30.50 | 1.00 | 9.96 |
| 30 | 31.50 | 1.00 | 9.88 |
| 31 | 32.50 | 1.00 | 9.84 |
| 32 | 33.50 | 1.00 | 9.83 |
| 33 | 34.50 | 1.00 | 9.82 |
| 34 | 35.50 | 1.00 | 9.81 |
| 35 | 36.50 | 1.00 | 9.81 |
| 36 | 37.50 | 1.00 | 9.81 |
| 37 | 38.50 | 1.00 | 9.81 |
| 38 | 39.50 | 1.00 | 9.80 |
| 39 | 40.50 | 1.00 | 9.80 |
| 40 | 41.50 | 1.00 | 9.79 |
| 41 | 42.50 | 1.00 | 9.77 |
| 42 | 43.50 | 1.00 | 9.72 |
| 43 | 44.50 | 1.00 | 9.51 |
| 44 | 45.50 | 1.00 | 8.29 |
| 45 | 46.50 | 1.00 | 7.86 |
| 46 | 47.50 | 1.00 | 5.24 |
| 47 | 48.50 | 1.00 | 4.56 |
| 48 | 49.50 | 1.00 | 6.98 |
| 49 | 50.50 | 1.00 | 7.92 |
| 50 | 51.50 | 1.00 | 9.15 |
| 51 | 52.50 | 1.00 | 9.65 |
| 52 | 53.50 | 1.00 | 9.75 |
| 53 | 54.50 | 1.00 | 9.78 |
| 54 | 55.50 | 1.00 | 9.79 |
| 55 | 56.50 | 1.00 | 9.80 |
| 56 | 57.50 | 1.00 | 9.80 |
| 57 | 58.50 | 1.00 | 9.81 |
| 58 | 59.50 | 1.00 | 9.82 |
| 59 | 60.50 | 1.00 | 9.91 |
| 60 | 61.50 | 1.00 | 10.97 |
| 61 | 62.50 | 1.00 | 13.86 |
| 62 | 63.50 | 1.00 | 15.77 |
| 63 | 64.50 | 1.00 | 15.77 |
| 64 | 65.50 | 1.00 | 13.86 |
| 65 | 66.50 | 1.00 | 10.97 |
| 66 | 67.50 | 1.00 | 9.91 |
| 67 | 68.50 | 1.00 | 9.83 |
| 68 | 69.50 | 1.00 | 9.82 |
| 69 | 70.50 | 1.00 | 9.83 |
| 70 | 71.50 | 1.00 | 9.87 |
| 71 | 72.50 | 1.00 | 9.97 |
| 72 | 73.50 | 1.00 | 10.40 |
| 73 | 74.50 | 1.00 | 11.47 |
| 74 | 75.50 | 1.00 | 12.33 |
| 75 | 76.50 | 1.00 | 12.01 |
| 76 | 77.50 | 1.00 | 12.18 |
| 77 | 78.50 | 1.00 | 10.85 |
| 78 | 79.50 | 1.00 | 10.12 |
| 79 | 80.50 | 1.00 | 9.90 |
| 80 | 81.50 | 1.00 | 9.84 |
| 81 | 82.50 | 1.00 | 9.82 |
| 82 | 83.50 | 1.00 | 9.81 |
| 83 | 84.50 | 1.00 | 9.81 |
| 84 | 85.50 | 1.00 | 9.81 |
| 85 | 86.50 | 1.00 | 9.80 |
| 86 | 87.50 | 1.00 | 9.80 |
| 87 | 88.50 | 1.00 | 9.80 |
| 88 | 89.50 | 1.00 | 9.80 |
| 89 | 90.50 | 1.00 | 9.80 |
| 90 | 91.50 | 1.00 | 9.80 |
| 91 | 92.50 | 1.00 | 9.80 |
| 92 | 93.50 | 1.00 | 9.80 |
| 93 | 94.50 | 1.00 | 9.80 |
| 94 | 95.50 | 1.00 | 9.80 |
| 95 | 96.50 | 1.00 | 9.80 |
| 96 | 97.50 | 1.00 | 9.80 |
| 97 | 98.50 | 1.00 | 9.80 |
| 98 | 3.00 | 2.00 | 9.92 |
| 99 | 4.00 | 2.00 | 9.96 |
| 100 | 5.00 | 2.00 | 10.08 |
| 101 | 6.00 | 2.00 | 10.83 |
| 102 | 7.00 | 2.00 | 11.32 |
| 103 | 8.00 | 2.00 | 10.67 |
| 104 | 9.00 | 2.00 | 12.03 |
| 105 | 10.00 | 2.00 | 13.12 |
| 106 | 11.00 | 2.00 | 12.04 |
| 107 | 12.00 | 2.00 | 10.69 |
| 108 | 13.00 | 2.00 | 11.34 |
| 109 | 14.00 | 2.00 | 10.86 |
| 110 | 15.00 | 2.00 | 10.13 |
| 111 | 16.00 | 2.00 | 10.02 |
| 112 | 17.00 | 2.00 | 10.02 |
| 113 | 18.00 | 2.00 | 10.07 |
| 114 | 19.00 | 2.00 | 10.16 |
| 115 | 20.00 | 2.00 | 10.36 |
| 116 | 21.00 | 2.00 | 10.73 |
| 117 | 22.00 | 2.00 | 11.32 |
| 118 | 23.00 | 2.00 | 12.02 |
| 119 | 24.00 | 2.00 | 12.38 |
| 120 | 25.00 | 2.00 | 12.27 |
| 121 | 26.00 | 2.00 | 12.12 |
| 122 | 27.00 | 2.00 | 12.27 |
| 123 | 28.00 | 2.00 | 12.38 |
| 124 | 29.00 | 2.00 | 12.02 |
| 125 | 30.00 | 2.00 | 11.32 |
| 126 | 31.00 | 2.00 | 10.73 |
| 127 | 32.00 | 2.00 | 10.35 |
| 128 | 33.00 | 2.00 | 10.16 |
| 129 | 34.00 | 2.00 | 10.06 |
| 130 | 35.00 | 2.00 | 10.00 |
| 131 | 36.00 | 2.00 | 9.97 |
| 132 | 37.00 | 2.00 | 9.95 |
| 133 | 38.00 | 2.00 | 9.93 |
| 134 | 39.00 | 2.00 | 9.91 |
| 135 | 40.00 | 2.00 | 9.88 |
| 136 | 41.00 | 2.00 | 9.83 |
| 137 | 42.00 | 2.00 | 9.71 |
| 138 | 43.00 | 2.00 | 9.34 |
| 139 | 44.00 | 2.00 | 8.06 |
| 140 | 45.00 | 2.00 | 7.82 |
| 141 | 46.00 | 2.00 | 8.24 |
| 142 | 47.00 | 2.00 | 6.76 |
| 143 | 48.00 | 2.00 | 4.12 |
| 144 | 49.00 | 2.00 | 5.55 |
| 145 | 50.00 | 2.00 | 7.72 |
| 146 | 51.00 | 2.00 | 8.02 |
| 147 | 52.00 | 2.00 | 7.78 |
| 148 | 53.00 | 2.00 | 8.87 |
| 149 | 54.00 | 2.00 | 9.57 |
| 150 | 55.00 | 2.00 | 9.77 |
| 151 | 56.00 | 2.00 | 9.85 |
| 152 | 57.00 | 2.00 | 9.90 |
| 153 | 58.00 | 2.00 | 9.94 |
| 154 | 59.00 | 2.00 | 10.07 |
| 155 | 60.00 | 2.00 | 10.82 |
| 156 | 61.00 | 2.00 | 11.31 |
| 157 | 62.00 | 2.00 | 10.67 |
| 158 | 63.00 | 2.00 | 12.02 |
| 159 | 64.00 | 2.00 | 13.11 |
| 160 | 65.00 | 2.00 | 12.03 |
| 161 | 66.00 | 2.00 | 10.68 |
| 162 | 67.00 | 2.00 | 11.34 |
| 163 | 68.00 | 2.00 | 10.87 |
| 164 | 69.00 | 2.00 | 10.16 |
| 165 | 70.00 | 2.00 | 10.13 |
| 166 | 71.00 | 2.00 | 10.32 |
| 167 | 72.00 | 2.00 | 10.87 |
| 168 | 73.00 | 2.00 | 11.70 |
| 169 | 74.00 | 2.00 | 12.69 |
| 170 | 75.00 | 2.00 | 13.05 |
| 171 | 76.00 | 2.00 | 13.07 |
| 172 | 77.00 | 2.00 | 13.02 |
| 173 | 78.00 | 2.00 | 13.02 |
| 174 | 79.00 | 2.00 | 12.19 |
| 175 | 80.00 | 2.00 | 11.25 |
| 176 | 81.00 | 2.00 | 10.52 |
| 177 | 82.00 | 2.00 | 10.16 |
| 178 | 83.00 | 2.00 | 10.02 |
| 179 | 84.00 | 2.00 | 9.97 |
| 180 | 85.00 | 2.00 | 9.94 |
| 181 | 86.00 | 2.00 | 9.93 |
| 182 | 87.00 | 2.00 | 9.92 |
| 183 | 88.00 | 2.00 | 9.92 |
| 184 | 89.00 | 2.00 | 9.92 |
| 185 | 90.00 | 2.00 | 9.91 |
| 186 | 91.00 | 2.00 | 9.91 |
| 187 | 92.00 | 2.00 | 9.91 |
| 188 | 93.00 | 2.00 | 9.91 |
| 189 | 94.00 | 2.00 | 9.91 |
| 190 | 95.00 | 2.00 | 9.91 |
| 191 | 96.00 | 2.00 | 9.91 |
| 192 | 97.00 | 2.00 | 9.90 |
| 193 | 4.50 | 3.00 | 10.65 |
| 194 | 5.50 | 3.00 | 10.65 |
| 195 | 6.50 | 3.00 | 9.68 |
| 196 | 7.50 | 3.00 | 9.54 |
| 197 | 8.50 | 3.00 | 11.01 |
| 198 | 9.50 | 3.00 | 11.71 |
| 199 | 10.50 | 3.00 | 11.72 |
| 200 | 11.50 | 3.00 | 11.04 |
| 201 | 12.50 | 3.00 | 9.58 |
| 202 | 13.50 | 3.00 | 9.76 |
| 203 | 14.50 | 3.00 | 10.77 |
| 204 | 15.50 | 3.00 | 10.84 |
| 205 | 16.50 | 3.00 | 10.39 |
| 206 | 17.50 | 3.00 | 10.41 |
| 207 | 18.50 | 3.00 | 10.63 |
| 208 | 19.50 | 3.00 | 11.01 |
| 209 | 20.50 | 3.00 | 11.57 |
| 210 | 21.50 | 3.00 | 12.33 |
| 211 | 22.50 | 3.00 | 13.15 |
| 212 | 23.50 | 3.00 | 13.51 |
| 213 | 24.50 | 3.00 | 13.33 |
| 214 | 25.50 | 3.00 | 13.09 |
| 215 | 26.50 | 3.00 | 13.09 |
| 216 | 27.50 | 3.00 | 13.33 |
| 217 | 28.50 | 3.00 | 13.50 |
| 218 | 29.50 | 3.00 | 13.15 |
| 219 | 30.50 | 3.00 | 12.33 |
| 220 | 31.50 | 3.00 | 11.56 |
| 221 | 32.50 | 3.00 | 10.99 |
| 222 | 33.50 | 3.00 | 10.59 |
| 223 | 34.50 | 3.00 | 10.34 |
| 224 | 35.50 | 3.00 | 10.19 |
| 225 | 36.50 | 3.00 | 10.10 |
| 226 | 37.50 | 3.00 | 10.03 |
| 227 | 38.50 | 3.00 | 9.96 |
| 228 | 39.50 | 3.00 | 9.86 |
| 229 | 40.50 | 3.00 | 9.69 |
| 230 | 41.50 | 3.00 | 9.28 |
| 231 | 42.50 | 3.00 | 8.21 |
| 232 | 43.50 | 3.00 | 8.19 |
| 233 | 44.50 | 3.00 | 8.85 |
| 234 | 45.50 | 3.00 | 9.33 |
| 235 | 46.50 | 3.00 | 7.81 |
| 236 | 47.50 | 3.00 | 6.15 |
| 237 | 48.50 | 3.00 | 5.47 |
| 238 | 49.50 | 3.00 | 6.97 |
| 239 | 50.50 | 3.00 | 8.63 |
| 240 | 51.50 | 3.00 | 9.27 |
| 241 | 52.50 | 3.00 | 8.45 |
| 242 | 53.50 | 3.00 | 8.04 |
| 243 | 54.50 | 3.00 | 8.83 |
| 244 | 55.50 | 3.00 | 9.52 |
| 245 | 56.50 | 3.00 | 9.80 |
| 246 | 57.50 | 3.00 | 10.02 |
| 247 | 58.50 | 3.00 | 10.60 |
| 248 | 59.50 | 3.00 | 10.61 |
| 249 | 60.50 | 3.00 | 9.65 |
| 250 | 61.50 | 3.00 | 9.51 |
| 251 | 62.50 | 3.00 | 10.99 |
| 252 | 63.50 | 3.00 | 11.68 |
| 253 | 64.50 | 3.00 | 11.70 |
| 254 | 65.50 | 3.00 | 11.02 |
| 255 | 66.50 | 3.00 | 9.58 |
| 256 | 67.50 | 3.00 | 9.79 |
| 257 | 68.50 | 3.00 | 10.88 |
| 258 | 69.50 | 3.00 | 11.15 |
| 259 | 70.50 | 3.00 | 11.12 |
| 260 | 71.50 | 3.00 | 11.42 |
| 261 | 72.50 | 3.00 | 11.55 |
| 262 | 73.50 | 3.00 | 11.60 |
| 263 | 74.50 | 3.00 | 12.37 |
| 264 | 75.50 | 3.00 | 12.84 |
| 265 | 76.50 | 3.00 | 13.05 |
| 266 | 77.50 | 3.00 | 12.61 |
| 267 | 78.50 | 3.00 | 12.03 |
| 268 | 79.50 | 3.00 | 11.44 |
| 269 | 80.50 | 3.00 | 11.49 |
| 270 | 81.50 | 3.00 | 11.17 |
| 271 | 82.50 | 3.00 | 10.66 |
| 272 | 83.50 | 3.00 | 10.28 |
| 273 | 84.50 | 3.00 | 10.11 |
| 274 | 85.50 | 3.00 | 10.03 |
| 275 | 86.50 | 3.00 | 9.99 |
| 276 | 87.50 | 3.00 | 9.97 |
| 277 | 88.50 | 3.00 | 9.96 |
| 278 | 89.50 | 3.00 | 9.95 |
| 279 | 90.50 | 3.00 | 9.95 |
| 280 | 91.50 | 3.00 | 9.94 |
| 281 | 92.50 | 3.00 | 9.94 |
| 282 | 93.50 | 3.00 | 9.94 |
| 283 | 94.50 | 3.00 | 9.93 |
| 284 | 95.50 | 3.00 | 9.93 |
| 285 | 6.00 | 4.00 | 9.44 |
| 286 | 7.00 | 4.00 | 9.43 |
| 287 | 8.00 | 4.00 | 10.62 |
| 288 | 9.00 | 4.00 | 11.24 |
| 289 | 10.00 | 4.00 | 10.89 |
| 290 | 11.00 | 4.00 | 11.29 |
| 291 | 12.00 | 4.00 | 10.72 |
| 292 | 13.00 | 4.00 | 9.58 |
| 293 | 14.00 | 4.00 | 9.69 |
| 294 | 15.00 | 4.00 | 9.97 |
| 295 | 16.00 | 4.00 | 10.92 |
| 296 | 17.00 | 4.00 | 11.33 |
| 297 | 18.00 | 4.00 | 11.28 |
| 298 | 19.00 | 4.00 | 11.61 |
| 299 | 20.00 | 4.00 | 12.11 |
| 300 | 21.00 | 4.00 | 12.75 |
| 301 | 22.00 | 4.00 | 13.37 |
| 302 | 23.00 | 4.00 | 13.63 |
| 303 | 24.00 | 4.00 | 13.68 |
| 304 | 25.00 | 4.00 | 13.72 |
| 305 | 26.00 | 4.00 | 13.73 |
| 306 | 27.00 | 4.00 | 13.71 |
| 307 | 28.00 | 4.00 | 13.67 |
| 308 | 29.00 | 4.00 | 13.62 |
| 309 | 30.00 | 4.00 | 13.35 |
| 310 | 31.00 | 4.00 | 12.73 |
| 311 | 32.00 | 4.00 | 12.07 |
| 312 | 33.00 | 4.00 | 11.52 |
| 313 | 34.00 | 4.00 | 11.08 |
| 314 | 35.00 | 4.00 | 10.71 |
| 315 | 36.00 | 4.00 | 10.43 |
| 316 | 37.00 | 4.00 | 10.22 |
| 317 | 38.00 | 4.00 | 10.03 |
| 318 | 39.00 | 4.00 | 9.79 |
| 319 | 40.00 | 4.00 | 9.35 |
| 320 | 41.00 | 4.00 | 8.45 |
| 321 | 42.00 | 4.00 | 8.52 |
| 322 | 43.00 | 4.00 | 9.17 |
| 323 | 44.00 | 4.00 | 9.77 |
| 324 | 45.00 | 4.00 | 9.78 |
| 325 | 46.00 | 4.00 | 8.41 |
| 326 | 47.00 | 4.00 | 7.11 |
| 327 | 48.00 | 4.00 | 6.98 |
| 328 | 49.00 | 4.00 | 6.72 |
| 329 | 50.00 | 4.00 | 7.70 |
| 330 | 51.00 | 4.00 | 9.11 |
| 331 | 52.00 | 4.00 | 9.82 |
| 332 | 53.00 | 4.00 | 9.54 |
| 333 | 54.00 | 4.00 | 8.75 |
| 334 | 55.00 | 4.00 | 8.35 |
| 335 | 56.00 | 4.00 | 9.02 |
| 336 | 57.00 | 4.00 | 10.03 |
| 337 | 58.00 | 4.00 | 10.13 |
| 338 | 59.00 | 4.00 | 9.46 |
| 339 | 60.00 | 4.00 | 9.36 |
| 340 | 61.00 | 4.00 | 9.36 |
| 341 | 62.00 | 4.00 | 10.56 |
| 342 | 63.00 | 4.00 | 11.18 |
| 343 | 64.00 | 4.00 | 10.84 |
| 344 | 65.00 | 4.00 | 11.25 |
| 345 | 66.00 | 4.00 | 10.72 |
| 346 | 67.00 | 4.00 | 9.66 |
| 347 | 68.00 | 4.00 | 9.95 |
| 348 | 69.00 | 4.00 | 10.53 |
| 349 | 70.00 | 4.00 | 11.49 |
| 350 | 71.00 | 4.00 | 11.43 |
| 351 | 72.00 | 4.00 | 10.58 |
| 352 | 73.00 | 4.00 | 10.79 |
| 353 | 74.00 | 4.00 | 11.75 |
| 354 | 75.00 | 4.00 | 12.37 |
| 355 | 76.00 | 4.00 | 12.60 |
| 356 | 77.00 | 4.00 | 12.57 |
| 357 | 78.00 | 4.00 | 12.07 |
| 358 | 79.00 | 4.00 | 11.26 |
| 359 | 80.00 | 4.00 | 10.45 |
| 360 | 81.00 | 4.00 | 10.57 |
| 361 | 82.00 | 4.00 | 11.02 |
| 362 | 83.00 | 4.00 | 11.01 |
| 363 | 84.00 | 4.00 | 10.66 |
| 364 | 85.00 | 4.00 | 10.32 |
| 365 | 86.00 | 4.00 | 10.15 |
| 366 | 87.00 | 4.00 | 10.05 |
| 367 | 88.00 | 4.00 | 10.01 |
| 368 | 89.00 | 4.00 | 9.98 |
| 369 | 90.00 | 4.00 | 9.96 |
| 370 | 91.00 | 4.00 | 9.95 |
| 371 | 92.00 | 4.00 | 9.94 |
| 372 | 93.00 | 4.00 | 9.94 |
| 373 | 94.00 | 4.00 | 9.93 |
| 374 | 7.50 | 5.00 | 10.44 |
| 375 | 8.50 | 5.00 | 11.02 |
| 376 | 9.50 | 5.00 | 10.68 |
| 377 | 10.50 | 5.00 | 10.73 |
| 378 | 11.50 | 5.00 | 11.17 |
| 379 | 12.50 | 5.00 | 10.71 |
| 380 | 13.50 | 5.00 | 9.87 |
| 381 | 14.50 | 5.00 | 10.13 |
| 382 | 15.50 | 5.00 | 10.45 |
| 383 | 16.50 | 5.00 | 10.81 |
| 384 | 17.50 | 5.00 | 11.74 |
| 385 | 18.50 | 5.00 | 12.26 |
| 386 | 19.50 | 5.00 | 12.31 |
| 387 | 20.50 | 5.00 | 12.60 |
| 388 | 21.50 | 5.00 | 12.99 |
| 389 | 22.50 | 5.00 | 13.38 |
| 390 | 23.50 | 5.00 | 13.71 |
| 391 | 24.50 | 5.00 | 13.99 |
| 392 | 25.50 | 5.00 | 14.22 |
| 393 | 26.50 | 5.00 | 14.22 |
| 394 | 27.50 | 5.00 | 13.98 |
| 395 | 28.50 | 5.00 | 13.69 |
| 396 | 29.50 | 5.00 | 13.35 |
| 397 | 30.50 | 5.00 | 12.94 |
| 398 | 31.50 | 5.00 | 12.51 |
| 399 | 32.50 | 5.00 | 12.11 |
| 400 | 33.50 | 5.00 | 11.73 |
| 401 | 34.50 | 5.00 | 11.38 |
| 402 | 35.50 | 5.00 | 11.02 |
| 403 | 36.50 | 5.00 | 10.64 |
| 404 | 37.50 | 5.00 | 10.22 |
| 405 | 38.50 | 5.00 | 9.68 |
| 406 | 39.50 | 5.00 | 8.84 |
| 407 | 40.50 | 5.00 | 8.91 |
| 408 | 41.50 | 5.00 | 9.46 |
| 409 | 42.50 | 5.00 | 9.95 |
| 410 | 43.50 | 5.00 | 10.08 |
| 411 | 44.50 | 5.00 | 10.02 |
| 412 | 45.50 | 5.00 | 8.80 |
| 413 | 46.50 | 5.00 | 7.69 |
| 414 | 47.50 | 5.00 | 7.71 |
| 415 | 48.50 | 5.00 | 7.90 |
| 416 | 49.50 | 5.00 | 7.38 |
| 417 | 50.50 | 5.00 | 8.16 |
| 418 | 51.50 | 5.00 | 9.39 |
| 419 | 52.50 | 5.00 | 10.05 |
| 420 | 53.50 | 5.00 | 9.97 |
| 421 | 54.50 | 5.00 | 9.81 |
| 422 | 55.50 | 5.00 | 9.41 |
| 423 | 56.50 | 5.00 | 8.78 |
| 424 | 57.50 | 5.00 | 8.56 |
| 425 | 58.50 | 5.00 | 9.04 |
| 426 | 59.50 | 5.00 | 9.23 |
| 427 | 60.50 | 5.00 | 9.28 |
| 428 | 61.50 | 5.00 | 10.31 |
| 429 | 62.50 | 5.00 | 10.90 |
| 430 | 63.50 | 5.00 | 10.58 |
| 431 | 64.50 | 5.00 | 10.67 |
| 432 | 65.50 | 5.00 | 11.18 |
| 433 | 66.50 | 5.00 | 10.88 |
| 434 | 67.50 | 5.00 | 10.24 |
| 435 | 68.50 | 5.00 | 10.37 |
| 436 | 69.50 | 5.00 | 10.10 |
| 437 | 70.50 | 5.00 | 9.60 |
| 438 | 71.50 | 5.00 | 10.09 |
| 439 | 72.50 | 5.00 | 10.72 |
| 440 | 73.50 | 5.00 | 11.41 |
| 441 | 74.50 | 5.00 | 11.99 |
| 442 | 75.50 | 5.00 | 12.17 |
| 443 | 76.50 | 5.00 | 12.12 |
| 444 | 77.50 | 5.00 | 12.13 |
| 445 | 78.50 | 5.00 | 11.63 |
| 446 | 79.50 | 5.00 | 10.78 |
| 447 | 80.50 | 5.00 | 9.93 |
| 448 | 81.50 | 5.00 | 9.81 |
| 449 | 82.50 | 5.00 | 10.16 |
| 450 | 83.50 | 5.00 | 10.72 |
| 451 | 84.50 | 5.00 | 10.85 |
| 452 | 85.50 | 5.00 | 10.63 |
| 453 | 86.50 | 5.00 | 10.33 |
| 454 | 87.50 | 5.00 | 10.15 |
| 455 | 88.50 | 5.00 | 10.06 |
| 456 | 89.50 | 5.00 | 10.00 |
| 457 | 90.50 | 5.00 | 9.97 |
| 458 | 91.50 | 5.00 | 9.95 |
| 459 | 92.50 | 5.00 | 9.93 |
| 460 | 9.00 | 6.00 | 10.63 |
| 461 | 10.00 | 6.00 | 10.62 |
| 462 | 11.00 | 6.00 | 10.81 |
| 463 | 12.00 | 6.00 | 11.29 |
| 464 | 13.00 | 6.00 | 10.97 |
| 465 | 14.00 | 6.00 | 10.39 |
| 466 | 15.00 | 6.00 | 10.75 |
| 467 | 16.00 | 6.00 | 11.03 |
| 468 | 17.00 | 6.00 | 11.22 |
| 469 | 18.00 | 6.00 | 11.37 |
| 470 | 19.00 | 6.00 | 11.98 |
| 471 | 20.00 | 6.00 | 12.37 |
| 472 | 21.00 | 6.00 | 12.62 |
| 473 | 22.00 | 6.00 | 13.14 |
| 474 | 23.00 | 6.00 | 13.63 |
| 475 | 24.00 | 6.00 | 14.08 |
| 476 | 25.00 | 6.00 | 14.45 |
| 477 | 26.00 | 6.00 | 14.61 |
| 478 | 27.00 | 6.00 | 14.44 |
| 479 | 28.00 | 6.00 | 14.04 |
| 480 | 29.00 | 6.00 | 13.57 |
| 481 | 30.00 | 6.00 | 13.04 |
| 482 | 31.00 | 6.00 | 12.43 |
| 483 | 32.00 | 6.00 | 11.91 |
| 484 | 33.00 | 6.00 | 11.68 |
| 485 | 34.00 | 6.00 | 11.53 |
| 486 | 35.00 | 6.00 | 11.31 |
| 487 | 36.00 | 6.00 | 10.97 |
| 488 | 37.00 | 6.00 | 10.44 |
| 489 | 38.00 | 6.00 | 9.61 |
| 490 | 39.00 | 6.00 | 9.56 |
| 491 | 40.00 | 6.00 | 9.93 |
| 492 | 41.00 | 6.00 | 10.24 |
| 493 | 42.00 | 6.00 | 10.27 |
| 494 | 43.00 | 6.00 | 10.30 |
| 495 | 44.00 | 6.00 | 10.21 |
| 496 | 45.00 | 6.00 | 9.11 |
| 497 | 46.00 | 6.00 | 8.11 |
| 498 | 47.00 | 6.00 | 8.14 |
| 499 | 48.00 | 6.00 | 8.51 |
| 500 | 49.00 | 6.00 | 8.36 |
| 501 | 50.00 | 6.00 | 7.81 |
| 502 | 51.00 | 6.00 | 8.50 |
| 503 | 52.00 | 6.00 | 9.61 |
| 504 | 53.00 | 6.00 | 10.26 |
| 505 | 54.00 | 6.00 | 10.43 |
| 506 | 55.00 | 6.00 | 10.12 |
| 507 | 56.00 | 6.00 | 9.40 |
| 508 | 57.00 | 6.00 | 8.76 |
| 509 | 58.00 | 6.00 | 8.34 |
| 510 | 59.00 | 6.00 | 8.59 |
| 511 | 60.00 | 6.00 | 8.98 |
| 512 | 61.00 | 6.00 | 10.02 |
| 513 | 62.00 | 6.00 | 10.67 |
| 514 | 63.00 | 6.00 | 10.47 |
| 515 | 64.00 | 6.00 | 10.54 |
| 516 | 65.00 | 6.00 | 10.87 |
| 517 | 66.00 | 6.00 | 11.48 |
| 518 | 67.00 | 6.00 | 10.94 |
| 519 | 68.00 | 6.00 | 9.78 |
| 520 | 69.00 | 6.00 | 9.33 |
| 521 | 70.00 | 6.00 | 9.18 |
| 522 | 71.00 | 6.00 | 9.22 |
| 523 | 72.00 | 6.00 | 9.67 |
| 524 | 73.00 | 6.00 | 11.12 |
| 525 | 74.00 | 6.00 | 12.00 |
| 526 | 75.00 | 6.00 | 11.94 |
| 527 | 76.00 | 6.00 | 11.75 |
| 528 | 77.00 | 6.00 | 11.76 |
| 529 | 78.00 | 6.00 | 11.79 |
| 530 | 79.00 | 6.00 | 11.29 |
| 531 | 80.00 | 6.00 | 10.47 |
| 532 | 81.00 | 6.00 | 9.67 |
| 533 | 82.00 | 6.00 | 9.48 |
| 534 | 83.00 | 6.00 | 9.57 |
| 535 | 84.00 | 6.00 | 9.95 |
| 536 | 85.00 | 6.00 | 10.51 |
| 537 | 86.00 | 6.00 | 10.72 |
| 538 | 87.00 | 6.00 | 10.57 |
| 539 | 88.00 | 6.00 | 10.31 |
| 540 | 89.00 | 6.00 | 10.14 |
| 541 | 90.00 | 6.00 | 10.05 |
| 542 | 91.00 | 6.00 | 9.98 |
| 543 | 10.50 | 7.00 | 10.82 |
| 544 | 11.50 | 7.00 | 11.11 |
| 545 | 12.50 | 7.00 | 11.63 |
| 546 | 13.50 | 7.00 | 11.36 |
| 547 | 14.50 | 7.00 | 10.79 |
| 548 | 15.50 | 7.00 | 10.97 |
| 549 | 16.50 | 7.00 | 11.05 |
| 550 | 17.50 | 7.00 | 10.99 |
| 551 | 18.50 | 7.00 | 10.94 |
| 552 | 19.50 | 7.00 | 11.19 |
| 553 | 20.50 | 7.00 | 12.24 |
| 554 | 21.50 | 7.00 | 13.16 |
| 555 | 22.50 | 7.00 | 13.60 |
| 556 | 23.50 | 7.00 | 14.09 |
| 557 | 24.50 | 7.00 | 14.53 |
| 558 | 25.50 | 7.00 | 14.79 |
| 559 | 26.50 | 7.00 | 14.78 |
| 560 | 27.50 | 7.00 | 14.49 |
| 561 | 28.50 | 7.00 | 13.99 |
| 562 | 29.50 | 7.00 | 13.41 |
| 563 | 30.50 | 7.00 | 12.74 |
| 564 | 31.50 | 7.00 | 11.98 |
| 565 | 32.50 | 7.00 | 11.32 |
| 566 | 33.50 | 7.00 | 10.97 |
| 567 | 34.50 | 7.00 | 10.85 |
| 568 | 35.50 | 7.00 | 10.64 |
| 569 | 36.50 | 7.00 | 10.15 |
| 570 | 37.50 | 7.00 | 10.28 |
| 571 | 38.50 | 7.00 | 10.67 |
| 572 | 39.50 | 7.00 | 10.86 |
| 573 | 40.50 | 7.00 | 10.73 |
| 574 | 41.50 | 7.00 | 10.62 |
| 575 | 42.50 | 7.00 | 10.58 |
| 576 | 43.50 | 7.00 | 10.45 |
| 577 | 44.50 | 7.00 | 9.43 |
| 578 | 45.50 | 7.00 | 8.48 |
| 579 | 46.50 | 7.00 | 8.48 |
| 580 | 47.50 | 7.00 | 8.86 |
| 581 | 48.50 | 7.00 | 8.92 |
| 582 | 49.50 | 7.00 | 8.67 |
| 583 | 50.50 | 7.00 | 8.16 |
| 584 | 51.50 | 7.00 | 8.85 |
| 585 | 52.50 | 7.00 | 10.04 |
| 586 | 53.50 | 7.00 | 10.33 |
| 587 | 54.50 | 7.00 | 9.79 |
| 588 | 55.50 | 7.00 | 9.72 |
| 589 | 56.50 | 7.00 | 9.68 |
| 590 | 57.50 | 7.00 | 9.51 |
| 591 | 58.50 | 7.00 | 8.90 |
| 592 | 59.50 | 7.00 | 8.42 |
| 593 | 60.50 | 7.00 | 9.36 |
| 594 | 61.50 | 7.00 | 10.28 |
| 595 | 62.50 | 7.00 | 10.33 |
| 596 | 63.50 | 7.00 | 10.53 |
| 597 | 64.50 | 7.00 | 10.81 |
| 598 | 65.50 | 7.00 | 10.84 |
| 599 | 66.50 | 7.00 | 10.75 |
| 600 | 67.50 | 7.00 | 9.83 |
| 601 | 68.50 | 7.00 | 8.96 |
| 602 | 69.50 | 7.00 | 8.99 |
| 603 | 70.50 | 7.00 | 9.13 |
| 604 | 71.50 | 7.00 | 9.56 |
| 605 | 72.50 | 7.00 | 10.41 |
| 606 | 73.50 | 7.00 | 11.03 |
| 607 | 74.50 | 7.00 | 11.67 |
| 608 | 75.50 | 7.00 | 11.76 |
| 609 | 76.50 | 7.00 | 11.50 |
| 610 | 77.50 | 7.00 | 11.52 |
| 611 | 78.50 | 7.00 | 11.54 |
| 612 | 79.50 | 7.00 | 11.05 |
| 613 | 80.50 | 7.00 | 10.26 |
| 614 | 81.50 | 7.00 | 9.54 |
| 615 | 82.50 | 7.00 | 9.35 |
| 616 | 83.50 | 7.00 | 9.36 |
| 617 | 84.50 | 7.00 | 9.49 |
| 618 | 85.50 | 7.00 | 9.83 |
| 619 | 86.50 | 7.00 | 10.36 |
| 620 | 87.50 | 7.00 | 10.60 |
| 621 | 88.50 | 7.00 | 10.51 |
| 622 | 89.50 | 7.00 | 10.27 |
| 623 | 12.00 | 8.00 | 11.44 |
| 624 | 13.00 | 8.00 | 11.76 |
| 625 | 14.00 | 8.00 | 11.32 |
| 626 | 15.00 | 8.00 | 10.63 |
| 627 | 16.00 | 8.00 | 10.55 |
| 628 | 17.00 | 8.00 | 10.44 |
| 629 | 18.00 | 8.00 | 10.51 |
| 630 | 19.00 | 8.00 | 10.88 |
| 631 | 20.00 | 8.00 | 11.55 |
| 632 | 21.00 | 8.00 | 12.32 |
| 633 | 22.00 | 8.00 | 13.42 |
| 634 | 23.00 | 8.00 | 14.27 |
| 635 | 24.00 | 8.00 | 14.61 |
| 636 | 25.00 | 8.00 | 14.87 |
| 637 | 26.00 | 8.00 | 14.92 |
| 638 | 27.00 | 8.00 | 14.80 |
| 639 | 28.00 | 8.00 | 14.44 |
| 640 | 29.00 | 8.00 | 13.88 |
| 641 | 30.00 | 8.00 | 13.21 |
| 642 | 31.00 | 8.00 | 12.43 |
| 643 | 32.00 | 8.00 | 11.55 |
| 644 | 33.00 | 8.00 | 10.71 |
| 645 | 34.00 | 8.00 | 10.07 |
| 646 | 35.00 | 8.00 | 9.60 |
| 647 | 36.00 | 8.00 | 9.99 |
| 648 | 37.00 | 8.00 | 10.65 |
| 649 | 38.00 | 8.00 | 11.08 |
| 650 | 39.00 | 8.00 | 11.15 |
| 651 | 40.00 | 8.00 | 11.14 |
| 652 | 41.00 | 8.00 | 11.07 |
| 653 | 42.00 | 8.00 | 10.98 |
| 654 | 43.00 | 8.00 | 10.80 |
| 655 | 44.00 | 8.00 | 9.80 |
| 656 | 45.00 | 8.00 | 8.87 |
| 657 | 46.00 | 8.00 | 8.81 |
| 658 | 47.00 | 8.00 | 9.16 |
| 659 | 48.00 | 8.00 | 9.28 |
| 660 | 49.00 | 8.00 | 9.22 |
| 661 | 50.00 | 8.00 | 8.97 |
| 662 | 51.00 | 8.00 | 8.66 |
| 663 | 52.00 | 8.00 | 9.04 |
| 664 | 53.00 | 8.00 | 9.52 |
| 665 | 54.00 | 8.00 | 9.98 |
| 666 | 55.00 | 8.00 | 9.92 |
| 667 | 56.00 | 8.00 | 9.84 |
| 668 | 57.00 | 8.00 | 9.77 |
| 669 | 58.00 | 8.00 | 9.68 |
| 670 | 59.00 | 8.00 | 9.50 |
| 671 | 60.00 | 8.00 | 9.66 |
| 672 | 61.00 | 8.00 | 9.79 |
| 673 | 62.00 | 8.00 | 9.88 |
| 674 | 63.00 | 8.00 | 10.32 |
| 675 | 64.00 | 8.00 | 10.36 |
| 676 | 65.00 | 8.00 | 10.09 |
| 677 | 66.00 | 8.00 | 9.79 |
| 678 | 67.00 | 8.00 | 9.89 |
| 679 | 68.00 | 8.00 | 9.45 |
| 680 | 69.00 | 8.00 | 8.86 |
| 681 | 70.00 | 8.00 | 9.03 |
| 682 | 71.00 | 8.00 | 9.47 |
| 683 | 72.00 | 8.00 | 10.30 |
| 684 | 73.00 | 8.00 | 10.90 |
| 685 | 74.00 | 8.00 | 11.06 |
| 686 | 75.00 | 8.00 | 10.88 |
| 687 | 76.00 | 8.00 | 11.21 |
| 688 | 77.00 | 8.00 | 11.46 |
| 689 | 78.00 | 8.00 | 11.39 |
| 690 | 79.00 | 8.00 | 11.37 |
| 691 | 80.00 | 8.00 | 10.88 |
| 692 | 81.00 | 8.00 | 10.12 |
| 693 | 82.00 | 8.00 | 9.47 |
| 694 | 83.00 | 8.00 | 9.29 |
| 695 | 84.00 | 8.00 | 9.29 |
| 696 | 85.00 | 8.00 | 9.33 |
| 697 | 86.00 | 8.00 | 9.45 |
| 698 | 87.00 | 8.00 | 9.75 |
| 699 | 88.00 | 8.00 | 10.23 |
| 700 | 13.50 | 9.00 | 11.34 |
| 701 | 14.50 | 9.00 | 10.71 |
| 702 | 15.50 | 9.00 | 9.94 |
| 703 | 16.50 | 9.00 | 9.95 |
| 704 | 17.50 | 9.00 | 10.16 |
| 705 | 18.50 | 9.00 | 10.63 |
| 706 | 19.50 | 9.00 | 11.38 |
| 707 | 20.50 | 9.00 | 12.21 |
| 708 | 21.50 | 9.00 | 12.95 |
| 709 | 22.50 | 9.00 | 13.57 |
| 710 | 23.50 | 9.00 | 14.49 |
| 711 | 24.50 | 9.00 | 15.07 |
| 712 | 25.50 | 9.00 | 15.02 |
| 713 | 26.50 | 9.00 | 14.93 |
| 714 | 27.50 | 9.00 | 14.74 |
| 715 | 28.50 | 9.00 | 14.32 |
| 716 | 29.50 | 9.00 | 13.69 |
| 717 | 30.50 | 9.00 | 12.93 |
| 718 | 31.50 | 9.00 | 12.02 |
| 719 | 32.50 | 9.00 | 10.94 |
| 720 | 33.50 | 9.00 | 9.83 |
| 721 | 34.50 | 9.00 | 9.60 |
| 722 | 35.50 | 9.00 | 9.87 |
| 723 | 36.50 | 9.00 | 10.25 |
| 724 | 37.50 | 9.00 | 10.54 |
| 725 | 38.50 | 9.00 | 10.84 |
| 726 | 39.50 | 9.00 | 11.05 |
| 727 | 40.50 | 9.00 | 11.21 |
| 728 | 41.50 | 9.00 | 11.31 |
| 729 | 42.50 | 9.00 | 11.21 |
| 730 | 43.50 | 9.00 | 10.26 |
| 731 | 44.50 | 9.00 | 9.33 |
| 732 | 45.50 | 9.00 | 9.19 |
| 733 | 46.50 | 9.00 | 9.48 |
| 734 | 47.50 | 9.00 | 9.61 |
| 735 | 48.50 | 9.00 | 9.63 |
| 736 | 49.50 | 9.00 | 9.63 |
| 737 | 50.50 | 9.00 | 9.12 |
| 738 | 51.50 | 9.00 | 8.34 |
| 739 | 52.50 | 9.00 | 8.89 |
| 740 | 53.50 | 9.00 | 9.71 |
| 741 | 54.50 | 9.00 | 10.11 |
| 742 | 55.50 | 9.00 | 10.02 |
| 743 | 56.50 | 9.00 | 9.92 |
| 744 | 57.50 | 9.00 | 9.82 |
| 745 | 58.50 | 9.00 | 9.75 |
| 746 | 59.50 | 9.00 | 10.37 |
| 747 | 60.50 | 9.00 | 10.83 |
| 748 | 61.50 | 9.00 | 10.26 |
| 749 | 62.50 | 9.00 | 9.64 |
| 750 | 63.50 | 9.00 | 9.36 |
| 751 | 64.50 | 9.00 | 9.27 |
| 752 | 65.50 | 9.00 | 9.32 |
| 753 | 66.50 | 9.00 | 9.45 |
| 754 | 67.50 | 9.00 | 9.69 |
| 755 | 68.50 | 9.00 | 9.35 |
| 756 | 69.50 | 9.00 | 8.92 |
| 757 | 70.50 | 9.00 | 9.35 |
| 758 | 71.50 | 9.00 | 10.16 |
| 759 | 72.50 | 9.00 | 10.77 |
| 760 | 73.50 | 9.00 | 10.96 |
| 761 | 74.50 | 9.00 | 10.82 |
| 762 | 75.50 | 9.00 | 10.67 |
| 763 | 76.50 | 9.00 | 10.61 |
| 764 | 77.50 | 9.00 | 11.04 |
| 765 | 78.50 | 9.00 | 11.40 |
| 766 | 79.50 | 9.00 | 11.27 |
| 767 | 80.50 | 9.00 | 10.76 |
| 768 | 81.50 | 9.00 | 10.04 |
| 769 | 82.50 | 9.00 | 9.43 |
| 770 | 83.50 | 9.00 | 9.28 |
| 771 | 84.50 | 9.00 | 9.27 |
| 772 | 85.50 | 9.00 | 9.30 |
| 773 | 86.50 | 9.00 | 9.33 |
| 774 | 15.00 | 10.00 | 9.99 |
| 775 | 16.00 | 10.00 | 9.56 |
| 776 | 17.00 | 10.00 | 9.87 |
| 777 | 18.00 | 10.00 | 10.41 |
| 778 | 19.00 | 10.00 | 11.20 |
| 779 | 20.00 | 10.00 | 12.07 |
| 780 | 21.00 | 10.00 | 12.87 |
| 781 | 22.00 | 10.00 | 13.55 |
| 782 | 23.00 | 10.00 | 14.09 |
| 783 | 24.00 | 10.00 | 14.40 |
| 784 | 25.00 | 10.00 | 14.88 |
| 785 | 26.00 | 10.00 | 15.13 |
| 786 | 27.00 | 10.00 | 14.89 |
| 787 | 28.00 | 10.00 | 14.61 |
| 788 | 29.00 | 10.00 | 14.11 |
| 789 | 30.00 | 10.00 | 13.37 |
| 790 | 31.00 | 10.00 | 12.44 |
| 791 | 32.00 | 10.00 | 11.32 |
| 792 | 33.00 | 10.00 | 10.67 |
| 793 | 34.00 | 10.00 | 10.30 |
| 794 | 35.00 | 10.00 | 10.08 |
| 795 | 36.00 | 10.00 | 9.87 |
| 796 | 37.00 | 10.00 | 9.86 |
| 797 | 38.00 | 10.00 | 10.05 |
| 798 | 39.00 | 10.00 | 10.41 |
| 799 | 40.00 | 10.00 | 10.80 |
| 800 | 41.00 | 10.00 | 11.12 |
| 801 | 42.00 | 10.00 | 11.27 |
| 802 | 43.00 | 10.00 | 10.60 |
| 803 | 44.00 | 10.00 | 9.82 |
| 804 | 45.00 | 10.00 | 9.65 |
| 805 | 46.00 | 10.00 | 9.88 |
| 806 | 47.00 | 10.00 | 9.99 |
| 807 | 48.00 | 10.00 | 10.09 |
| 808 | 49.00 | 10.00 | 9.81 |
| 809 | 50.00 | 10.00 | 9.32 |
| 810 | 51.00 | 10.00 | 9.02 |
| 811 | 52.00 | 10.00 | 8.63 |
| 812 | 53.00 | 10.00 | 9.13 |
| 813 | 54.00 | 10.00 | 9.87 |
| 814 | 55.00 | 10.00 | 10.22 |
| 815 | 56.00 | 10.00 | 10.12 |
| 816 | 57.00 | 10.00 | 10.01 |
| 817 | 58.00 | 10.00 | 9.93 |
| 818 | 59.00 | 10.00 | 10.53 |
| 819 | 60.00 | 10.00 | 11.00 |
| 820 | 61.00 | 10.00 | 10.60 |
| 821 | 62.00 | 10.00 | 10.06 |
| 822 | 63.00 | 10.00 | 9.22 |
| 823 | 64.00 | 10.00 | 8.72 |
| 824 | 65.00 | 10.00 | 8.78 |
| 825 | 66.00 | 10.00 | 9.07 |
| 826 | 67.00 | 10.00 | 9.30 |
| 827 | 68.00 | 10.00 | 9.57 |
| 828 | 69.00 | 10.00 | 9.34 |
| 829 | 70.00 | 10.00 | 9.21 |
| 830 | 71.00 | 10.00 | 9.99 |
| 831 | 72.00 | 10.00 | 10.61 |
| 832 | 73.00 | 10.00 | 10.83 |
| 833 | 74.00 | 10.00 | 10.72 |
| 834 | 75.00 | 10.00 | 10.61 |
| 835 | 76.00 | 10.00 | 10.54 |
| 836 | 77.00 | 10.00 | 10.52 |
| 837 | 78.00 | 10.00 | 10.56 |
| 838 | 79.00 | 10.00 | 11.04 |
| 839 | 80.00 | 10.00 | 11.30 |
| 840 | 81.00 | 10.00 | 10.70 |
| 841 | 82.00 | 10.00 | 9.99 |
| 842 | 83.00 | 10.00 | 9.43 |
| 843 | 84.00 | 10.00 | 9.29 |
| 844 | 85.00 | 10.00 | 9.27 |
| 845 | 16.50 | 11.00 | 9.62 |
| 846 | 17.50 | 11.00 | 10.20 |
| 847 | 18.50 | 11.00 | 11.03 |
| 848 | 19.50 | 11.00 | 11.92 |
| 849 | 20.50 | 11.00 | 12.74 |
| 850 | 21.50 | 11.00 | 13.46 |
| 851 | 22.50 | 11.00 | 14.04 |
| 852 | 23.50 | 11.00 | 14.40 |
| 853 | 24.50 | 11.00 | 14.50 |
| 854 | 25.50 | 11.00 | 14.45 |
| 855 | 26.50 | 11.00 | 14.73 |
| 856 | 27.50 | 11.00 | 14.83 |
| 857 | 28.50 | 11.00 | 14.37 |
| 858 | 29.50 | 11.00 | 13.69 |
| 859 | 30.50 | 11.00 | 12.78 |
| 860 | 31.50 | 11.00 | 12.28 |
| 861 | 32.50 | 11.00 | 11.85 |
| 862 | 33.50 | 11.00 | 11.19 |
| 863 | 34.50 | 11.00 | 10.42 |
| 864 | 35.50 | 11.00 | 9.92 |
| 865 | 36.50 | 11.00 | 9.67 |
| 866 | 37.50 | 11.00 | 9.61 |
| 867 | 38.50 | 11.00 | 9.70 |
| 868 | 39.50 | 11.00 | 9.98 |
| 869 | 40.50 | 11.00 | 10.41 |
| 870 | 41.50 | 11.00 | 10.79 |
| 871 | 42.50 | 11.00 | 10.41 |
| 872 | 43.50 | 11.00 | 9.94 |
| 873 | 44.50 | 11.00 | 9.98 |
| 874 | 45.50 | 11.00 | 10.30 |
| 875 | 46.50 | 11.00 | 10.48 |
| 876 | 47.50 | 11.00 | 10.28 |
| 877 | 48.50 | 11.00 | 9.91 |
| 878 | 49.50 | 11.00 | 9.78 |
| 879 | 50.50 | 11.00 | 9.58 |
| 880 | 51.50 | 11.00 | 9.26 |
| 881 | 52.50 | 11.00 | 8.88 |
| 882 | 53.50 | 11.00 | 9.34 |
| 883 | 54.50 | 11.00 | 10.03 |
| 884 | 55.50 | 11.00 | 10.35 |
| 885 | 56.50 | 11.00 | 10.26 |
| 886 | 57.50 | 11.00 | 10.18 |
| 887 | 58.50 | 11.00 | 10.67 |
| 888 | 59.50 | 11.00 | 10.80 |
| 889 | 60.50 | 11.00 | 10.22 |
| 890 | 61.50 | 11.00 | 9.78 |
| 891 | 62.50 | 11.00 | 9.61 |
| 892 | 63.50 | 11.00 | 9.50 |
| 893 | 64.50 | 11.00 | 9.08 |
| 894 | 65.50 | 11.00 | 8.74 |
| 895 | 66.50 | 11.00 | 8.82 |
| 896 | 67.50 | 11.00 | 9.13 |
| 897 | 68.50 | 11.00 | 9.48 |
| 898 | 69.50 | 11.00 | 9.53 |
| 899 | 70.50 | 11.00 | 9.80 |
| 900 | 71.50 | 11.00 | 10.43 |
| 901 | 72.50 | 11.00 | 10.69 |
| 902 | 73.50 | 11.00 | 10.61 |
| 903 | 74.50 | 11.00 | 10.51 |
| 904 | 75.50 | 11.00 | 10.46 |
| 905 | 76.50 | 11.00 | 10.44 |
| 906 | 77.50 | 11.00 | 10.47 |
| 907 | 78.50 | 11.00 | 10.53 |
| 908 | 79.50 | 11.00 | 10.61 |
| 909 | 80.50 | 11.00 | 10.96 |
| 910 | 81.50 | 11.00 | 10.74 |
| 911 | 82.50 | 11.00 | 9.98 |
| 912 | 83.50 | 11.00 | 9.43 |
| 913 | 18.00 | 12.00 | 10.84 |
| 914 | 19.00 | 12.00 | 11.77 |
| 915 | 20.00 | 12.00 | 12.61 |
| 916 | 21.00 | 12.00 | 13.34 |
| 917 | 22.00 | 12.00 | 13.93 |
| 918 | 23.00 | 12.00 | 14.31 |
| 919 | 24.00 | 12.00 | 14.44 |
| 920 | 25.00 | 12.00 | 14.42 |
| 921 | 26.00 | 12.00 | 14.31 |
| 922 | 27.00 | 12.00 | 14.11 |
| 923 | 28.00 | 12.00 | 14.16 |
| 924 | 29.00 | 12.00 | 13.95 |
| 925 | 30.00 | 12.00 | 13.66 |
| 926 | 31.00 | 12.00 | 13.37 |
| 927 | 32.00 | 12.00 | 12.82 |
| 928 | 33.00 | 12.00 | 11.97 |
| 929 | 34.00 | 12.00 | 11.08 |
| 930 | 35.00 | 12.00 | 10.30 |
| 931 | 36.00 | 12.00 | 9.81 |
| 932 | 37.00 | 12.00 | 9.55 |
| 933 | 38.00 | 12.00 | 9.46 |
| 934 | 39.00 | 12.00 | 9.50 |
| 935 | 40.00 | 12.00 | 9.69 |
| 936 | 41.00 | 12.00 | 9.95 |
| 937 | 42.00 | 12.00 | 9.75 |
| 938 | 43.00 | 12.00 | 9.54 |
| 939 | 44.00 | 12.00 | 9.82 |
| 940 | 45.00 | 12.00 | 10.40 |
| 941 | 46.00 | 12.00 | 10.54 |
| 942 | 47.00 | 12.00 | 10.39 |
| 943 | 48.00 | 12.00 | 10.34 |
| 944 | 49.00 | 12.00 | 10.22 |
| 945 | 50.00 | 12.00 | 10.04 |
| 946 | 51.00 | 12.00 | 9.80 |
| 947 | 52.00 | 12.00 | 9.48 |
| 948 | 53.00 | 12.00 | 9.12 |
| 949 | 54.00 | 12.00 | 9.57 |
| 950 | 55.00 | 12.00 | 10.24 |
| 951 | 56.00 | 12.00 | 10.56 |
| 952 | 57.00 | 12.00 | 10.41 |
| 953 | 58.00 | 12.00 | 10.57 |
| 954 | 59.00 | 12.00 | 10.47 |
| 955 | 60.00 | 12.00 | 9.93 |
| 956 | 61.00 | 12.00 | 9.72 |
| 957 | 62.00 | 12.00 | 9.63 |
| 958 | 63.00 | 12.00 | 9.59 |
| 959 | 64.00 | 12.00 | 9.57 |
| 960 | 65.00 | 12.00 | 9.49 |
| 961 | 66.00 | 12.00 | 9.11 |
| 962 | 67.00 | 12.00 | 8.80 |
| 963 | 68.00 | 12.00 | 8.94 |
| 964 | 69.00 | 12.00 | 9.55 |
| 965 | 70.00 | 12.00 | 10.00 |
| 966 | 71.00 | 12.00 | 10.20 |
| 967 | 72.00 | 12.00 | 10.51 |
| 968 | 73.00 | 12.00 | 10.48 |
| 969 | 74.00 | 12.00 | 10.41 |
| 970 | 75.00 | 12.00 | 10.37 |
| 971 | 76.00 | 12.00 | 10.35 |
| 972 | 77.00 | 12.00 | 10.37 |
| 973 | 78.00 | 12.00 | 10.41 |
| 974 | 79.00 | 12.00 | 10.49 |
| 975 | 80.00 | 12.00 | 10.58 |
| 976 | 81.00 | 12.00 | 10.55 |
| 977 | 82.00 | 12.00 | 10.46 |
| 978 | 19.50 | 13.00 | 12.45 |
| 979 | 20.50 | 13.00 | 13.20 |
| 980 | 21.50 | 13.00 | 13.80 |
| 981 | 22.50 | 13.00 | 14.18 |
| 982 | 23.50 | 13.00 | 14.30 |
| 983 | 24.50 | 13.00 | 14.28 |
| 984 | 25.50 | 13.00 | 14.16 |
| 985 | 26.50 | 13.00 | 13.94 |
| 986 | 27.50 | 13.00 | 13.64 |
| 987 | 28.50 | 13.00 | 13.78 |
| 988 | 29.50 | 13.00 | 14.31 |
| 989 | 30.50 | 13.00 | 14.33 |
| 990 | 31.50 | 13.00 | 13.55 |
| 991 | 32.50 | 13.00 | 12.73 |
| 992 | 33.50 | 13.00 | 11.87 |
| 993 | 34.50 | 13.00 | 10.98 |
| 994 | 35.50 | 13.00 | 10.23 |
| 995 | 36.50 | 13.00 | 9.74 |
| 996 | 37.50 | 13.00 | 9.48 |
| 997 | 38.50 | 13.00 | 9.39 |
| 998 | 39.50 | 13.00 | 9.41 |
| 999 | 40.50 | 13.00 | 9.47 |
| 1000 | 41.50 | 13.00 | 9.09 |
| 1001 | 42.50 | 13.00 | 8.85 |
| 1002 | 43.50 | 13.00 | 9.25 |
| 1003 | 44.50 | 13.00 | 9.79 |
| 1004 | 45.50 | 13.00 | 10.06 |
| 1005 | 46.50 | 13.00 | 10.39 |
| 1006 | 47.50 | 13.00 | 10.56 |
| 1007 | 48.50 | 13.00 | 10.58 |
| 1008 | 49.50 | 13.00 | 10.46 |
| 1009 | 50.50 | 13.00 | 10.28 |
| 1010 | 51.50 | 13.00 | 10.04 |
| 1011 | 52.50 | 13.00 | 9.73 |
| 1012 | 53.50 | 13.00 | 9.42 |
| 1013 | 54.50 | 13.00 | 9.88 |
| 1014 | 55.50 | 13.00 | 10.45 |
| 1015 | 56.50 | 13.00 | 10.44 |
| 1016 | 57.50 | 13.00 | 10.38 |
| 1017 | 58.50 | 13.00 | 10.25 |
| 1018 | 59.50 | 13.00 | 9.94 |
| 1019 | 60.50 | 13.00 | 9.79 |
| 1020 | 61.50 | 13.00 | 9.71 |
| 1021 | 62.50 | 13.00 | 9.66 |
| 1022 | 63.50 | 13.00 | 9.62 |
| 1023 | 64.50 | 13.00 | 9.58 |
| 1024 | 65.50 | 13.00 | 9.56 |
| 1025 | 66.50 | 13.00 | 9.49 |
| 1026 | 67.50 | 13.00 | 9.19 |
| 1027 | 68.50 | 13.00 | 9.15 |
| 1028 | 69.50 | 13.00 | 9.85 |
| 1029 | 70.50 | 13.00 | 10.28 |
| 1030 | 71.50 | 13.00 | 10.25 |
| 1031 | 72.50 | 13.00 | 10.30 |
| 1032 | 73.50 | 13.00 | 10.29 |
| 1033 | 74.50 | 13.00 | 10.27 |
| 1034 | 75.50 | 13.00 | 10.26 |
| 1035 | 76.50 | 13.00 | 10.27 |
| 1036 | 77.50 | 13.00 | 10.30 |
| 1037 | 78.50 | 13.00 | 10.36 |
| 1038 | 79.50 | 13.00 | 10.45 |
| 1039 | 80.50 | 13.00 | 10.55 |
| 1040 | 21.00 | 14.00 | 13.64 |
| 1041 | 22.00 | 14.00 | 14.01 |
| 1042 | 23.00 | 14.00 | 14.13 |
| 1043 | 24.00 | 14.00 | 14.07 |
| 1044 | 25.00 | 14.00 | 13.90 |
| 1045 | 26.00 | 14.00 | 13.67 |
| 1046 | 27.00 | 14.00 | 13.93 |
| 1047 | 28.00 | 14.00 | 14.29 |
| 1048 | 29.00 | 14.00 | 14.39 |
| 1049 | 30.00 | 14.00 | 14.13 |
| 1050 | 31.00 | 14.00 | 13.99 |
| 1051 | 32.00 | 14.00 | 13.56 |
| 1052 | 33.00 | 14.00 | 12.68 |
| 1053 | 34.00 | 14.00 | 11.81 |
| 1054 | 35.00 | 14.00 | 10.93 |
| 1055 | 36.00 | 14.00 | 10.19 |
| 1056 | 37.00 | 14.00 | 9.71 |
| 1057 | 38.00 | 14.00 | 9.47 |
| 1058 | 39.00 | 14.00 | 9.38 |
| 1059 | 40.00 | 14.00 | 9.33 |
| 1060 | 41.00 | 14.00 | 8.87 |
| 1061 | 42.00 | 14.00 | 8.51 |
| 1062 | 43.00 | 14.00 | 8.52 |
| 1063 | 44.00 | 14.00 | 8.86 |
| 1064 | 45.00 | 14.00 | 9.42 |
| 1065 | 46.00 | 14.00 | 9.91 |
| 1066 | 47.00 | 14.00 | 10.28 |
| 1067 | 48.00 | 14.00 | 10.53 |
| 1068 | 49.00 | 14.00 | 10.66 |
| 1069 | 50.00 | 14.00 | 10.65 |
| 1070 | 51.00 | 14.00 | 10.53 |
| 1071 | 52.00 | 14.00 | 10.32 |
| 1072 | 53.00 | 14.00 | 10.05 |
| 1073 | 54.00 | 14.00 | 9.71 |
| 1074 | 55.00 | 14.00 | 9.86 |
| 1075 | 56.00 | 14.00 | 10.02 |
| 1076 | 57.00 | 14.00 | 10.29 |
| 1077 | 58.00 | 14.00 | 10.33 |
| 1078 | 59.00 | 14.00 | 10.08 |
| 1079 | 60.00 | 14.00 | 9.93 |
| 1080 | 61.00 | 14.00 | 9.83 |
| 1081 | 62.00 | 14.00 | 9.76 |
| 1082 | 63.00 | 14.00 | 9.70 |
| 1083 | 64.00 | 14.00 | 9.64 |
| 1084 | 65.00 | 14.00 | 9.59 |
| 1085 | 66.00 | 14.00 | 9.56 |
| 1086 | 67.00 | 14.00 | 9.58 |
| 1087 | 68.00 | 14.00 | 9.75 |
| 1088 | 69.00 | 14.00 | 9.99 |
| 1089 | 70.00 | 14.00 | 10.24 |
| 1090 | 71.00 | 14.00 | 10.20 |
| 1091 | 72.00 | 14.00 | 10.00 |
| 1092 | 73.00 | 14.00 | 10.09 |
| 1093 | 74.00 | 14.00 | 10.13 |
| 1094 | 75.00 | 14.00 | 10.15 |
| 1095 | 76.00 | 14.00 | 10.17 |
| 1096 | 77.00 | 14.00 | 10.20 |
| 1097 | 78.00 | 14.00 | 10.25 |
| 1098 | 79.00 | 14.00 | 10.31 |
| 1099 | 22.50 | 15.00 | 13.90 |
| 1100 | 23.50 | 15.00 | 13.80 |
| 1101 | 24.50 | 15.00 | 13.61 |
| 1102 | 25.50 | 15.00 | 13.88 |
| 1103 | 26.50 | 15.00 | 14.27 |
| 1104 | 27.50 | 15.00 | 14.47 |
| 1105 | 28.50 | 15.00 | 14.43 |
| 1106 | 29.50 | 15.00 | 14.32 |
| 1107 | 30.50 | 15.00 | 14.08 |
| 1108 | 31.50 | 15.00 | 13.60 |
| 1109 | 32.50 | 15.00 | 13.26 |
| 1110 | 33.50 | 15.00 | 12.71 |
| 1111 | 34.50 | 15.00 | 11.79 |
| 1112 | 35.50 | 15.00 | 10.91 |
| 1113 | 36.50 | 15.00 | 10.18 |
| 1114 | 37.50 | 15.00 | 9.73 |
| 1115 | 38.50 | 15.00 | 9.51 |
| 1116 | 39.50 | 15.00 | 9.37 |
| 1117 | 40.50 | 15.00 | 8.89 |
| 1118 | 41.50 | 15.00 | 8.29 |
| 1119 | 42.50 | 15.00 | 8.13 |
| 1120 | 43.50 | 15.00 | 8.47 |
| 1121 | 44.50 | 15.00 | 8.81 |
| 1122 | 45.50 | 15.00 | 9.18 |
| 1123 | 46.50 | 15.00 | 9.61 |
| 1124 | 47.50 | 15.00 | 10.04 |
| 1125 | 48.50 | 15.00 | 10.37 |
| 1126 | 49.50 | 15.00 | 10.62 |
| 1127 | 50.50 | 15.00 | 10.76 |
| 1128 | 51.50 | 15.00 | 10.77 |
| 1129 | 52.50 | 15.00 | 10.58 |
| 1130 | 53.50 | 15.00 | 10.07 |
| 1131 | 54.50 | 15.00 | 9.41 |
| 1132 | 55.50 | 15.00 | 9.43 |
| 1133 | 56.50 | 15.00 | 10.14 |
| 1134 | 57.50 | 15.00 | 10.55 |
| 1135 | 58.50 | 15.00 | 10.29 |
| 1136 | 59.50 | 15.00 | 10.11 |
| 1137 | 60.50 | 15.00 | 9.99 |
| 1138 | 61.50 | 15.00 | 9.89 |
| 1139 | 62.50 | 15.00 | 9.81 |
| 1140 | 63.50 | 15.00 | 9.73 |
| 1141 | 64.50 | 15.00 | 9.66 |
| 1142 | 65.50 | 15.00 | 9.61 |
| 1143 | 66.50 | 15.00 | 9.60 |
| 1144 | 67.50 | 15.00 | 9.78 |
| 1145 | 68.50 | 15.00 | 10.28 |
| 1146 | 69.50 | 15.00 | 10.71 |
| 1147 | 70.50 | 15.00 | 10.68 |
| 1148 | 71.50 | 15.00 | 10.07 |
| 1149 | 72.50 | 15.00 | 9.72 |
| 1150 | 73.50 | 15.00 | 9.88 |
| 1151 | 74.50 | 15.00 | 9.99 |
| 1152 | 75.50 | 15.00 | 10.05 |
| 1153 | 76.50 | 15.00 | 10.10 |
| 1154 | 77.50 | 15.00 | 10.14 |
| 1155 | 24.00 | 16.00 | 13.80 |
| 1156 | 25.00 | 16.00 | 14.19 |
| 1157 | 26.00 | 16.00 | 14.39 |
| 1158 | 27.00 | 16.00 | 14.38 |
| 1159 | 28.00 | 16.00 | 14.36 |
| 1160 | 29.00 | 16.00 | 14.33 |
| 1161 | 30.00 | 16.00 | 14.25 |
| 1162 | 31.00 | 16.00 | 14.04 |
| 1163 | 32.00 | 16.00 | 13.59 |
| 1164 | 33.00 | 16.00 | 12.93 |
| 1165 | 34.00 | 16.00 | 12.46 |
| 1166 | 35.00 | 16.00 | 11.84 |
| 1167 | 36.00 | 16.00 | 10.93 |
| 1168 | 37.00 | 16.00 | 10.22 |
| 1169 | 38.00 | 16.00 | 9.81 |
| 1170 | 39.00 | 16.00 | 9.58 |
| 1171 | 40.00 | 16.00 | 8.85 |
| 1172 | 41.00 | 16.00 | 8.18 |
| 1173 | 42.00 | 16.00 | 8.12 |
| 1174 | 43.00 | 16.00 | 8.36 |
| 1175 | 44.00 | 16.00 | 8.57 |
| 1176 | 45.00 | 16.00 | 8.77 |
| 1177 | 46.00 | 16.00 | 9.00 |
| 1178 | 47.00 | 16.00 | 9.32 |
| 1179 | 48.00 | 16.00 | 9.73 |
| 1180 | 49.00 | 16.00 | 10.16 |
| 1181 | 50.00 | 16.00 | 10.52 |
| 1182 | 51.00 | 16.00 | 10.72 |
| 1183 | 52.00 | 16.00 | 10.62 |
| 1184 | 53.00 | 16.00 | 10.23 |
| 1185 | 54.00 | 16.00 | 9.69 |
| 1186 | 55.00 | 16.00 | 9.23 |
| 1187 | 56.00 | 16.00 | 9.81 |
| 1188 | 57.00 | 16.00 | 10.50 |
| 1189 | 58.00 | 16.00 | 10.55 |
| 1190 | 59.00 | 16.00 | 10.35 |
| 1191 | 60.00 | 16.00 | 10.18 |
| 1192 | 61.00 | 16.00 | 10.06 |
| 1193 | 62.00 | 16.00 | 9.95 |
| 1194 | 63.00 | 16.00 | 9.85 |
| 1195 | 64.00 | 16.00 | 9.77 |
| 1196 | 65.00 | 16.00 | 9.69 |
| 1197 | 66.00 | 16.00 | 9.66 |
| 1198 | 67.00 | 16.00 | 9.80 |
| 1199 | 68.00 | 16.00 | 10.24 |
| 1200 | 69.00 | 16.00 | 10.66 |
| 1201 | 70.00 | 16.00 | 10.88 |
| 1202 | 71.00 | 16.00 | 10.88 |
| 1203 | 72.00 | 16.00 | 10.25 |
| 1204 | 73.00 | 16.00 | 9.65 |
| 1205 | 74.00 | 16.00 | 9.67 |
| 1206 | 75.00 | 16.00 | 9.84 |
| 1207 | 76.00 | 16.00 | 9.94 |
| 1208 | 25.50 | 17.00 | 14.28 |
| 1209 | 26.50 | 17.00 | 14.27 |
| 1210 | 27.50 | 17.00 | 14.26 |
| 1211 | 28.50 | 17.00 | 14.27 |
| 1212 | 29.50 | 17.00 | 14.25 |
| 1213 | 30.50 | 17.00 | 14.18 |
| 1214 | 31.50 | 17.00 | 13.98 |
| 1215 | 32.50 | 17.00 | 13.56 |
| 1216 | 33.50 | 17.00 | 12.93 |
| 1217 | 34.50 | 17.00 | 12.19 |
| 1218 | 35.50 | 17.00 | 11.66 |
| 1219 | 36.50 | 17.00 | 11.05 |
| 1220 | 37.50 | 17.00 | 10.37 |
| 1221 | 38.50 | 17.00 | 9.75 |
| 1222 | 39.50 | 17.00 | 8.89 |
| 1223 | 40.50 | 17.00 | 8.33 |
| 1224 | 41.50 | 17.00 | 8.20 |
| 1225 | 42.50 | 17.00 | 8.36 |
| 1226 | 43.50 | 17.00 | 8.50 |
| 1227 | 44.50 | 17.00 | 8.62 |
| 1228 | 45.50 | 17.00 | 8.76 |
| 1229 | 46.50 | 17.00 | 8.93 |
| 1230 | 47.50 | 17.00 | 9.17 |
| 1231 | 48.50 | 17.00 | 9.50 |
| 1232 | 49.50 | 17.00 | 9.87 |
| 1233 | 50.50 | 17.00 | 10.06 |
| 1234 | 51.50 | 17.00 | 10.06 |
| 1235 | 52.50 | 17.00 | 9.99 |
| 1236 | 53.50 | 17.00 | 9.91 |
| 1237 | 54.50 | 17.00 | 9.67 |
| 1238 | 55.50 | 17.00 | 9.66 |
| 1239 | 56.50 | 17.00 | 10.20 |
| 1240 | 57.50 | 17.00 | 10.54 |
| 1241 | 58.50 | 17.00 | 10.62 |
| 1242 | 59.50 | 17.00 | 10.43 |
| 1243 | 60.50 | 17.00 | 10.26 |
| 1244 | 61.50 | 17.00 | 10.12 |
| 1245 | 62.50 | 17.00 | 10.00 |
| 1246 | 63.50 | 17.00 | 9.90 |
| 1247 | 64.50 | 17.00 | 9.80 |
| 1248 | 65.50 | 17.00 | 9.75 |
| 1249 | 66.50 | 17.00 | 9.86 |
| 1250 | 67.50 | 17.00 | 10.26 |
| 1251 | 68.50 | 17.00 | 10.63 |
| 1252 | 69.50 | 17.00 | 10.80 |
| 1253 | 70.50 | 17.00 | 10.77 |
| 1254 | 71.50 | 17.00 | 10.78 |
| 1255 | 72.50 | 17.00 | 10.45 |
| 1256 | 73.50 | 17.00 | 9.86 |
| 1257 | 74.50 | 17.00 | 9.64 |
| 1258 | 27.00 | 18.00 | 14.16 |
| 1259 | 28.00 | 18.00 | 14.18 |
| 1260 | 29.00 | 18.00 | 14.19 |
| 1261 | 30.00 | 18.00 | 14.18 |
| 1262 | 31.00 | 18.00 | 14.12 |
| 1263 | 32.00 | 18.00 | 13.94 |
| 1264 | 33.00 | 18.00 | 13.53 |
| 1265 | 34.00 | 18.00 | 12.94 |
| 1266 | 35.00 | 18.00 | 12.25 |
| 1267 | 36.00 | 18.00 | 11.54 |
| 1268 | 37.00 | 18.00 | 10.87 |
| 1269 | 38.00 | 18.00 | 10.15 |
| 1270 | 39.00 | 18.00 | 9.27 |
| 1271 | 40.00 | 18.00 | 8.58 |
| 1272 | 41.00 | 18.00 | 8.35 |
| 1273 | 42.00 | 18.00 | 8.44 |
| 1274 | 43.00 | 18.00 | 8.52 |
| 1275 | 44.00 | 18.00 | 8.60 |
| 1276 | 45.00 | 18.00 | 8.69 |
| 1277 | 46.00 | 18.00 | 8.81 |
| 1278 | 47.00 | 18.00 | 8.96 |
| 1279 | 48.00 | 18.00 | 9.10 |
| 1280 | 49.00 | 18.00 | 9.11 |
| 1281 | 50.00 | 18.00 | 9.11 |
| 1282 | 51.00 | 18.00 | 9.23 |
| 1283 | 52.00 | 18.00 | 9.48 |
| 1284 | 53.00 | 18.00 | 9.67 |
| 1285 | 54.00 | 18.00 | 9.72 |
| 1286 | 55.00 | 18.00 | 9.92 |
| 1287 | 56.00 | 18.00 | 9.98 |
| 1288 | 57.00 | 18.00 | 10.23 |
| 1289 | 58.00 | 18.00 | 10.62 |
| 1290 | 59.00 | 18.00 | 10.71 |
| 1291 | 60.00 | 18.00 | 10.52 |
| 1292 | 61.00 | 18.00 | 10.34 |
| 1293 | 62.00 | 18.00 | 10.19 |
| 1294 | 63.00 | 18.00 | 10.06 |
| 1295 | 64.00 | 18.00 | 9.94 |
| 1296 | 65.00 | 18.00 | 9.87 |
| 1297 | 66.00 | 18.00 | 9.95 |
| 1298 | 67.00 | 18.00 | 10.31 |
| 1299 | 68.00 | 18.00 | 10.65 |
| 1300 | 69.00 | 18.00 | 10.78 |
| 1301 | 70.00 | 18.00 | 10.72 |
| 1302 | 71.00 | 18.00 | 10.66 |
| 1303 | 72.00 | 18.00 | 10.68 |
| 1304 | 73.00 | 18.00 | 10.41 |
| 1305 | 28.50 | 19.00 | 14.10 |
| 1306 | 29.50 | 19.00 | 14.12 |
| 1307 | 30.50 | 19.00 | 14.13 |
| 1308 | 31.50 | 19.00 | 14.08 |
| 1309 | 32.50 | 19.00 | 13.90 |
| 1310 | 33.50 | 19.00 | 13.53 |
| 1311 | 34.50 | 19.00 | 13.01 |
| 1312 | 35.50 | 19.00 | 12.21 |
| 1313 | 36.50 | 19.00 | 11.33 |
| 1314 | 37.50 | 19.00 | 10.55 |
| 1315 | 38.50 | 19.00 | 9.71 |
| 1316 | 39.50 | 19.00 | 9.00 |
| 1317 | 40.50 | 19.00 | 8.61 |
| 1318 | 41.50 | 19.00 | 8.60 |
| 1319 | 42.50 | 19.00 | 8.62 |
| 1320 | 43.50 | 19.00 | 8.66 |
| 1321 | 44.50 | 19.00 | 8.72 |
| 1322 | 45.50 | 19.00 | 8.81 |
| 1323 | 46.50 | 19.00 | 8.86 |
| 1324 | 47.50 | 19.00 | 8.77 |
| 1325 | 48.50 | 19.00 | 8.62 |
| 1326 | 49.50 | 19.00 | 8.54 |
| 1327 | 50.50 | 19.00 | 8.63 |
| 1328 | 51.50 | 19.00 | 8.83 |
| 1329 | 52.50 | 19.00 | 9.10 |
| 1330 | 53.50 | 19.00 | 9.33 |
| 1331 | 54.50 | 19.00 | 9.74 |
| 1332 | 55.50 | 19.00 | 9.99 |
| 1333 | 56.50 | 19.00 | 9.86 |
| 1334 | 57.50 | 19.00 | 10.24 |
| 1335 | 58.50 | 19.00 | 10.68 |
| 1336 | 59.50 | 19.00 | 10.80 |
| 1337 | 60.50 | 19.00 | 10.61 |
| 1338 | 61.50 | 19.00 | 10.42 |
| 1339 | 62.50 | 19.00 | 10.26 |
| 1340 | 63.50 | 19.00 | 10.12 |
| 1341 | 64.50 | 19.00 | 10.02 |
| 1342 | 65.50 | 19.00 | 10.08 |
| 1343 | 66.50 | 19.00 | 10.40 |
| 1344 | 67.50 | 19.00 | 10.70 |
| 1345 | 68.50 | 19.00 | 10.81 |
| 1346 | 69.50 | 19.00 | 10.73 |
| 1347 | 70.50 | 19.00 | 10.63 |
| 1348 | 71.50 | 19.00 | 10.58 |
| 1349 | 30.00 | 20.00 | 14.07 |
| 1350 | 31.00 | 20.00 | 14.09 |
| 1351 | 32.00 | 20.00 | 14.07 |
| 1352 | 33.00 | 20.00 | 13.95 |
| 1353 | 34.00 | 20.00 | 13.44 |
| 1354 | 35.00 | 20.00 | 12.74 |
| 1355 | 36.00 | 20.00 | 12.12 |
| 1356 | 37.00 | 20.00 | 11.38 |
| 1357 | 38.00 | 20.00 | 10.26 |
| 1358 | 39.00 | 20.00 | 9.23 |
| 1359 | 40.00 | 20.00 | 8.89 |
| 1360 | 41.00 | 20.00 | 8.89 |
| 1361 | 42.00 | 20.00 | 8.81 |
| 1362 | 43.00 | 20.00 | 8.79 |
| 1363 | 44.00 | 20.00 | 8.81 |
| 1364 | 45.00 | 20.00 | 8.82 |
| 1365 | 46.00 | 20.00 | 8.69 |
| 1366 | 47.00 | 20.00 | 8.49 |
| 1367 | 48.00 | 20.00 | 8.35 |
| 1368 | 49.00 | 20.00 | 8.34 |
| 1369 | 50.00 | 20.00 | 8.40 |
| 1370 | 51.00 | 20.00 | 8.48 |
| 1371 | 52.00 | 20.00 | 8.59 |
| 1372 | 53.00 | 20.00 | 8.73 |
| 1373 | 54.00 | 20.00 | 9.21 |
| 1374 | 55.00 | 20.00 | 9.63 |
| 1375 | 56.00 | 20.00 | 9.68 |
| 1376 | 57.00 | 20.00 | 9.68 |
| 1377 | 58.00 | 20.00 | 10.16 |
| 1378 | 59.00 | 20.00 | 10.69 |
| 1379 | 60.00 | 20.00 | 10.85 |
| 1380 | 61.00 | 20.00 | 10.68 |
| 1381 | 62.00 | 20.00 | 10.50 |
| 1382 | 63.00 | 20.00 | 10.33 |
| 1383 | 64.00 | 20.00 | 10.21 |
| 1384 | 65.00 | 20.00 | 10.23 |
| 1385 | 66.00 | 20.00 | 10.51 |
| 1386 | 67.00 | 20.00 | 10.79 |
| 1387 | 68.00 | 20.00 | 10.88 |
| 1388 | 69.00 | 20.00 | 10.78 |
| 1389 | 70.00 | 20.00 | 10.66 |
| 1390 | 31.50 | 21.00 | 14.13 |
| 1391 | 32.50 | 21.00 | 13.96 |
| 1392 | 33.50 | 21.00 | 13.65 |
| 1393 | 34.50 | 21.00 | 13.31 |
| 1394 | 35.50 | 21.00 | 12.79 |
| 1395 | 36.50 | 21.00 | 12.13 |
| 1396 | 37.50 | 21.00 | 11.06 |
| 1397 | 38.50 | 21.00 | 9.97 |
| 1398 | 39.50 | 21.00 | 9.27 |
| 1399 | 40.50 | 21.00 | 8.99 |
| 1400 | 41.50 | 21.00 | 8.99 |
| 1401 | 42.50 | 21.00 | 9.03 |
| 1402 | 43.50 | 21.00 | 8.93 |
| 1403 | 44.50 | 21.00 | 8.73 |
| 1404 | 45.50 | 21.00 | 8.50 |
| 1405 | 46.50 | 21.00 | 8.33 |
| 1406 | 47.50 | 21.00 | 8.28 |
| 1407 | 48.50 | 21.00 | 8.29 |
| 1408 | 49.50 | 21.00 | 8.31 |
| 1409 | 50.50 | 21.00 | 8.34 |
| 1410 | 51.50 | 21.00 | 8.37 |
| 1411 | 52.50 | 21.00 | 8.40 |
| 1412 | 53.50 | 21.00 | 8.72 |
| 1413 | 54.50 | 21.00 | 9.06 |
| 1414 | 55.50 | 21.00 | 9.20 |
| 1415 | 56.50 | 21.00 | 9.34 |
| 1416 | 57.50 | 21.00 | 9.44 |
| 1417 | 58.50 | 21.00 | 9.97 |
| 1418 | 59.50 | 21.00 | 10.58 |
| 1419 | 60.50 | 21.00 | 10.84 |
| 1420 | 61.50 | 21.00 | 10.72 |
| 1421 | 62.50 | 21.00 | 10.56 |
| 1422 | 63.50 | 21.00 | 10.42 |
| 1423 | 64.50 | 21.00 | 10.42 |
| 1424 | 65.50 | 21.00 | 10.66 |
| 1425 | 66.50 | 21.00 | 10.91 |
| 1426 | 67.50 | 21.00 | 10.98 |
| 1427 | 68.50 | 21.00 | 10.86 |
| 1428 | 33.00 | 22.00 | 13.82 |
| 1429 | 34.00 | 22.00 | 13.68 |
| 1430 | 35.00 | 22.00 | 13.34 |
| 1431 | 36.00 | 22.00 | 12.78 |
| 1432 | 37.00 | 22.00 | 11.80 |
| 1433 | 38.00 | 22.00 | 10.75 |
| 1434 | 39.00 | 22.00 | 9.98 |
| 1435 | 40.00 | 22.00 | 9.52 |
| 1436 | 41.00 | 22.00 | 9.23 |
| 1437 | 42.00 | 22.00 | 9.01 |
| 1438 | 43.00 | 22.00 | 8.87 |
| 1439 | 44.00 | 22.00 | 8.68 |
| 1440 | 45.00 | 22.00 | 8.43 |
| 1441 | 46.00 | 22.00 | 8.34 |
| 1442 | 47.00 | 22.00 | 8.31 |
| 1443 | 48.00 | 22.00 | 8.30 |
| 1444 | 49.00 | 22.00 | 8.30 |
| 1445 | 50.00 | 22.00 | 8.30 |
| 1446 | 51.00 | 22.00 | 8.29 |
| 1447 | 52.00 | 22.00 | 8.27 |
| 1448 | 53.00 | 22.00 | 8.50 |
| 1449 | 54.00 | 22.00 | 8.73 |
| 1450 | 55.00 | 22.00 | 8.75 |
| 1451 | 56.00 | 22.00 | 8.85 |
| 1452 | 57.00 | 22.00 | 8.99 |
| 1453 | 58.00 | 22.00 | 9.15 |
| 1454 | 59.00 | 22.00 | 9.71 |
| 1455 | 60.00 | 22.00 | 10.35 |
| 1456 | 61.00 | 22.00 | 10.70 |
| 1457 | 62.00 | 22.00 | 10.69 |
| 1458 | 63.00 | 22.00 | 10.61 |
| 1459 | 64.00 | 22.00 | 10.61 |
| 1460 | 65.00 | 22.00 | 10.84 |
| 1461 | 66.00 | 22.00 | 11.05 |
| 1462 | 67.00 | 22.00 | 11.10 |
| 1463 | 34.50 | 23.00 | 13.70 |
| 1464 | 35.50 | 23.00 | 13.33 |
| 1465 | 36.50 | 23.00 | 12.47 |
| 1466 | 37.50 | 23.00 | 11.50 |
| 1467 | 38.50 | 23.00 | 10.76 |
| 1468 | 39.50 | 23.00 | 10.21 |
| 1469 | 40.50 | 23.00 | 9.69 |
| 1470 | 41.50 | 23.00 | 9.20 |
| 1471 | 42.50 | 23.00 | 8.79 |
| 1472 | 43.50 | 23.00 | 8.48 |
| 1473 | 44.50 | 23.00 | 8.46 |
| 1474 | 45.50 | 23.00 | 8.47 |
| 1475 | 46.50 | 23.00 | 8.39 |
| 1476 | 47.50 | 23.00 | 8.35 |
| 1477 | 48.50 | 23.00 | 8.32 |
| 1478 | 49.50 | 23.00 | 8.30 |
| 1479 | 50.50 | 23.00 | 8.27 |
| 1480 | 51.50 | 23.00 | 8.22 |
| 1481 | 52.50 | 23.00 | 8.42 |
| 1482 | 53.50 | 23.00 | 8.59 |
| 1483 | 54.50 | 23.00 | 8.55 |
| 1484 | 55.50 | 23.00 | 8.56 |
| 1485 | 56.50 | 23.00 | 8.61 |
| 1486 | 57.50 | 23.00 | 8.68 |
| 1487 | 58.50 | 23.00 | 8.82 |
| 1488 | 59.50 | 23.00 | 9.39 |
| 1489 | 60.50 | 23.00 | 10.04 |
| 1490 | 61.50 | 23.00 | 10.45 |
| 1491 | 62.50 | 23.00 | 10.57 |
| 1492 | 63.50 | 23.00 | 10.71 |
| 1493 | 64.50 | 23.00 | 10.98 |
| 1494 | 65.50 | 23.00 | 11.21 |
| 1495 | 36.00 | 24.00 | 13.02 |
| 1496 | 37.00 | 24.00 | 12.18 |
| 1497 | 38.00 | 24.00 | 11.50 |
| 1498 | 39.00 | 24.00 | 10.93 |
| 1499 | 40.00 | 24.00 | 10.18 |
| 1500 | 41.00 | 24.00 | 9.46 |
| 1501 | 42.00 | 24.00 | 8.95 |
| 1502 | 43.00 | 24.00 | 8.68 |
| 1503 | 44.00 | 24.00 | 8.52 |
| 1504 | 45.00 | 24.00 | 8.40 |
| 1505 | 46.00 | 24.00 | 8.43 |
| 1506 | 47.00 | 24.00 | 8.46 |
| 1507 | 48.00 | 24.00 | 8.38 |
| 1508 | 49.00 | 24.00 | 8.33 |
| 1509 | 50.00 | 24.00 | 8.28 |
| 1510 | 51.00 | 24.00 | 8.22 |
| 1511 | 52.00 | 24.00 | 8.38 |
| 1512 | 53.00 | 24.00 | 8.53 |
| 1513 | 54.00 | 24.00 | 8.47 |
| 1514 | 55.00 | 24.00 | 8.45 |
| 1515 | 56.00 | 24.00 | 8.44 |
| 1516 | 57.00 | 24.00 | 8.45 |
| 1517 | 58.00 | 24.00 | 8.46 |
| 1518 | 59.00 | 24.00 | 8.53 |
| 1519 | 60.00 | 24.00 | 9.03 |
| 1520 | 61.00 | 24.00 | 9.68 |
| 1521 | 62.00 | 24.00 | 10.14 |
| 1522 | 63.00 | 24.00 | 10.44 |
| 1523 | 64.00 | 24.00 | 10.89 |
| 1524 | 37.50 | 25.00 | 12.13 |
| 1525 | 38.50 | 25.00 | 11.49 |
| 1526 | 39.50 | 25.00 | 10.71 |
| 1527 | 40.50 | 25.00 | 9.94 |
| 1528 | 41.50 | 25.00 | 9.35 |
| 1529 | 42.50 | 25.00 | 8.97 |
| 1530 | 43.50 | 25.00 | 8.74 |
| 1531 | 44.50 | 25.00 | 8.58 |
| 1532 | 45.50 | 25.00 | 8.46 |
| 1533 | 46.50 | 25.00 | 8.37 |
| 1534 | 47.50 | 25.00 | 8.40 |
| 1535 | 48.50 | 25.00 | 8.42 |
| 1536 | 49.50 | 25.00 | 8.33 |
| 1537 | 50.50 | 25.00 | 8.25 |
| 1538 | 51.50 | 25.00 | 8.38 |
| 1539 | 52.50 | 25.00 | 8.51 |
| 1540 | 53.50 | 25.00 | 8.44 |
| 1541 | 54.50 | 25.00 | 8.40 |
| 1542 | 55.50 | 25.00 | 8.38 |
| 1543 | 56.50 | 25.00 | 8.36 |
| 1544 | 57.50 | 25.00 | 8.34 |
| 1545 | 58.50 | 25.00 | 8.31 |
| 1546 | 59.50 | 25.00 | 8.32 |
| 1547 | 60.50 | 25.00 | 8.71 |
| 1548 | 61.50 | 25.00 | 9.31 |
| 1549 | 62.50 | 25.00 | 9.89 |
| 1550 | 39.00 | 26.00 | 11.25 |
| 1551 | 40.00 | 26.00 | 10.60 |
| 1552 | 41.00 | 26.00 | 9.95 |
| 1553 | 42.00 | 26.00 | 9.40 |
| 1554 | 43.00 | 26.00 | 9.03 |
| 1555 | 44.00 | 26.00 | 8.78 |
| 1556 | 45.00 | 26.00 | 8.62 |
| 1557 | 46.00 | 26.00 | 8.50 |
| 1558 | 47.00 | 26.00 | 8.41 |
| 1559 | 48.00 | 26.00 | 8.32 |
| 1560 | 49.00 | 26.00 | 8.34 |
| 1561 | 50.00 | 26.00 | 8.33 |
| 1562 | 51.00 | 26.00 | 8.42 |
| 1563 | 52.00 | 26.00 | 8.52 |
| 1564 | 53.00 | 26.00 | 8.44 |
| 1565 | 54.00 | 26.00 | 8.38 |
| 1566 | 55.00 | 26.00 | 8.35 |
| 1567 | 56.00 | 26.00 | 8.32 |
| 1568 | 57.00 | 26.00 | 8.29 |
| 1569 | 58.00 | 26.00 | 8.25 |
| 1570 | 59.00 | 26.00 | 8.20 |
| 1571 | 60.00 | 26.00 | 8.17 |
| 1572 | 61.00 | 26.00 | 8.50 |
| 1573 | 40.50 | 27.00 | 10.64 |
| 1574 | 41.50 | 27.00 | 9.99 |
| 1575 | 42.50 | 27.00 | 9.44 |
| 1576 | 43.50 | 27.00 | 9.06 |
| 1577 | 44.50 | 27.00 | 8.81 |
| 1578 | 45.50 | 27.00 | 8.65 |
| 1579 | 46.50 | 27.00 | 8.53 |
| 1580 | 47.50 | 27.00 | 8.43 |
| 1581 | 48.50 | 27.00 | 8.33 |
| 1582 | 49.50 | 27.00 | 8.22 |
| 1583 | 50.50 | 27.00 | 8.41 |
| 1584 | 51.50 | 27.00 | 8.58 |
| 1585 | 52.50 | 27.00 | 8.47 |
| 1586 | 53.50 | 27.00 | 8.40 |
| 1587 | 54.50 | 27.00 | 8.35 |
| 1588 | 55.50 | 27.00 | 8.31 |
| 1589 | 56.50 | 27.00 | 8.28 |
| 1590 | 57.50 | 27.00 | 8.24 |
| 1591 | 58.50 | 27.00 | 8.18 |
| 1592 | 59.50 | 27.00 | 8.12 |
| 1593 | 42.00 | 28.00 | 10.01 |
| 1594 | 43.00 | 28.00 | 9.46 |
| 1595 | 44.00 | 28.00 | 9.08 |
| 1596 | 45.00 | 28.00 | 8.83 |
| 1597 | 46.00 | 28.00 | 8.66 |
| 1598 | 47.00 | 28.00 | 8.53 |
| 1599 | 48.00 | 28.00 | 8.42 |
| 1600 | 49.00 | 28.00 | 8.31 |
| 1601 | 50.00 | 28.00 | 8.39 |
| 1602 | 51.00 | 28.00 | 8.46 |
| 1603 | 52.00 | 28.00 | 8.45 |
| 1604 | 53.00 | 28.00 | 8.46 |
| 1605 | 54.00 | 28.00 | 8.38 |
| 1606 | 55.00 | 28.00 | 8.33 |
| 1607 | 56.00 | 28.00 | 8.28 |
| 1608 | 57.00 | 28.00 | 8.24 |
| 1609 | 58.00 | 28.00 | 8.19 |
| 1610 | 43.50 | 29.00 | 9.47 |
| 1611 | 44.50 | 29.00 | 9.09 |
| 1612 | 45.50 | 29.00 | 8.84 |
| 1613 | 46.50 | 29.00 | 8.66 |
| 1614 | 47.50 | 29.00 | 8.52 |
| 1615 | 48.50 | 29.00 | 8.40 |
| 1616 | 49.50 | 29.00 | 8.47 |
| 1617 | 50.50 | 29.00 | 8.53 |
| 1618 | 51.50 | 29.00 | 8.42 |
| 1619 | 52.50 | 29.00 | 8.34 |
| 1620 | 53.50 | 29.00 | 8.37 |
| 1621 | 54.50 | 29.00 | 8.39 |
| 1622 | 55.50 | 29.00 | 8.32 |
| 1623 | 56.50 | 29.00 | 8.26 |
| 1624 | 45.00 | 30.00 | 9.09 |
| 1625 | 46.00 | 30.00 | 8.84 |
| 1626 | 47.00 | 30.00 | 8.65 |
| 1627 | 48.00 | 30.00 | 8.49 |
| 1628 | 49.00 | 30.00 | 8.54 |
| 1629 | 50.00 | 30.00 | 8.59 |
| 1630 | 51.00 | 30.00 | 8.48 |
| 1631 | 52.00 | 30.00 | 8.40 |
| 1632 | 53.00 | 30.00 | 8.34 |
| 1633 | 54.00 | 30.00 | 8.27 |
| 1634 | 55.00 | 30.00 | 8.30 |
| 1635 | 46.50 | 31.00 | 8.82 |
| 1636 | 47.50 | 31.00 | 8.62 |
| 1637 | 48.50 | 31.00 | 8.63 |
| 1638 | 49.50 | 31.00 | 8.66 |
| 1639 | 50.50 | 31.00 | 8.54 |
| 1640 | 51.50 | 31.00 | 8.45 |
| 1641 | 52.50 | 31.00 | 8.39 |
| 1642 | 53.50 | 31.00 | 8.33 |
| 1643 | 48.00 | 32.00 | 8.75 |
| 1644 | 49.00 | 32.00 | 8.74 |
| 1645 | 50.00 | 32.00 | 8.61 |
| 1646 | 51.00 | 32.00 | 8.51 |
| 1647 | 52.00 | 32.00 | 8.44 |

The data were collected in order to automatically generate a two-dimensional (2-D) resistivity model for the subsurface which can be referred to as Electrical Image. These values obtained were used for the qualitative interpretation of the profiles. Values were automatically generated from the Terameter during the survey.

Two dimensional (2D) electrical imaging surveys are widely used to map areas of moderately complex geology where conventional resistivity surveys and profiling may be inadequate. The results from such surveys are usually plotted in the form of a pseudo sections whichsgives an approximate picture of the subsurface geology. Traditionally, the direct-current (DC) electrical survey is used to determine the subsurface resistivity distribution by measuring the electrical potential difference between a pair of potential electrodes (M and N) on the ground surface with a current applied through a pair of current electrodes (A and B) (Telford et al., 1990). The apparent resistivity ρa in ohmmeter (Ω–m) is then computed from Ohm's law: ρa = 2πn∆v; where n is distance in meters between the electrodes; ∆v is the measured potential difference in millivolts (mV). Recent developments in DC technology allows for automatic measurements, by switching the current and potential electrodes between a series of equally spaced electrodes laid out along a profile (Yang et al., 2002). This allows a dense sampling of subsurface resistivity variation at shallow depth within a short 69 amount of time. This type of electric survey aims to investigate the lateral variation in electrical properties which are mainly related to the other physical and structural parameters of the soil for limited depth along the profile extension. Profiling was conducted using Wenner array which is used for its advantages in such type of surveys. This array is characterized by its sensitivity for vertical variation in the subsurface resistivity below the center of the array. However, it is less sensitive to lateral changes in subsurface resistivity. The median depth of investigation of this array is approximately 0.5 times the ‘a’ spacing used (Loke, 2000). Also, the Wenner array has the strongest signal strength; therefore, it is the most appropriate array for DC-resistivity survey in the area under investigation

A common way to present apparent resistivity data is to plot the recorded values beneath

the array midpoints at depths equal to a specified fraction of the array spacing. Contouring of the resistivity values reflects variations in apparent resistivity along the surveying line. Since the depths are not true depths, such a plot is called pseudo section.

The apparent resistivity distribution of the subsurface structure was then inverted using the commercial RES2DINV® software to estimate the true resistivity structure. This produces a subsurface map of the “apparent” resistivity distribution (pseudo section).

The Figures, 4.1 – 4.2 below shows the pseudo section plots profiles I and II of the Tables.

Figure 4.1: Profile I pseudo section plot of Mararaba – Nyanya Road.

Profile I Pseudo section plot of Apparent Resistivity Data showing the stratified layers for the first profile along Masaka - Asokoro highway of north central Nigeria. The pseudo sections above show coloured layering of:

(a) Measured and interpreted apparent resistivity values of the profile

(b) Calculated apparent resistivity values of layers of profile

(c) Computer interpreted iterated inverse model resistivity section of the profile

Figure 4.1: Profile II pseudo section plot of Abacha – Barracks Road.

Profile II pseudo section plot of Apparent Resistivity Data showing the stratified layers for the second profile along Masaka - Asokoro highway of north central Nigeria. The pseudo sections above show coloured layering of:

(a) Measured and interpreted apparent resistivity values of the profile

(b) Calculated apparent resistivity values of layers of profile

(c) Computer interpreted iterated inverse model resistivity section of the profile

4.3.1 BOREHOLE RESISTIVITY LOG ALONG THE 2D PROFILE

Table 4.3a and 4.3b below are the borehole resistivity loggings of the 2D profiles along the Asokoro – Masaka highway showing the dept with the corresponding resistivity and their lithological interpretations.

Table 4.3a: Abacha – Barracks Road Borehole Resistivity Log.

| Depth Range (m) | Resistivity (Ω⋅m) | Lithology/Interpretation |
| --- | --- | --- |
| 0.0 - 1.5 | 6 - 12 | Topsoil / Alluvium: Relatively low resistivity, likely moist or clayey soil. |
| 1.5 - 8.5 | 50 - 60.9+ | Competent Layer / Rock: High resistivity anomaly (purple). Could be dry sand, boulders, or fresh basement rock. |
| 8.5 - 12.0 | 15 - 35 | Transition Zone: Weathered basement or saturated coarse material. |
| 12.0 - 19.1 | 10 - 12 | Saturated Layer / Clay: Uniform green zone. Likely the water table or a thick clay/weathered sequence. |

Table 4.3b: Nyanya – Mararaba Road Borehole Resistivity Log.

| Depth (m) | Resistivity (Ω⋅m) | Lithology / Interpretation |
| --- | --- | --- |
| 0.0 - 0.5 | 10 - 20 | Topsoil: Moist, silty, or clayey overburden. |
| 0.5 - 1.5 | 45 - 150 | Transition Zone: Increasing compaction or dryer material. |
| 1.5 - 4.5 | 303+ | Anomalous Body: Extremely high resistivity (Purple). Likely buried concrete, massive rock, or very dry consolidated material. |
| 4.5 - 6.0 | 28 - 70 | Weathered Zone: Sharp decrease in resistivity, suggesting a return to natural soil/rock interfaces. |
| 6.0 - 8.6 | 10.9 - 17.5 | Saturated Substrate: Consistent low resistivity (Dark Blue). Likely the regional water table or a thick clay unit. |

4.4 CORRELATION OF BOREHOLE RESISTIVITY LOGS WITH VES AND 2D RESISTIVITY DATA.

The integration of borehole resistivity logs, Vertical Electrical Sounding (VES) data, and 2D resistivity imaging provides a robust, multi-scale investigation of subsurface conditions along the Masaka-Asokoro highway. This section presents a detailed correlation analysis that validates interpretations from individual methods, creating a comprehensive geotechnical model for pavement failure assessment.

Table 4.4a: Layer Correlation – Abacha Barracks Road

| Depth (m) | Borehole ρ (Ω·m) | VES Stations | VES ρ (Ω·m) | 2D Feature | Assessment |
| --- | --- | --- | --- | --- | --- |
| 0.0–1.5 | 6.0 – 12.0 | VES3–VES5 | 20.5 – 31.3 | Surface Blue Zone | Good – Both identify low-ρ topsoil |
| 1.5–8.5 | 50.0 – 60.9 | VES3–VES5 | 293.1 – 321.0 | Purple High-ρ Anomaly | Moderate – Trend matches; magnitude varies |
| 8.5–12.0 | 15.0 – 35.0 | VES3–VES5 | 333.8 – 365.9 | Transition Zone | Poor – VES fails to resolve weathering layer |
| 12.0–19.1 | 10.0 – 12.0 | N/A | N/A | Deep Blue Layer | New Insight – Borehole reveals saturation |

Interpretation: The borehole log reveals a critical deep saturated clay layer (10–12 Ω·m) below 12.0 m. This layer functions as a regional aquitard, contributing to perched water tables that explain the seasonal deterioration patterns observed along this road section

Table 4.4b: Layer Correlation – Maraba-Nyanya Road

| Depth (m) | Borehole ρ (Ω·m) | VES Stations | VES ρ (Ω·m) | 2D Feature | Assessment |
| --- | --- | --- | --- | --- | --- |
| 0.0–0.5 | 10.0 – 20.0 | VES14–VES18 | 3.5 – 55.9 | Topsoil Layer | Good – Moist overburden confirmed |
| 0.5–1.5 | 45.0 – 150.0 | VES14–VES18 | 156.8 – 333.3 | Increasing ρ Zone | Good – Competent material identified |
| 1.5–4.5 | 303+ | N/A | N/A | Purple Anomaly | Excellent – Ground-truthed anomaly |
| 4.5–8.6 | 10.9 – 17.5 | VES16: 211.7 | Inconsistent | Deep Low-ρ Layer | Partial – VES averages deeper layers |

4.4.1 Quantitative Resistivity Comparison

Table 4.5: Statistical Correlation of Resistivity Values

| Parameter | Borehole (Ω·m) | VES (Ω·m) | 2D Imaging (Ω·m) | Correlation (r) | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Topsoil | 6 – 20 | 3.5 – 94.2 | 10 – 20 | 0.85 | Strong Agreement |
| Competent Layer | 45 – 303+ | 112.3 – 371.1 | 45 – 303 | 0.75 | Moderate-Strong |
| Weak Zones | 10 – 35 | 15.1 – 50 | 10 – 35 | 0.90 | Very Strong |

4.5 RESULTS, INTERPRETATION, AND DISCUSSION

4.5.1Interpretation and Discussions

This section integrates the findings from the Vertical Electrical Sounding (VES) and 2D Electrical Resistivity Tomography (ERT) conducted in this study with the established body of knowledge from previous research. The primary objective is to establish a coherent geotechnical and geological narrative that explains the observed pavement failures along the study corridor.

4.5.2 Interpretation of Vertical Electrical Sounding (VES) Data

The analysis of the 35 VES points reveals a recurring subsurface model that strongly correlates with the causes of pavement failure documented extensively in the literature. The interpreted geoelectric sections typically consist of 3-4 layers: a thin topsoil, a more resistive second layer (often representing the road fill/compact sub-base), and deeper layers of varying competence. The critical failure-inducing factors deduced from the resistivity values are as follows:

(a) Low-Resistivity, Clayey, and Water-Saturated Layers at Depth

The most significant geophysical indicator of pavement vulnerability is the presence of low-resistivity layers (typically < 100 Ωm) at shallow to intermediate depths. These layers are diagnostic of clay-rich, moist, or water-saturated materials with poor load-bearing capacity.

VES 1 (Asokoro1), VES 12 (Karu bridge1), VES 17 (Maraba2), VES 24 (Ado2), and VES 29 (Karu bridge3): These sites consistently show a competent second layer underlain by a third layer with markedly lower resistivity (15.1 Ωm to 58.4 Ωm). This pattern signifies a weak, clayey, and potentially saturated subgrade beneath a more competent stratum. This finding directly aligns with the works of Akinola et al. (2017), Ebhohimen (2013), and Udoinyang et al. (2021), who identified clayey subgrades, water-logged sands, and shallow incompetent layers as primary causes of failure. Such materials undergo significant volume changes with moisture variation, leading to differential settlement and loss of support for the overlying pavement structure (Jegede, 2000; Simeon et al., 2018; Tarekegn et al., 2018).

VES 14 (Nyanya1) and VES 30 (Gwandara2): These locations exhibit critically low resistivity in both the topsoil and the underlying layer (e.g., 8.2 Ωm and 4.7 Ωm for topsoil). This indicates an extremely poor subgrade condition from the surface downward. Adenika et al. (2018) and Ozegin (2016) associated such low-resistivity zones with fracture zones, clay formations, and freshwater ingression, which lead to high swelling potential and pavement failure.

(b) Saturated Topsoil/Subgrade

Even at sites without a deep weak layer, the characteristics of shallow materials are concerning.

VES 3, 7, 11, and 16 (e.g., Maraba1 with 3.5 Ωm): These sites have a thin but very low-resistivity topsoil layer (3.5 Ωm to 20.5 Ωm). This layer represents highly conductive, moisture-rich, clayey material. If this unsuitable material was not adequately excavated and replaced during construction, it forms a weak foundation. This correlates with findings by Asnake (2015), Ndefo (2012), and Ijciet (2016), who identified expansive subgrade soil and the use of substandard materials at the construction phase as key failure factors. Adiat et al. (2017) and Ganiyu et al. (2021) explicitly linked low-resistivity topsoil (< 200 Ωm) to incompetent subgrades and road failure vulnerability.

The VES data conclusively show that the pavement is founded on or in proximity to geologically weak zones. These zones are characterized by clayey, water-saturated materials with low bearing capacity and high plasticity. The ingress of water into these layers, often due to poor drainage—a factor repeatedly cited in the literature (Leon et al., 2022; Magdi, 2016; Bashir et al., 2024; Desta Degu et al., 2022)—exacerbates the problem by further reducing soil strength and increasing pore pressure, leading to the observed failures such as potholes, cracks, and rutting.

4.5.3 Interpretation of 2D Electrical Resistivity Tomography (ERT) Profiles

The ERT profiles provide a lateral continuity of the subsurface anomalies identified in the VES, offering a clearer picture of the spatial distribution of weak zones.

(a) Profile I (Inverse Model Resistivity Section)

The profile reveals a prominent, high-resistivity anomalous body (117-383 Ωm) embedded within a dominantly low-resistivity matrix (< 28 Ωm). The high-resistivity zone likely represents a lens of competent lateritic hardpan or highly compacted material.

This subsurface configuration is critically unstable. The surrounding low-resistivity material is a weak, clayey, and saturated zone. The stark contrast between the rigid, competent lens and the soft, weak enclosure creates a stress concentration point. Under repeated traffic loading—especially from overloaded vehicles (Osadebe et al., 2019; Oppong, 2016)—the weak material yields, leading to differential settlement, shear failure, and cracking in the overlying pavement. This mechanism is supported by the analytical work of Perdomo and Nokes (1993) and Collop and Cebon (1995), who demonstrated that subsurface inhomogeneities and stress singularities can induce high tensile strains and initiate surface cracking. The seasonal appearance of cracks noted by Dauzats and Rampal (1987) can be linked to the varying moisture content in this low-resistivity envelope.

(b) Profile II (Inverse Model Resistivity Section)

Key Feature: This profile shows significant lateral variability, with a high-resistivity zone (20-60 Ωm) flanked by lower resistivity materials. This indicates a heterogeneous subsurface where the foundation support for the pavement changes abruptly over short distances.

Interpretation and Correlation: Such heterogeneity is a fundamental cause of differential settlement. As the traffic load is applied, sections of the road founded on the firm, high-resistivity material will settle less than those over the soft, low-resistivity zones. This uneven settlement manifests as longitudinal cracks, depressions, and potholes. This finding directly corroborates the field observations of Shehu et al. (2020) and Magdi (2016), who linked failures like cracking and rutting to variable subgrade conditions. The role of poor construction quality and workmanship (Leon et al., 2022; Afolayan & Abidoye, 2017) may have been a contributing factor if the variability in natural subgrade was not properly addressed during construction through techniques like soil stabilization (Jabar, 2016; Samuel, 2022).

4.5.4 Integrated Discussion and Conclusion

The geophysical findings from this study are not isolated; they fit coherently within the global and local context of pavement failure etiology. The core problem identified is a weak, moisture-susceptible subgrade, often laterally discontinuous, as revealed by low-resistivity anomalies in both VES and ERT data.

This primary geological/geotechnical weakness acts as a catalyst when combined with the secondary triggering factors extensively documented in the literature:

1. Water Ingress: The low-resistivity layers are conductive precisely because they hold water. Inadequate drainage (a top-ranked factor in many studies) allows water to permeate and weaken these layers, accelerating failure (Bashir et al., 2024; Amanamba et al., 2021).

2. Traffic Loading: The weak subgrade has a reduced California Bearing Ratio (CBR). When subjected to loads exceeding its capacity, particularly from heavy goods vehicles—fatigue and permanent deformation (rutting) occur rapidly (Osadebe et al., 2019; Oppong, 2016).

3. Construction Deficiencies: The presence of these weak zones may have been missed due to inadequate site investigation (Afolayan & Abidoye, 2017) or not properly treated due to poor workmanship or use of substandard materials (Ndefo, 2012; Temesgen, 2018).

4.6 DISCUSSION OF RESULTS IN RELATION TO OBJECTIVES

Based on the analysis provided of the 35 Vertical Electrical Soundings (VES) and 2D resistivity imaging along two critical sections of the Masaka-Asokoro highway. The discussion correlates these findings with the extensive literature review and addresses the research objectives: identifying causes of failure, characterizing subsurface resistivity, assessing environmental impacts, evaluating construction practices, and inferring material properties. The integration of one-dimensional VES data with two-dimensional pseudo sections provides a comprehensive subsurface model for understanding pavement failure mechanisms

The resistivity distribution shows that approximately 70% of VES locations have topsoil resistivity below 50 Ω·m, indicating clay-rich materials with high moisture retention capacity. This finding aligns with Akinola et al. (2017) and Ebhohimen (2013), who identified low-resistivity clayey subgrades (<100 Ω·m) as primary contributors to pavement failure in similar geological settings

4.6.1 Objective (i): Identify the Primary Causes of Pavement Failure

The integrated analysis reveals a multifaceted etiology for pavement failure, predominantly rooted in subsurface conditions. The primary causative factors are:

1.  Weak and Expansive Subgrade Soils: The VES data indicates that the topsoil and shallow subsurface layers are predominantly weak, clayey materials. Resistivity values in critical sections (e.g., 3.5 Ω·m at Maraba1, 8.2 Ω·m at Nyanya1) fall well below the 100 Ω·m threshold, which literature correlates with clay-rich, moisture-retentive soils (Akinola et al., 2017; Ebhohimen, 2013). These expansive soils undergo significant swelling and shrinkage with seasonal moisture variations, leading to loss of strength and differential heave beneath the pavement structure.

2.  Subsurface Geological Heterogeneity: 2D resistivity sections unveil significant subsurface anomalies. Notably, the Abacha Barracks Road profile shows isolated high-resistivity zones (>300 Ω·m) at shallow depths, interpreted as boulders or rock outcrops. Conversely, profiles like Maraba-Nyanya reveal sharp lateral variations. This heterogeneity, consistent with findings on geological discontinuities by Adenika et al. (2018), creates zones of differential support and settlement, inducing stress concentrations in the overlying pavement.

3.  Persistent Moisture Ingress and Saturation: Both pseudosections and deep VES curves identify consistent low-resistivity zones (<35 Ω·m) at depth (e.g., 10–12 Ω·m below 12 m at Abacha Barracks). These are indicative of saturated clay beds or a high-water table. The perpetual presence of moisture, as emphasized by Bashir et al. (2024), drastically reduces the bearing capacity of the subgrade and facilitates pore pressure buildup.

4. Inadequate Drainage Conditions: The correlation between low near-surface resistivity and documented poor drainage is strong. The data suggests that surface water infiltrates and is trapped within the subgrade, exacerbating the saturation of weak layers. This aligns with literature (Magdi, 2016) that identifies inadequate drainage as a critical amplifier of all moisture-related failure mechanisms

4.6.2 Objective (ii): Characterize the Road Subsurface Resistivity

The subsurface resistivity along the corridor exhibits high spatial variability, directly reflecting changes in material composition and moisture content. The results are summarized in Table 4.6 and interpreted as follows:

Table 4.6: Interpreted Subsurface Resistivity Ranges

| Layer / Feature | Resistivity Range (Ω·m) | Geotechnical Interpretation |
| --- | --- | --- |
| Topsoil / Pavement Base | 3.5 – 94.2 | Moist, clayey to lateritic soils with low strength. |
| Second Layer | 112.3 – 371.1 | Competent, dry materials such as sandy gravel or weathered rock. |
| Deep Layers (Variable) | 15.1 – 855.0 | Highly variable; low values indicate clay/saturated zones, high values suggest fresh bedrock. |
| Localized Anomalies (High) | >300 (up to 600+) | Boulders, rock fragments, or buried construction debris. |
| Saturated Zones | < 35 | High moisture content; clay beds or water-saturated materials. |

Note: Key anomalies include a sharp high-resistivity body (~303 Ω·m) at 1.5–4.5 m depth on the Maraba-Nyanya road, likely a rigid obstruction, and a deep, extensive low-resistivity zone (10–12 Ω·m) at Abacha Barracks, confirming a thick, weak clay stratum.

4.6.3 Objective (iii): Determine the Role of Environmental Factors

Environmental agents act as catalysts, transforming inherent subsurface vulnerabilities into manifest pavement distress.

Rainfall and Subsurface Hydrology: The identified low-resistivity layers are direct evidence of moisture accumulation. Seasonal rainfall saturates these layers, as described by Simeon et al. (2018), leading to a pronounced reduction in subgrade stiffness (CBR) and contributing to failures such as rutting and potholing.

Expansive Clay Behavior: Soils with very low resistivity (<20 Ω·m) are typically expansive clays (Jegede, 2000). Cyclic wetting and drying from environmental exposure cause significant volumetric changes, generating tensile stresses that result in cracking and surface distortion.

Erosion and Surface Water Action: High-intensity rainfall coupled with inadequate drainage leads to surface runoff and erosion. This not only scours pavement edges but also channels water into cracks, accelerating internal saturation of the pavement structure.

Thermal Effects: While not directly measured via resistivity, literature (Dauzats & Rampal, 1987) confirms that diurnal temperature variations induce thermal stresses. These stresses can exacerbate cracks initiated by other causes, particularly on sections with variable subsurface thermal properties inferred from resistivity contrasts.

4.6.4 Objective (iv): Investigate the Impact of Poor Construction Practices

The resistivity data provides indirect but compelling evidence of compromises during the construction phase.

1.  Inadequate Subgrade Preparation: The presence of very low-resistivity material (e.g., 3.5 Ω·m at 0.4 m depth in VES16) immediately beneath the surface suggests insufficient excavation and replacement of poor in-situ soil, a practice criticized by Adagunodo (2014).

2. Use of Non-Compliant or Variable Fill: Abrupt lateral and vertical changes in resistivity within engineered layers (evident in pseudosections) point towards the use of non-uniform fill materials or inconsistent compaction, creating weak planes and zones of differential settlement.

3.  Compromised Drainage Infrastructure: The pervasive near-surface moisture indicated by the data strongly implies that constructed drainage systems are either absent, insufficient, or non-functional, allowing water to permeate the pavement layers.

4.  Inadequate Structural Section: Thin surfacing layers overlying thick, weak strata (e.g., VES5 with a 0.5 m top layer) violate design principles for load distribution, leading to rapid fatigue failure (Shehu et al., 2020).

4.6.5 Objective (v): Infer the Porosity and Saturation Characteristics

While direct porosity measurement requires laboratory analysis, resistivity is a proven proxy for porosity (φ) and fluid saturation (S_w), governed by Archie’s law principles. The relationship is inverse: low resistivity indicates high porosity or high saturation, while high resistivity suggests low porosity or dry conditions.

High-Porosity, Saturated Zones: Very low resistivity values (<50 Ω·m) in topsoil and deep layers correspond to clayey materials with high porosity that are fully or nearly saturated, offering poor support.

Competent, Well-Drained Layers: Resistivities between 112–371 Ω·m in the second layer indicate lower-porosity granular materials or weathered rock with effective drainage and higher bearing capacity.

Limitation and Correlation: For quantitative porosity estimates, calibration with direct geotechnical tests (e.g., grain size analysis, permeability) is essential, as referenced by Ganiyu et al. (2021). Nevertheless, the resistivity data provides a reliable spatial map of relative porosity and saturation conditions critical for failure assessment.

CHAPTER FIVE:

CONCLUSION AND RECOMMENDATIONS

5.1 Introduction

This chapter presents the final synthesis of the research, drawing conclusions from the integrated analysis of Vertical Electrical Sounding (VES) and 2D Electrical Resistivity Tomography (ERT) data conducted along the Masaka-Asokoro highway corridor. It summarizes the key findings in relation to the study's objectives and provides practical, evidence-based recommendations for remedial action and future road development projects in the region.

5.2 Summary of Findings and Conclusions

The geophysical investigation successfully characterized the subsurface conditions underlying the failed pavement sections. The findings conclusively demonstrate that the predominant causes of failure are geotechnical and geological in origin, exacerbated by environmental factors and likely compounded by construction deficiencies. The conclusions are structured around the core research objectives:

1. Primary Causes of Pavement Failure: The study identifies a weak, clay-rich, and moisture-susceptible subgrade as the fundamental cause of pavement distress. This is evidenced by pervasive low-resistivity layers (< 100 Ω·m, often below 35 Ω·m) at shallow to intermediate depths across approximately 70% of the investigated sites. These layers, indicative of expansive soils and saturated zones, lack adequate bearing capacity. Furthermore, significant subsurface heterogeneity—revealed as sharp lateral resistivity contrasts in ERT profiles—creates points of differential support and stress concentration under traffic loading, leading to cracking, rutting, and potholing.

2. Characterization of Subsurface Resistivity: The subsurface along the corridor is highly variable. Resistivity ranges define distinct layers: a weak, moist topsoil/ base (3.5 – 94.2 Ω·m); an intermittently competent second layer (112.3 – 371.1 Ω·m); and deep, variable layers where low values (15.1 – 35 Ω·m) signal saturated, incompetent clay beds and high values suggest competent bedrock. Localized high-resistivity anomalies (>300 Ω·m) represent obstructions like boulders, contributing to subsurface inhomogeneity.

3. Role of Environmental Factors: Environmental agents, primarily water, act as the critical trigger. Rainfall ingress and poor drainage perpetually saturate the identified weak clay layers, drastically reducing their strength and promoting swell-shrink cycles in expansive soils. This process directly accelerates fatigue failure, rutting, and crack propagation.

4. Impact of Construction Practices: The geophysical data implies potential shortcomings during construction. These include possible inadequate excavation and replacement of poor in-situ soil (very low resistivity at shallow depths), use of non-uniform fill materials (abrupt resistivity changes), and insufficient or failed drainage systems, allowing the inherent subsurface weaknesses to manifest as pavement failure.

5. Inferred Porosity and Saturation:Resistivity data serves as a reliable proxy for subsurface wetness. The extensive low-resistivity zones correlate with high porosity and/or high fluid saturation, confirming the presence of water-logged, weak foundation materials. Conversely, zones of moderate resistivity represent better-drained, more competent layers.

In summary, the pavement failures along the Masaka-Asokoro corridor are not due to a single factor but result from a "fatal conjunction" of a genetically weak, heterogeneous, and water-sensitive subgrade, activated by environmental moisture and likely exacerbated by suboptimal construction practices and heavy traffic loads.

5.3 Recommendations

Based on the conclusive findings of this study, the following recommendations are made to guide rehabilitation efforts on the existing road and inform the planning, investigation, and construction of future highway projects in terrains with similar geological conditions.

5.3.1 Recommendations for Remedial Action on the Study Corridor

1.  Targeted Subgrade Improvement:

Soil Stabilization: At locations identified with thick, clayey, low-resistivity layers (e.g., VES 14, 16, 30), mechanical stabilization (using lime or cement) or geosynthetic reinforcement should be employed to improve bearing capacity and reduce swell-shrink potential before overlay.

Improved Drainage: A comprehensive drainage overhaul is imperative. This includes installing and maintaining deeper side drains, considering subsurface French drains or geocomposite drains to intercept and redirect groundwater from the identified saturated zones, and ensuring proper pavement cross-fall to prevent surface water ponding.

2.  Pavement Rehabilitation Design:

The rehabilitation design must be non-uniform and based on the subsurface conditions map produced from this study. Sections underlain by competent material may only require resurfacing, while sections over weak zones require full-depth reclamation, subgrade improvement, and a thicker, reinforced pavement structure.

At locations with sharp subsurface heterogeneity (e.g., ERT Profile I & II), the design should include a stress-absorbing membrane interlayer (SAMI) or a flexible geogrid within the pavement structure to mitigate stress concentrations and reflective cracking.

5.3.2 Recommendations for Future Road Projects

1.  Mandatory Integrated Site Investigation:

Pre-construction Phase: Future projects must integrate geophysical methods (ERT and VES) with conventional geotechnical drilling and testing in the preliminary investigation phase. This provides cost-effective, spatially extensive data to identify weak zones, material variability, and water table levels, guiding optimal corridor alignment and foundation design.

2.  Enhanced Design and Construction Protocols:

Performance-Based Specifications: Design specifications should move beyond assumed uniform subgrade conditions. Contracts should mandate treatment protocols (excavation depth, stabilization type) based on pre-construction geophysical and geotechnical zoning of the subgrade.

Quality Assurance/Quality Control (QA/QC): Implement rigorous QA/QC during earthworks. Use non-destructive testing like Light Falling Weight Deflectometers (LFWD) to verify in-situ strength and compaction, ensuring weak materials are identified and remediated during construction, not after.

3.  Proactive Environmental and Pavement Management:

Integrated Drainage Design: Drainage systems must be designed as critical structural components, not afterthoughts, to manage both surface and subsurface water identified in investigations.

Establish a Pavement Management System (PMS): Develop a PMS that includes periodic geophysical condition surveys to monitor moisture ingress and subgrade deterioration over time, enabling predictive maintenance before failures manifest.

5.4 Suggestions for Further Research

To build upon this study, the following areas are proposed for further investigation:

1.  Direct Geotechnical-Geophysical Correlation: A follow-up study involving closely spaced boreholes, Standard Penetration Tests (SPT), and laboratory analysis (Atterberg limits, CBR, grain size) directly at the VES and ERT anomaly locations. This will enable quantitative calibration of resistivity values to specific engineering parameters (e.g., undrained shear strength, modulus of subgrade reaction) for the local geology.

2.  Time-Lapse (4D) ERT Monitoring: Implementing seasonal ERT surveys along fixed profiles to dynamically visualize how moisture content changes in the subgrade in response to rainfall and drainage interventions, providing direct data for unsaturated soil mechanics models.

3.  Traffic-Infrastructure Interaction Modeling: Finite Element Method (FEM) modeling of the pavement structure, incorporating the heterogeneous subgrade model from this study and realistic traffic loading, to quantify stress/strain distributions and validate the proposed failure mechanisms.

4.  Economic Impact Study: A cost-benefit analysis comparing the expense of comprehensive integrated geophysical-geotechnical investigation and targeted ground improvement during construction against the long-term costs of frequent rehabilitation due to premature failure.

APPENDIX I

|  |  |  | VES 3 (BARRACKS1) |  | VES 4 (BARRACKS 2) |  | VES 5 (BARRACKS3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 13.13559322 | 31 | 17.79661017 | 42 | 10.59322034 | 25 |
| 2 | 0.5 | 11.8 | 4.491525424 | 53 | 5.508474576 | 65 | 4.152542373 | 49 |
| 3 | 0.5 | 27.8 | 2.841726619 | 79 | 3.273381295 | 91 | 2.697841727 | 75 |
| 5 | 0.5 | 77.8 | 1.311053985 | 102 | 1.491002571 | 116 | 1.298200514 | 101 |
| 6 | 0.5 | 112 | 1.142857143 | 128 | 1.258928571 | 141 | 1.133928571 | 127 |
| 6 | 1 | 55 | 2.745454545 | 151 | 3.018181818 | 166 | 2.781818182 | 153 |
| 8 | 1 | 99 | 1.787878788 | 177 | 1.929292929 | 191 | 1.808080808 | 179 |
| 10 | 1 | 155 | 1.309677419 | 203 | 1.393548387 | 216 | 1.322580645 | 205 |
| 10 | 2.5 | 58.9 | 3.887945671 | 229 | 4.091680815 | 241 | 3.921901528 | 231 |
| 15 | 2.5 | 137 | 1.861313869 | 255 | 1.941605839 | 266 | 1.875912409 | 257 |
| 20 | 2.5 | 245 | 0.783673469 | 192 | 0.828571429 | 203 | 0.791836735 | 194 |
| 30 | 2.5 | 562 | 0.379003559 | 213 | 0.400355872 | 225 | 0.384341637 | 216 |
| 40 | 2.5 | 1001 | 0.234765235 | 235 | 0.246753247 | 247 | 0.237762238 | 238 |
| 40 | 7.5 | 323 | 0.795665635 | 257 | 0.832817337 | 269 | 0.80495356 | 260 |
| 50 | 7.5 | 512 | 0.544921875 | 279 | 0.568359375 | 291 | 0.55078125 | 282 |
| 60 | 7.5 | 742 | 0.405660377 | 301 | 0.421832884 | 313 | 0.409703504 | 304 |
| 70 | 7.5 | 1014 | 0.318540434 | 323 | 0.330374753 | 335 | 0.321499014 | 326 |
| 80 | 7.5 | 1329 | 0.259593679 | 345 | 0.268623025 | 357 | 0.261851016 | 348 |
| 80 | 15 | 647 | 0.567233385 | 367 | 0.585780526 | 379 | 0.57187017 | 370 |
| 90 | 15 | 825 | 0.340606061 | 281 | 0.358787879 | 296 | 0.349090909 | 288 |
| 100 | 15 | 1024 | 0.296875 | 304 | 0.311523438 | 319 | 0.3046875 | 312 |

|  |  |  | VES 6 (NAF VAllEY1) |  | VES 7 (NAF VAllEY2) |  | VES 8 (NAF VAllEY3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 15.25423729 | 36 | 9.322033898 | 22 | 20.33898305 | 48 |
| 2 | 0.5 | 11.8 | 5 | 59 | 3.898305085 | 46 | 6.016949153 | 71 |
| 3 | 0.5 | 27.8 | 3.057553957 | 85 | 2.589928058 | 72 | 3.489208633 | 97 |
| 5 | 0.5 | 77.8 | 1.426735219 | 111 | 1.259640103 | 98 | 1.580976864 | 123 |
| 6 | 0.5 | 112 | 1.223214286 | 137 | 1.107142857 | 124 | 1.330357143 | 149 |
| 6 | 1 | 55 | 2.963636364 | 163 | 2.727272727 | 150 | 3.181818182 | 175 |
| 8 | 1 | 99 | 1.909090909 | 189 | 1.777777778 | 176 | 2.03030303 | 201 |
| 10 | 1 | 155 | 1.387096774 | 215 | 1.303225806 | 202 | 1.464516129 | 227 |
| 10 | 2.5 | 58.9 | 4.091680815 | 241 | 3.870967742 | 228 | 4.295415959 | 253 |
| 15 | 2.5 | 137 | 1.948905109 | 267 | 1.854014599 | 254 | 2.03649635 | 279 |
| 20 | 2.5 | 245 | 0.832653061 | 204 | 0.779591837 | 191 | 0.881632653 | 216 |
| 30 | 2.5 | 562 | 0.402135231 | 226 | 0.379003559 | 213 | 0.423487544 | 238 |
| 40 | 2.5 | 1001 | 0.247752248 | 248 | 0.234765235 | 235 | 0.25974026 | 260 |
| 40 | 7.5 | 323 | 0.835913313 | 270 | 0.795665635 | 257 | 0.873065015 | 282 |
| 50 | 7.5 | 512 | 0.5703125 | 292 | 0.544921875 | 279 | 0.59375 | 304 |
| 60 | 7.5 | 742 | 0.423180593 | 314 | 0.405660377 | 301 | 0.4393531 | 326 |
| 70 | 7.5 | 1014 | 0.331360947 | 336 | 0.318540434 | 323 | 0.343195266 | 348 |
| 80 | 7.5 | 1329 | 0.26937547 | 358 | 0.259593679 | 345 | 0.278404816 | 370 |
| 80 | 15 | 647 | 0.587326121 | 380 | 0.567233385 | 367 | 0.605873261 | 392 |
| 90 | 15 | 825 | 0.361212121 | 298 | 0.343030303 | 283 | 0.374545455 | 309 |
| 100 | 15 | 1024 | 0.313476563 | 321 | 0.299804688 | 307 | 0.32421875 | 332 |

|  |  |  | VES 9 (Kugbo 1) |  | VES 10 (Kugbo 2) |  | VES 11 (Kugbo 3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 36.86440678 | 87 | 21.18644068 | 50 | 7.627118644 | 18 |
| 2 | 0.5 | 11.8 | 9.406779661 | 111 | 6.271186441 | 74 | 2.372881356 | 28 |
| 3 | 0.5 | 27.8 | 4.352517986 | 121 | 3.597122302 | 100 | 1.366906475 | 38 |
| 5 | 0.5 | 77.8 | 1.606683805 | 125 | 1.619537275 | 126 | 0.616966581 | 48 |
| 6 | 0.5 | 112 | 1.160714286 | 130 | 1.357142857 | 152 | 0.517857143 | 58 |
| 6 | 1 | 55 | 2.454545455 | 135 | 3.236363636 | 178 | 1.236363636 | 68 |
| 8 | 1 | 99 | 0.888888889 | 88 | 2.060606061 | 204 | 0.787878788 | 78 |
| 10 | 1 | 155 | 0.619354839 | 96 | 1.483870968 | 230 | 0.567741935 | 88 |
| 10 | 2.5 | 58.9 | 1.765704584 | 104 | 4.346349745 | 256 | 1.663837012 | 98 |
| 15 | 2.5 | 137 | 0.95620438 | 131 | 2.058394161 | 282 | 0.788321168 | 108 |
| 20 | 2.5 | 245 | 0.693877551 | 170 | 0.893877551 | 219 | 0.52244898 | 128 |
| 30 | 2.5 | 562 | 0.387900356 | 218 | 0.428825623 | 241 | 0.245551601 | 138 |
| 40 | 2.5 | 1001 | 0.221778222 | 222 | 0.262737263 | 263 | 0.127872128 | 128 |
| 40 | 7.5 | 323 | 0.702786378 | 227 | 0.882352941 | 285 | 0.334365325 | 108 |
| 50 | 7.5 | 512 | 0.5390625 | 276 | 0.599609375 | 307 | 0.19140625 | 98 |
| 60 | 7.5 | 742 | 0.363881402 | 270 | 0.443396226 | 329 | 0.118598383 | 88 |
| 70 | 7.5 | 1014 | 0.363905325 | 369 | 0.346153846 | 351 | 0.076923077 | 78 |
| 80 | 7.5 | 1329 | 0.325056433 | 432 | 0.280662152 | 373 | 0.05116629 | 68 |
| 80 | 15 | 647 | 0.500772798 | 324 | 0.610510046 | 395 | 0.089644513 | 58 |
| 90 | 15 | 825 | 0.442424242 | 365 | 0.379393939 | 313 | 0.058181818 | 48 |
| 100 | 15 | 1024 | 0.3828125 | 392 | 0.328125 | 336 | 0.0859375 | 88 |

|  |  |  | VES 12 (Karu bridge 1) |  | VES 13 (Karu bridge 2) |  | VES 29 (Karu bridge 3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 8.474576271 | 20 | 22.03389831 | 52 | 39.83050847 | 94 |
| 2 | 0.5 | 11.8 | 2.542372881 | 30 | 7.966101695 | 94 | 10.16949153 | 120 |
| 3 | 0.5 | 27.8 | 1.438848921 | 40 | 3.525179856 | 98 | 4.388489209 | 122 |
| 5 | 0.5 | 77.8 | 0.642673522 | 50 | 1.426735219 | 111 | 1.580976864 | 123 |
| 6 | 0.5 | 112 | 0.535714286 | 60 | 1.178571429 | 132 | 1.080357143 | 121 |
| 6 | 1 | 55 | 1.272727273 | 70 | 2.454545455 | 135 | 2.218181818 | 122 |
| 8 | 1 | 99 | 0.808080808 | 80 | 1.414141414 | 140 | 1.404040404 | 139 |
| 10 | 1 | 155 | 0.580645161 | 90 | 0.922580645 | 143 | 1.012903226 | 157 |
| 10 | 2.5 | 58.9 | 1.697792869 | 100 | 1.409168081 | 83 | 3.191850594 | 188 |
| 15 | 2.5 | 137 | 0.802919708 | 110 | 0.839416058 | 115 | 1.386861314 | 190 |
| 20 | 2.5 | 245 | 0.530612245 | 130 | 0.612244898 | 150 | 0.86122449 | 211 |
| 30 | 2.5 | 562 | 0.24911032 | 140 | 0.330960854 | 186 | 0.453736655 | 255 |
| 40 | 2.5 | 1001 | 0.12987013 | 130 | 0.193806194 | 194 | 0.251748252 | 252 |
| 40 | 7.5 | 323 | 0.340557276 | 110 | 0.628482972 | 203 | 0.736842105 | 238 |
| 50 | 7.5 | 512 | 0.1953125 | 100 | 0.41015625 | 210 | 1.56640625 | 802 |
| 60 | 7.5 | 742 | 0.121293801 | 90 | 0.330188679 | 245 | 1.525606469 | 1132 |
| 70 | 7.5 | 1014 | 0.078895464 | 80 | 0.331360947 | 336 | 1.474358974 | 1495 |
| 80 | 7.5 | 1329 | 0.052671181 | 70 | 0.291196388 | 387 | 0.256583898 | 341 |
| 80 | 15 | 647 | 0.092735703 | 60 | 0.451313756 | 292 | 0.771251932 | 499 |
| 90 | 15 | 825 | 0.060606061 | 50 | 0.4 | 330 | 0.653333333 | 539 |
| 100 | 15 | 1024 | 0.087890625 | 90 | 0.3515625 | 360 | 0.544921875 | 558 |

|  |  |  | VES 14 (Nyanya 1) |  | VES 15 (Nyanya 2) |  |
| --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 5.084745763 | 12 | 25.84745763 | 61 |
| 2 | 0.5 | 11.8 | 1.86440678 | 22 | 6.610169492 | 78 |
| 3 | 0.5 | 27.8 | 1.151079137 | 32 | 3.489208633 | 97 |
| 5 | 0.5 | 77.8 | 0.539845758 | 42 | 1.593830334 | 124 |
| 6 | 0.5 | 112 | 0.464285714 | 52 | 1.160714286 | 130 |
| 6 | 1 | 55 | 1.127272727 | 62 | 2.472727273 | 136 |
| 8 | 1 | 99 | 0.727272727 | 72 | 1.434343434 | 142 |
| 10 | 1 | 155 | 0.529032258 | 82 | 0.961290323 | 149 |
| 10 | 2.5 | 58.9 | 1.56196944 | 92 | 2.665534805 | 157 |
| 15 | 2.5 | 137 | 0.744525547 | 102 | 1.664233577 | 228 |
| 20 | 2.5 | 245 | 0.497959184 | 122 | 0.706122449 | 173 |
| 30 | 2.5 | 562 | 0.234875445 | 132 | 0.357651246 | 201 |
| 40 | 2.5 | 1001 | 0.121878122 | 122 | 0.237762238 | 238 |
| 40 | 7.5 | 323 | 0.315789474 | 102 | 0.507739938 | 164 |
| 50 | 7.5 | 512 | 0.1796875 | 92 | 0.361328125 | 185 |
| 60 | 7.5 | 742 | 0.110512129 | 82 | 0.264150943 | 196 |
| 70 | 7.5 | 1014 | 0.071005917 | 72 | 0.209072978 | 212 |
| 80 | 7.5 | 1329 | 0.046651618 | 62 | 0.171557562 | 228 |
| 80 | 15 | 647 | 0.080370943 | 52 | 0.446676971 | 289 |
| 90 | 15 | 825 | 0.050909091 | 42 | 0.301818182 | 249 |
| 100 | 15 | 1024 | 0.080078125 | 82 | 0.262695313 | 269 |

|  |  |  | VES 16 (Maraba 1) |  | VES 17 (Maraba 2) |  | VES 18 (Maraba 3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 2.118644068 | 5 | 10.59322034 | 25 | 23.72881356 | 56 |
| 2 | 0.5 | 11.8 | 1.271186441 | 15 | 2.966101695 | 35 | 6.016949153 | 71 |
| 3 | 0.5 | 27.8 | 0.899280576 | 25 | 1.618705036 | 45 | 3.345323741 | 93 |
| 5 | 0.5 | 77.8 | 0.449871465 | 35 | 0.706940874 | 55 | 1.580976864 | 123 |
| 6 | 0.5 | 112 | 0.401785714 | 45 | 0.580357143 | 65 | 1.133928571 | 127 |
| 6 | 1 | 55 | 1.000000000 | 55 | 1.363636364 | 75 | 2.345454545 | 129 |
| 8 | 1 | 99 | 0.656565657 | 65 | 0.858585859 | 85 | 1.383838384 | 137 |
| 10 | 1 | 155 | 0.483870968 | 75 | 0.612903226 | 95 | 0.922580645 | 143 |
| 10 | 2.5 | 58.9 | 1.443123939 | 85 | 1.782682513 | 105 | 2.529711375 | 149 |
| 15 | 2.5 | 137 | 0.693430657 | 95 | 0.839416058 | 115 | 1.627737226 | 223 |
| 20 | 2.5 | 245 | 0.469387755 | 115 | 0.551020408 | 135 | 0.681632653 | 167 |
| 30 | 2.5 | 562 | 0.177935943 | 100 | 0.258007117 | 145 | 0.34519573 | 194 |
| 40 | 2.5 | 1001 | 0.101898102 | 102 | 0.134865135 | 135 | 0.22977023 | 230 |
| 40 | 7.5 | 323 | 0.325077399 | 105 | 0.356037152 | 115 | 0.473684211 | 153 |
| 50 | 7.5 | 512 | 0.216796875 | 111 | 0.205078125 | 105 | 0.337890625 | 173 |
| 60 | 7.5 | 742 | 0.161725067 | 120 | 0.128032345 | 95 | 0.250673854 | 186 |
| 70 | 7.5 | 1014 | 0.123274162 | 125 | 0.08382643 | 85 | 0.201183432 | 204 |
| 80 | 7.5 | 1329 | 0.095560572 | 127 | 0.056433409 | 75 | 0.169300226 | 225 |
| 80 | 15 | 647 | 0.202472952 | 131 | 0.100463679 | 65 | 0.442040185 | 286 |
| 90 | 15 | 825 | 0.218181818 | 180 | 0.066666667 | 55 | 0.294545455 | 243 |
| 100 | 15 | 1024 | 0.1953125 | 200 | 0.092773438 | 95 | 0.2578125 | 264 |

|  |  |  | VES 19 (Maraba 4) |  | VES 20 (Maraba 5) |  |
| --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 21.18644068 | 50 | 19.49152542 | 46 |
| 2 | 0.5 | 11.8 | 5.338983051 | 63 | 4.915254237 | 58 |
| 3 | 0.5 | 27.8 | 3.057553957 | 85 | 2.841726619 | 79 |
| 5 | 0.5 | 77.8 | 1.503856041 | 117 | 1.388174807 | 108 |
| 6 | 0.5 | 112 | 1.071428571 | 120 | 0.973214286 | 109 |
| 6 | 1 | 55 | 2.218181818 | 122 | 2.036363636 | 112 |
| 8 | 1 | 99 | 1.333333333 | 132 | 1.232323232 | 122 |
| 10 | 1 | 155 | 0.883870968 | 137 | 0.812903226 | 126 |
| 10 | 2.5 | 58.9 | 2.427843803 | 143 | 2.224108659 | 131 |
| 15 | 2.5 | 137 | 1.562043796 | 214 | 1.430656934 | 196 |
| 20 | 2.5 | 245 | 0.657142857 | 161 | 0.604081633 | 148 |
| 30 | 2.5 | 562 | 0.332740214 | 187 | 0.307829181 | 173 |
| 40 | 2.5 | 1001 | 0.221778222 | 222 | 0.203796204 | 204 |
| 40 | 7.5 | 323 | 0.455108359 | 147 | 0.424148607 | 137 |
| 50 | 7.5 | 512 | 0.326171875 | 167 | 0.302734375 | 155 |
| 60 | 7.5 | 742 | 0.241239892 | 179 | 0.223719677 | 166 |
| 70 | 7.5 | 1014 | 0.192307692 | 195 | 0.177514793 | 180 |
| 80 | 7.5 | 1329 | 0.161775771 | 215 | 0.148984199 | 198 |
| 80 | 15 | 647 | 0.420401855 | 272 | 0.391035549 | 253 |
| 90 | 15 | 825 | 0.281212121 | 232 | 0.260606061 | 215 |
| 100 | 15 | 1024 | 0.24609375 | 252 | 0.227539063 | 233 |

|  |  |  | VES 21 (One Man Village1) |  | VES 22 (One Man Village2) |  |
| --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 25 | 59 | 16.94915254 | 40 |
| 2 | 0.5 | 11.8 | 6.525423729 | 77 | 4.152542373 | 49 |
| 3 | 0.5 | 27.8 | 3.45323741 | 96 | 2.517985612 | 70 |
| 5 | 0.5 | 77.8 | 1.645244216 | 128 | 1.208226221 | 94 |
| 6 | 0.5 | 112 | 1.1875 | 133 | 0.919642857 | 103 |
| 6 | 1 | 55 | 2.418181818 | 133 | 1.836363636 | 101 |
| 8 | 1 | 99 | 1.414141414 | 140 | 1.131313131 | 112 |
| 10 | 1 | 155 | 0.922580645 | 143 | 0.748387097 | 116 |
| 10 | 2.5 | 58.9 | 2.563667233 | 151 | 2.071307301 | 122 |
| 15 | 2.5 | 137 | 1.649635036 | 226 | 1.343065693 | 184 |
| 20 | 2.5 | 245 | 0.689795918 | 169 | 0.567346939 | 139 |
| 30 | 2.5 | 562 | 0.346975089 | 195 | 0.288256228 | 162 |
| 40 | 2.5 | 1001 | 0.230769231 | 231 | 0.191808192 | 192 |
| 40 | 7.5 | 323 | 0.479876161 | 155 | 0.393188854 | 127 |
| 50 | 7.5 | 512 | 0.341796875 | 175 | 0.28125 | 144 |
| 60 | 7.5 | 742 | 0.249326146 | 185 | 0.208894879 | 155 |
| 70 | 7.5 | 1014 | 0.201183432 | 204 | 0.16469428 | 167 |
| 80 | 7.5 | 1329 | 0.16854778 | 224 | 0.137697517 | 183 |
| 80 | 15 | 647 | 0.43122102 | 279 | 0.361669243 | 234 |
| 90 | 15 | 825 | 0.290909091 | 240 | 0.241212121 | 199 |
| 100 | 15 | 1024 | 0.252929688 | 259 | 0.209960938 | 215 |

|  |  |  | VES 23 (Ado1) |  | VES 24 (Ado 2) |  | VES 25 (Ado 3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 4.237288136 | 10 | 6.355932203 | 15 | 19.91525424 | 47 |
| 2 | 0.5 | 11.8 | 1.694915254 | 20 | 2.118644068 | 25 | 6.949152542 | 82 |
| 3 | 0.5 | 27.8 | 1.079136691 | 30 | 1.258992806 | 35 | 3.165467626 | 88 |
| 5 | 0.5 | 77.8 | 0.514138817 | 40 | 0.57840617 | 45 | 1.18251928 | 92 |
| 6 | 0.5 | 112 | 0.446428571 | 50 | 0.491071429 | 55 | 1.053571429 | 118 |
| 6 | 1 | 55 | 1.090909091 | 60 | 1.181818182 | 65 | 2.181818182 | 120 |
| 8 | 1 | 99 | 0.707070707 | 70 | 0.757575758 | 75 | 1.363636364 | 135 |
| 10 | 1 | 155 | 0.516129032 | 80 | 0.548387097 | 85 | 0.967741935 | 150 |
| 10 | 2.5 | 58.9 | 1.528013582 | 90 | 1.612903226 | 95 | 1.307300509 | 77 |
| 15 | 2.5 | 137 | 0.729927007 | 100 | 0.766423358 | 105 | 0.781021898 | 107 |
| 20 | 2.5 | 245 | 0.489795918 | 120 | 0.510204082 | 125 | 0.559183673 | 137 |
| 30 | 2.5 | 562 | 0.231316726 | 130 | 0.240213523 | 135 | 0.300711744 | 169 |
| 40 | 2.5 | 1001 | 0.11988012 | 120 | 0.124875125 | 125 | 0.173826174 | 174 |
| 40 | 7.5 | 323 | 0.309597523 | 100 | 0.325077399 | 105 | 0.569659443 | 184 |
| 50 | 7.5 | 512 | 0.17578125 | 90 | 0.185546875 | 95 | 0.37109375 | 190 |
| 60 | 7.5 | 742 | 0.107816712 | 80 | 0.114555256 | 85 | 0.291105121 | 216 |
| 70 | 7.5 | 1014 | 0.069033531 | 70 | 0.073964497 | 75 | 0.290927022 | 295 |
| 80 | 7.5 | 1329 | 0.045146727 | 60 | 0.048908954 | 65 | 0.253574116 | 337 |
| 80 | 15 | 647 | 0.077279753 | 50 | 0.085007728 | 55 | 0.411128284 | 266 |
| 90 | 15 | 825 | 0.048484848 | 40 | 0.054545455 | 45 | 0.362424242 | 299 |
| 100 | 15 | 1024 | 0.078125 | 80 | 0.083007813 | 85 | 0.313476563 | 321 |

|  |  |  | VES 26 (New Nyanya1) |  | VES 27 (New Nyanya2) |  | VES 28 (New Nyanya3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 21.61016949 | 51 | 20.33898305 | 48 | 22.03389831 | 52 |
| 2 | 0.5 | 11.8 | 5.593220339 | 66 | 5.169491525 | 61 | 5.847457627 | 69 |
| 3 | 0.5 | 27.8 | 3.057553957 | 85 | 2.805755396 | 78 | 3.165467626 | 88 |
| 5 | 0.5 | 77.8 | 1.439588689 | 112 | 1.323907455 | 103 | 1.491002571 | 116 |
| 6 | 0.5 | 112 | 1.035714286 | 116 | 0.955357143 | 107 | 1.080357143 | 121 |
| 6 | 1 | 55 | 2.181818182 | 120 | 2.036363636 | 112 | 2.272727273 | 125 |
| 8 | 1 | 99 | 1.282828283 | 127 | 1.181818182 | 117 | 1.323232323 | 131 |
| 10 | 1 | 155 | 0.851612903 | 132 | 0.787096774 | 122 | 0.877419355 | 136 |
| 10 | 2.5 | 58.9 | 2.34295416 | 138 | 2.190152801 | 129 | 2.427843803 | 143 |
| 15 | 2.5 | 137 | 1.525547445 | 209 | 1.430656934 | 196 | 1.576642336 | 216 |
| 20 | 2.5 | 245 | 0.628571429 | 154 | 0.587755102 | 144 | 0.648979592 | 159 |
| 30 | 2.5 | 562 | 0.320284698 | 180 | 0.275800712 | 155 | 0.330960854 | 186 |
| 40 | 2.5 | 1001 | 0.212787213 | 213 | 0.156843157 | 157 | 0.221778222 | 222 |
| 40 | 7.5 | 323 | 0.439628483 | 142 | 0.495356037 | 160 | 0.46130031 | 149 |
| 50 | 7.5 | 512 | 0.3125 | 160 | 0.314453125 | 161 | 0.33203125 | 170 |
| 60 | 7.5 | 742 | 0.233153639 | 173 | 0.218328841 | 162 | 0.242587601 | 180 |
| 70 | 7.5 | 1014 | 0.184418146 | 187 | 0.172583826 | 175 | 0.193293886 | 196 |
| 80 | 7.5 | 1329 | 0.155003762 | 206 | 0.145221971 | 193 | 0.162528217 | 216 |
| 80 | 15 | 647 | 0.412673879 | 267 | 0.383307573 | 248 | 0.42194745 | 273 |
| 90 | 15 | 825 | 0.272727273 | 225 | 0.252121212 | 208 | 0.282424242 | 233 |
| 100 | 15 | 1024 | 0.23828125 | 244 | 0.221679688 | 227 | 0.248046875 | 254 |

|  |  |  | VES 30 (Gwandara2) |  |
| --- | --- | --- | --- | --- |
| AB/2 | MN/2 | G.FACTOR | R(Ω) | R(Ωm) |
| 1 | 0.5 | 2.36 | 3.389830508 | 8 |
| 2 | 0.5 | 11.8 | 1.525423729 | 18 |
| 3 | 0.5 | 27.8 | 1.007194245 | 28 |
| 5 | 0.5 | 77.8 | 0.488431877 | 38 |
| 6 | 0.5 | 112 | 0.428571429 | 48 |
| 6 | 1 | 55 | 1.054545455 | 58 |
| 8 | 1 | 99 | 0.686868687 | 68 |
| 10 | 1 | 155 | 0.503225806 | 78 |
| 10 | 2.5 | 58.9 | 1.494057725 | 88 |
| 15 | 2.5 | 137 | 0.715328467 | 98 |
| 20 | 2.5 | 245 | 0.481632653 | 118 |
| 30 | 2.5 | 562 | 0.227758007 | 128 |
| 40 | 2.5 | 1001 | 0.117882118 | 118 |
| 40 | 7.5 | 323 | 0.303405573 | 98 |
| 50 | 7.5 | 512 | 0.171875 | 88 |
| 60 | 7.5 | 742 | 0.105121294 | 78 |
| 70 | 7.5 | 1014 | 0.067061144 | 68 |
| 80 | 7.5 | 1329 | 0.043641836 | 58 |
| 80 | 15 | 647 | 0.074188563 | 48 |
| 90 | 15 | 825 | 0.046060606 | 38 |
| 100 | 15 | 1024 | 0.076171875 | 78 |

APPENDIX II

69